VECTOR AMR

Complete Technical & Interview Preparation Handbook

ROS 2 Humble • Jetson Orin Nano • TensorRT • Computer Vision • LiDAR • Voice AI • ESP32

| Area | Project implementation |
| --- | --- |
| Compute | NVIDIA Jetson Orin Nano Developer Kit, 8 GB |
| Robot | 4-wheel skid/differential-drive autonomous mobile robot |
| Perception | IMX219 CSI camera + YOLOv8n-pose TensorRT + SORT-style tracking + RPLIDAR C1 |
| Interaction | Whisper tiny.en STT + NVIDIA NIM text/vision + Piper TTS |
| Control | Decision state logic + independent watchdog + Twist-to-PWM motor bridge |
| Embedded | ESP32-C3 Super Mini + 2 × BTS7960 + 4 × 12 V 100 RPM geared motors |
| Power | 11.1 V 6000 mAh 3S2P battery; 100 kΩ / 33 kΩ battery divider to GPIO1 ADC |
| Web | rosbridge + web_video_server + custom browser UI |

Purpose of this handbook This is written for interviews and deep revision, not as a README. It explains the architecture, every custom runtime node, the algorithms behind the code, the embedded motor path, performance/debugging lessons, design trade-offs, and model interview answers. Full source snapshots are included in the appendices.

CHAPTER 0

# How to use this handbook

A study order that moves from system story to code-level mastery.

Use the handbook in three passes. Pass 1: learn the 30-second and 2-minute system story, the architecture, and the end-to-end data flows. Pass 2: study every node’s inputs, outputs, state and core algorithm. Pass 3: work through the interview questions and then read the full source appendices while tracing messages through the graph.

## Recommended interview revision order

Architecture and end-to-end flow: Chapters 1-4.

Perception: camera -> pose detector -> tracker -> LiDAR obstacle processing.

Interaction: STT -> local intent routing/NIM -> Piper TTS.

Control and safety: decision node -> watchdog -> motor-control bridge -> ESP32.

Optimization/debugging stories: Chapter 11. These are strong evidence of real engineering work.

Interview bank and cheat sheets: Chapters 14-16.

Full source: Appendices A-L. Use these after you can explain the logic without reading code.

## Contents

| Chapter | Focus |
| --- | --- |
| 1 | Interview-ready project overview |
| 2 | Hardware architecture and electrical design |
| 3 | ROS 2 software architecture |
| 4 | End-to-end data flow |
| 5 | Perception package |
| 6 | Interaction package |
| 7 | Control and safety package |
| 8 | ESP32-C3 firmware and motor electronics |
| 9 | Web UI and external ROS components |
| 10 | Docker, launch and bringup |
| 11 | Performance optimization and debugging lessons |
| 12 | Design choices, limitations and V2 roadmap |
| 13 | Theory you must be able to explain |
| 14 | Interview question bank with model answers |
| 15 | Troubleshooting interview scenarios |
| 16 | Quick-reference cheat sheets |

CHAPTER 1

# Interview-ready project overview

What the robot is, what it does, and how to explain it clearly.

## 1.1 One-sentence description

Interview one-liner “I designed and built a ROS 2 autonomous mobile robot on NVIDIA Jetson Orin Nano that combines TensorRT vision, LiDAR obstacle sensing, person tracking, voice interaction, independent safety supervision, and ESP32 motor control.”

## 1.2 30-second answer

The robot is a modular ROS 2 Humble system running on a Jetson Orin Nano. A CSI camera feeds a TensorRT YOLO pose detector and a SORT-style tracker; an RPLIDAR C1 provides 2D obstacle ranges. A decision node combines the tracked person, LiDAR state and voice commands to publish /cmd_vel. A separate watchdog can independently force an emergency stop. The motor-control node converts Twist commands into signed PWM values and sends them by USB serial to an ESP32-C3, which drives two BTS7960 motor-driver modules. I also built voice interaction with Whisper, NVIDIA NIM and Piper, plus a browser dashboard.

## 1.3 What it can do

Capture a low-latency CSI camera stream and publish ROS Image / CameraInfo messages.

Run YOLOv8n-pose as a TensorRT engine on the Jetson GPU and publish structured detections.

Track detections over time with Kalman prediction and Hungarian/IoU data association.

Detect front obstacles from RPLIDAR LaserScan data and choose a freer side with hysteresis.

Follow a person, maintain a simple visual distance, roam/search when appropriate, and obey direct voice maneuvers.

Pause motion at the wake word, execute normalized voice commands, answer questions, report battery level and describe the camera scene.

Speak locally using Piper while coordinating STT muting and face/mouth animation.

Independently detect stale critical topics, a dangerously close frontal obstacle, or critical battery state.

Convert ROS Twist commands into differential left/right PWM and actuate real hardware through an ESP32-C3.

Expose video and robot state to a custom web interface via web_video_server and rosbridge.

## 1.4 The engineering layers

| Layer | Components | Responsibility |
| --- | --- | --- |
| Sensing | IMX219 camera, RPLIDAR C1, USB microphone, battery ADC | Observe environment, user and robot state |
| Perception | camera node, pose detector, tracker, obstacle detector | Convert raw sensor data into structured information |
| Interaction | STT, LLM/vision router, TTS | Human-robot voice and visual interaction |
| Decision | decision node | Choose autonomous/manual motion intent |
| Safety | watchdog | Override normal behavior when unsafe or data is stale |
| Actuation | motor-control node, ESP32-C3, BTS7960, motors | Convert velocity intent into physical wheel motion |
| Web | rosbridge, web_video_server, HTML/CSS/JS UI | Remote visualization and UI control |

## 1.5 What is AI and what is not

| Component | AI / algorithm? | Why |
| --- | --- | --- |
| YOLO pose | Deep-learning model | Neural network predicts boxes, class and human pose keypoints. |
| Whisper | Deep-learning model | Speech-to-text neural model. |
| NVIDIA NIM LLM/VLM | Generative AI models | Text answer and image-grounded scene description. |
| Piper | Neural TTS | ONNX speech synthesis model. |
| Kalman filter | Classical estimation algorithm | Predicts target state using a motion model and uncertainty. |
| Hungarian assignment | Optimization algorithm | Finds a minimum-cost one-to-one matching between detections and tracks. |
| LiDAR sector logic | Rule-based algorithm | Computes sector ranges and thresholds; no learned model. |
| Decision node | Rule/state logic | Combines inputs and selects motion behavior. |
| Watchdog | Rule-based safety supervisor | Monitors timing, distance and battery thresholds. |

CHAPTER 2

# Hardware architecture and electrical design

Know the role of every board, sensor, driver and power path.

## 2.1 Bill of materials and role

| Hardware | Project specification | Role |
| --- | --- | --- |
| Main computer | NVIDIA Jetson Orin Nano Developer Kit, 8 GB | Runs ROS 2, GPU inference, STT, API client, TTS, control and web services. |
| Camera | Waveshare IMX219-120 CSI, ~120° FOV | Primary visual sensor. |
| LiDAR | SLAMTEC RPLIDAR C1M1, 2D ToF | Metric planar ranging for obstacle detection. |
| MCU | ESP32-C3 Super Mini | Low-level motor actuation and battery ADC reporting. |
| Motor drivers | 2 × BTS7960 43 A H-bridge modules | One left motor group and one right motor group. |
| Motors | 4 × Johnson 12 V, 100 RPM geared DC motors | 4WD skid/differential propulsion. |
| Battery | 11.1 V 6000 mAh 3S2P Li-ion pack | Main robot power. |
| Charger | 12.6 V, 2 A | 3S pack charging. |
| Buck converter | XL4015 5 A step-down | Voltage regulation where required. |
| Battery divider | 100 kΩ + 33 kΩ to ESP32 GPIO1 | Scales pack voltage for ADC measurement. |
| Microphone | USB PnP microphone | Voice input. |
| Speaker | Bluetooth speaker | Robot voice output. |
| Chassis | 4WD metal mobile-robot chassis | Mechanical structure and drivetrain. |

## 2.2 Why Jetson Orin Nano

The Jetson is the high-level compute layer. It provides CUDA/TensorRT acceleration, enough CPU and memory for multiple ROS 2 Python processes, CSI camera integration through NVIDIA’s multimedia stack, and a Linux environment suitable for Docker, ROS 2 and web services. The ESP32 is intentionally separate so real-time-ish pin/PWM work is not tied to Linux scheduling.

## 2.3 Camera path

The current camera snapshot captures the IMX219 CSI sensor at 1640×1232 and 30 FPS, then converts the frame through NVIDIA’s GStreamer path to a 640×480 BGR working image. Earlier runs also used 1920×1080 capture. The important architecture point is that inference and ROS transport use the smaller working frame while the CSI/ISP path handles sensor capture efficiently.

Conceptual GStreamer path

nvarguscamerasrc -> NVMM/NV12 -> nvvidconv -> BGRx -> videoconvert -> BGR -> appsink appsink drop=true sync=false

drop=true is a real-time latency choice: if processing falls behind, stale frames are discarded instead of forming a growing queue. For robot control, the newest frame is normally more useful than guaranteed delivery of old frames.

## 2.4 RPLIDAR and coordinate orientation

The LiDAR driver publishes sensor_msgs/LaserScan. Each array element corresponds to an angle computed as angle_min + index × angle_increment. In the current obstacle and watchdog code, the physical front is configured at 180°, not 0°. This is an important project-specific calibration that corrected the earlier front/back inversion caused by the sensor’s mounting/orientation.

Interview lesson Sensor coordinate conventions are not assumptions. Verify the physical mounting against the message frame, then make the transform/orientation explicit in parameters or TF.

## 2.5 Differential/skid drive

The four DC motors are grouped into left and right sides. The high-level controller produces linear.x and angular.z. The motor-control node performs differential mixing: base speed comes from linear velocity and turn speed comes from angular velocity. Equal wheel commands drive straight; unequal commands curve; opposite signs rotate in place.

Current host-side motor mixing

base = linear.x × 250 turn = angular.z × 150 left_pwm = base - turn right_pwm = (base + turn) × 0.90 clamp both to [-255, +255]

## 2.6 Battery measurement

The 3S battery reaches about 12.6 V when full, so it cannot be connected directly to a 3.3 V ESP32 ADC. The 100 kΩ / 33 kΩ divider scales the pack voltage. The Jetson-side motor node reconstructs battery voltage from the raw 12-bit ADC sample and then maps 9.0-12.6 V linearly to 0-100%. This percentage mapping is a practical estimate, not a true Li-ion state-of-charge model.

Battery conversion

divider_ratio = 33k / (100k + 33k) ≈ 0.2481 Vadc = (raw / 4095) × 3.3 Vbattery = Vadc / divider_ratio percent = clamp((Vbattery - 9.0) / (12.6 - 9.0) × 100, 0, 100)

## 2.7 Power and ground interview points

The high-current motor path and logic/compute path have very different load characteristics; brownouts/noise are a design concern.

The ESP32-to-Jetson USB connection provides serial communication and a common reference path, but motor power must still be wired correctly through the drivers.

The voltage divider is for sensing only; it does not power the ESP32.

A future robust design would add fusing, better power distribution, encoder feedback, current sensing and an MCU-side communication timeout.

CHAPTER 3

# ROS 2 software architecture

Packages, nodes, topics, message types, QoS and concurrency.

## 3.1 Package organization

| Package | Custom nodes | Purpose |
| --- | --- | --- |
| perception_pkg | camera, detector, tracking, obstacle detection | Sensor acquisition and environment perception. |
| interaction_pkg | STT, LLM/vision, TTS | Voice input, reasoning/intent routing and speech output. |
| control_pkg | decision, watchdog, motor control | Behavior selection, safety and hardware actuation. |
| robot_bringup | launch/configuration | Starts the integrated stack with parameterized executables. |
| external packages | rplidar_node, rosbridge_websocket, web_video_server | Hardware driver and browser integration. |

## 3.2 Node inventory

| # | Node | Package/source | Type | Role |
| --- | --- | --- | --- | --- |
| 1 | camera_node.py | Perception | Custom Python | Current camera snapshot (1640x1232 capture -> 640x480 working frame). |
| 2 | detector_node.py | Perception | Custom Python | YOLOv8n-pose TensorRT detector with hand-gesture audio control. |
| 3 | tracking_node.py | Perception | Custom Python | SORT-style tracking with Kalman prediction, IoU and Hungarian assignment. |
| 4 | obstacle_detection_node.py | Perception | Custom Python | LiDAR sector processing with detection/clear hysteresis and direction lock. |
| 5 | stt_node.py | Interaction | Custom Python | Whisper tiny.en wake-word + command speech recognition. |
| 6 | llm_node.py | Interaction | Custom Python | Local intent router plus NVIDIA NIM text/vision calls. |
| 7 | tts_node.py | Interaction | Custom Python | Offline Piper TTS, persistent audio playback, cache and lip-sync. |
| 8 | decision_node.py | Control | Custom Python | Latest supplied decision snapshot with 1 m timed linear maneuver. |
| 9 | watchdog_node.py | Control | Custom Python | Independent liveness/proximity/battery safety watchdog. |
| 10 | motor_control_node.py | Control | Custom Python | Twist-to-PWM serial bridge and battery conversion. |
| 11 | rplidar_node | SLAMTEC driver | External C/C++ | Publishes /scan. |
| 12 | rosbridge_websocket | rosbridge_suite | External | ROS <-> browser WebSocket/JSON bridge. |
| 13 | web_video_server | RobotWebTools | External C++ | HTTP/MJPEG ROS image streaming. |
| 14 | display_node (optional) | debug/display | Custom Python | Local OpenCV debug view; not required for core runtime. |

## 3.3 Topic map

| Topic | Type | Producer | Main consumers / meaning |
| --- | --- | --- | --- |
| /camera/image_raw | sensor_msgs/Image | camera | detector, LLM vision, web/video |
| /camera/image_compressed | sensor_msgs/CompressedImage | camera | optional compressed clients |
| /camera/camera_info | sensor_msgs/CameraInfo | camera | calibration metadata |
| /camera/heartbeat | std_msgs/Empty | camera | watchdog camera liveness |
| /detections | vision_msgs/Detection2DArray | detector | tracker |
| /tracked_detections | vision_msgs/Detection2DArray | tracker | decision, watchdog, web |
| /scan | sensor_msgs/LaserScan | rplidar_node | obstacle detector, watchdog |
| /obstacle_detected | std_msgs/Bool | obstacle detector | decision, watchdog liveness |
| /front_distance | std_msgs/Float32 | obstacle detector | decision / display logic |
| /free_direction | std_msgs/String | obstacle detector | decision |
| /voice_command | std_msgs/String | STT | decision, LLM, TTS confirmations |
| /llm_response | std_msgs/String | LLM/watchdog | TTS |
| /tts_status | std_msgs/String | TTS | STT and UI coordination |
| /robot_emotion | std_msgs/String | TTS/watchdog | web face UI |
| /mouth_level | std_msgs/Float32 | TTS | web lip-sync |
| /audio_enabled | std_msgs/Bool | detector/web interaction | STT, TTS, decision |
| /robot_ui_mode | std_msgs/String | LLM | browser face/vision mode |
| /cmd_vel | geometry_msgs/Twist | decision + watchdog failsafe | motor control, watchdog |
| /battery_status | std_msgs/Float32 | motor control | LLM, watchdog, UI |
| /emergency_stop | std_msgs/Bool | watchdog | decision |

## 3.4 Why topics instead of one monolithic program

ROS 2 topics decouple producers and consumers. Camera capture does not need to know whether the image goes to the detector, LLM or web stream. The decision node can be changed without rewriting the motor firmware. The watchdog can observe the same raw signals independently. This modularity improves debugging, replacement, reuse and fault isolation, at the cost of DDS serialization/transport overhead and more distributed state to reason about.

## 3.5 QoS choices

High-rate sensor streams use a “freshness over guaranteed delivery” philosophy. Camera image subscriptions commonly use the ROS sensor-data profile (BEST_EFFORT and shallow history) so a slow consumer does not demand retransmission of old frames. Lower-rate control/state topics use reliable queue-based profiles. Interviewers may ask about QoS compatibility: a publisher/subscriber pair with incompatible reliability/durability settings may exchange no data even if the topic name and type match.

Official ROS 2 concept The ROS 2 Humble documentation explicitly notes that sensor data usually prioritizes timely newest samples over guaranteed receipt of every sample; the predefined sensor-data profile therefore uses best-effort reliability and a smaller queue.

## 3.6 Concurrency patterns used in this project

Detector: a latest-frame slot plus worker thread prevents a backlog of images; old frames are replaced.

STT: sounddevice callback writes audio chunks into a queue; a background listener thread performs transcription/command collection.

LLM: network calls run in daemon threads so ROS callbacks are not blocked by cloud latency.

TTS: an asyncio speech queue plus a persistent sounddevice output stream separates synthesis, playback and lip-sync.

Motor control: ROS callback writes commands while a background serial-reader thread receives battery messages.

Timers: decision control loop and watchdog health loop run periodically instead of depending only on incoming sensor callbacks.

CHAPTER 4

# End-to-end data flow

Trace exactly what happens from a sensor or voice input to physical motion or speech.

## 4.1 Person-following path

Follow path

IMX219 -> camera_node -> /camera/image_raw -> detector -> /detections -> tracker -> /tracked_detections -> decision_node -> /cmd_vel -> motor_control_node -> USB serial "left,right\n" -> ESP32-C3 -> BTS7960 -> four motors

## 4.2 Obstacle path

Obstacle path

RPLIDAR C1 -> rplidar_node -> /scan -> obstacle_detection_node -> /obstacle_detected + /front_distance + /free_direction -> decision_node -> safer /cmd_vel

## 4.3 Independent safety path

Safety path

raw /scan + camera heartbeat + tracked detections + obstacle heartbeat + /cmd_vel + battery -> watchdog_node -> active fault-reason set -> /emergency_stop=True + direct zero /cmd_vel -> decision stops; motor-control receives zero command

## 4.4 Voice command path

Voice path

USB mic -> STT queue -> Whisper tiny.en -> wake "Vector" -> normalized /voice_command -> decision node for motion + LLM router for conversation/UI/battery/vision -> /llm_response -> Piper TTS -> speaker

## 4.5 Vision-question path

Vision question path

"Vector ... describe scene" -> STT -> DESCRIBE_SCENE / phrase -> LLM node -> copy latest ROS camera frame -> CvBridge -> resize <= 640 px -> JPEG -> base64 -> NVIDIA NIM vision model -> short response -> /llm_response -> Piper

## 4.6 Wake word and motion safety

The wake-word event itself is a control event. The decision node stores the current pre-wake mode, switches to STOP and publishes zero velocity while it waits for the user’s command. After a recognized action, conversation, battery query, vision query or timeout/RESUME behavior, it returns to the saved mode. This avoids the robot continuing to roam while the user is trying to command it.

CHAPTER 5

# Perception package

Camera, TensorRT pose inference, tracking and LiDAR obstacle processing.

## 5.1 camera_node - acquisition and heartbeat

Purpose: own the camera, publish the newest working frame, optionally publish compressed data and CameraInfo, and expose a cheap liveness heartbeat for safety monitoring.

| Input | Output / state | Key settings |
| --- | --- | --- |
| CSI IMX219 through GStreamer | /camera/image_raw | 1640×1232 capture; 640×480 working frame; 30 FPS |
| Successful frames | /camera/heartbeat | 1 Hz lightweight Empty message |
| Calibration parameters | /camera/camera_info | frame_id camera_link; simplified fx/fy parameters |
| Open/read failures | reconnect path | reopen after configurable failure count |

### Core functions

| Function | What it does |
| --- | --- |
| __init__ | Declares parameters, QoS, publishers, capture object, timers and stats state. |
| open_camera | Builds GStreamer or V4L2 pipeline and opens cv2.VideoCapture. |
| create_camera_info_msg | Builds intrinsic/camera metadata message. |
| publish_camera_heartbeat | Publishes only if a valid frame was received recently. |
| timer_callback | Reads frame, converts with CvBridge, stamps/publishes image and stats, handles reconnect. |
| destroy_node | Safely releases camera resources. |

### Why the heartbeat optimization matters

The watchdog originally consumed /camera/image_raw just to know whether the camera was alive. That forced another Python process to receive and deserialize every 640×480 frame. Replacing that with a ~1 Hz Empty heartbeat removed unnecessary DDS transport, allocation and memory-copy work while preserving liveness detection.

Interview answer “I separated health information from payload data. A watchdog does not need pixels to verify camera liveness, so I publish a lightweight heartbeat tied to successful frame acquisition.”

## 5.2 detector_node - TensorRT YOLO pose

The current detector loads /workspace/models/vision/yolov8n-pose.engine through Ultralytics YOLO with task="pose". It runs at imgsz=640 with a 0.6 confidence threshold. In addition to object/person boxes, pose keypoints are used to detect raised left/right hands as an audio-control gesture.

| Design | Implementation | Reason |
| --- | --- | --- |
| GPU inference | TensorRT .engine | Low-latency inference optimized for NVIDIA GPU deployment. |
| No image backlog | latest_image slot + worker thread | Replaces stale frame instead of queuing all frames. |
| Gesture robustness | hold time + cooldown | Avoids a single noisy pose frame rapidly toggling audio. |
| Detection persistence | short persistence window | Smooths brief missed detections. |
| Audio state | /audio_enabled | Left-hand/right-hand gesture can change interaction mode. |

### Pose gesture logic

The code uses COCO-style pose indices 5/6 for shoulders and 9/10 for wrists. A wrist is considered raised when its keypoint confidence is high enough and its y-coordinate is above the corresponding shoulder. The gesture must persist for the configured hold time before publishing a state change, and a cooldown prevents repeated toggles.

### TensorRT interview explanation

A TensorRT engine is a serialized, GPU-targeted inference plan. The build step selects/optimizes kernels and graph execution for the target NVIDIA platform; the runtime loads the engine and executes inference. This is different from training: the AMR is performing inference on a pre-trained/exported model. The engineering objective is low latency and stable throughput on the edge device.

## 5.3 tracking_node - SORT-style multi-object tracking

A detector answers “what is present in this frame?” A tracker adds temporal identity: “is this the same person as in the previous frame?” The tracker maintains a state per object and associates new detections to predicted tracks.

### Kalman state

The implementation uses an 8-state, 4-measurement linear Kalman model. A convenient conceptual state is position/box geometry plus corresponding velocities. The predict step estimates where the box should be next; the update step corrects the prediction using the measured detection. Covariance represents uncertainty.

Kalman filter concept

Prediction: x_k¦k-1 = F x_k-1¦k-1 Covariance: P_k¦k-1 = F P F^T + Q Innovation: y = z - H x_pred Update: x = x_pred + K y

### Data association

For every predicted track and new detection, the node calculates IoU. It converts that similarity into a cost and uses the Hungarian algorithm to compute a one-to-one assignment. Matches below the IoU threshold are rejected. Unmatched detections create new tracks; unmatched tracks age until max_age removes them. min_hits prevents unstable new tracks from being published immediately.

| Parameter | Current value | Meaning |
| --- | --- | --- |
| iou_threshold | 0.3 | Minimum overlap quality for a valid association. |
| max_age | 30 | How many missed updates a track can survive. |
| min_hits | 4 | Hits required before a track is considered established. |
| max_publish_age | 2 | Avoid publishing very stale tracks. |
| kalman_covariance_scale | 10.0 | Initial uncertainty scale. |
| measurement_noise | 1.0 | Measurement uncertainty. |
| process_noise | 0.01 | Motion-model uncertainty. |

## 5.4 obstacle_detection_node - LiDAR sectors and hysteresis

The obstacle node does not build a map. It is a reactive local perception component. It walks through each LaserScan sample, converts the array index to an angle, rotates it into a front-centered coordinate using front_center_angle_deg=180, and collects front/left/right sector ranges.

| Parameter | Current value | Purpose |
| --- | --- | --- |
| obstacle_distance_threshold | 0.50 m | Enter obstacle state below this distance. |
| obstacle_clear_threshold | 0.60 m | Exit obstacle state only above this distance; creates hysteresis. |
| front_angle_range | ±20° | Frontal collision sector around physical front. |
| front_center_angle_deg | 180° | Corrects the LiDAR mounting/orientation. |
| left sector | +30° to +90° | Compute average free space on left. |
| right sector | -90° to -30° | Compute average free space on right. |
| direction_deadband | 0.30 m | Avoid changing side for insignificant left/right difference. |
| default_direction | LEFT | Deterministic tie behavior. |

### Why direction lock was added

A noisy scene can make left_avg and right_avg alternate around the decision boundary every scan. If the robot immediately obeys each new result, it can oscillate left-right instead of escaping. The current code chooses avoidance_direction when the obstacle is first detected and holds that direction until the obstacle state clears. This is temporal hysteresis/commitment on top of the distance hysteresis.

Strong debugging story An unstable avoidance behavior was not solved by “more AI”; it was solved by treating sensor noise and state transitions explicitly: correct the LiDAR front angle, add enter/clear thresholds, add a direction deadband, and lock the selected avoidance side until the event clears.

## 5.5 display_node - optional debug visualization

The optional display node subscribes to the raw image and tracked detections, draws bounding boxes/IDs, displays the frame with OpenCV and can publish a debug image. It is useful during development but is not a required part of autonomous control. In an interview, call it instrumentation rather than core autonomy.

CHAPTER 6

# Interaction package

Speech recognition, intent routing, vision-language queries and local speech synthesis.

## 6.1 stt_node - wake word and speech commands

The STT node uses faster-whisper/CTranslate2 with tiny.en. The current parameters configure CUDA with int8_float16 compute, microphone input at 44.1 kHz, and resampling to Whisper’s 16 kHz processing rate.

| Stage | Implementation | Why |
| --- | --- | --- |
| Audio capture | sounddevice callback -> queue | Keep audio callback short; process elsewhere. |
| Wake detection | RMS gate then Whisper transcription | Avoid continuously transcribing pure silence. |
| Wake variants | Vector plus common misrecognitions | Improve practical wake recall. |
| Acknowledgement | publish WAKE_WORD_DETECTED and wait for TTS “Yes?” | Prevents command recording from racing robot speech. |
| Command collection | RMS/silence-based recording up to 6 s | Stop when user finishes speaking. |
| Normalization | _parse() -> control tokens | Decision/LLM receive stable commands rather than arbitrary text for known controls. |
| Feedback prevention | listen to /tts_status; drain/flush while SPEAKING | Stops the robot transcribing its own speaker. |
| Audio toggle | /audio_enabled | Allows web/gesture control of listening. |

### Important STT parameters

| Parameter | Value |
| --- | --- |
| model_name | tiny.en |
| device | cuda |
| compute_type | int8_float16 |
| sample_rate | 44100 |
| whisper_rate | 16000 |
| rms_wake | 600 |
| rms_command | 150 |
| silence_sec | 1.5 |
| command_max_sec | 6.0 |
| tts_echo_decay_sec | 0.6 |
| ack_start_timeout | 0.8 |
| ack_total_timeout | 8.0 |

### Why faster-whisper/CTranslate2

The project uses a Whisper-compatible transcription pipeline implemented through CTranslate2. The interview-level point is not just the library name: transcription is a compute-heavy neural inference task, so the stack uses a small English model and GPU/quantized compute settings to make always-available interaction feasible on an edge device.

## 6.2 llm_node - intent router before LLM

The LLM node is deliberately not the motion controller. It first routes deterministic robot intents locally, and only sends genuinely conversational or vision-language requests to the NVIDIA NIM API. This keeps STOP/FOLLOW/UI/battery behavior predictable and avoids paying network latency for commands that can be handled locally.

| Local route | Behavior |
| --- | --- |
| face | Publish /robot_ui_mode="face" and acknowledge. |
| vision/feed/camera | Publish UI vision mode. |
| battery | Read cached /battery_status and answer locally. |
| describe / DESCRIBE_SCENE | Start one-shot vision query using latest cached frame. |
| follow | Acknowledge; decision node handles motion. |
| explore/roam | Acknowledge; decision node owns behavior. |
| normalized motion tokens | Ignore in LLM path; decision node handles them. |
| other speech | Call text LLM after rate-limit/de-duplication checks. |

### Models and request controls

| Item | Current setting |
| --- | --- |
| Text model | meta/llama-3.1-8b-instruct |
| Vision model | meta/llama-3.2-11b-vision-instruct |
| Endpoint | NVIDIA OpenAI-compatible NIM endpoint |
| NVIDIA_API_KEY | Read from environment; not hard-coded |
| Text minimum interval | 4 s |
| Vision cooldown | 3 s |
| Duplicate window | 3 s |
| Vision max dimension | 640 px |
| Vision JPEG quality | 80 |

### Vision request optimization

The node caches only the latest ROS Image. When a vision command arrives it converts that one frame with CvBridge, downsizes it if its longest dimension is over 640, JPEG-encodes it, base64-encodes the bytes and sends the image in a multimodal request. This is event-driven vision-language use, not a VLM running on every camera frame.

Interview distinction YOLO pose is the continuous local perception model. The vision-language model is a cloud call used only on demand for natural-language scene understanding.

## 6.3 tts_node - offline Piper speech and lip-sync

Piper runs locally on CPU (use_cuda=False). The TTS node receives /llm_response and known command confirmations, synthesizes audio, plays it through a persistent sounddevice output stream, publishes SPEAKING/IDLE status, publishes robot emotion and derives mouth-level values for the browser face.

| Mechanism | Reason |
| --- | --- |
| Persistent audio stream | Avoid repeated audio-device open/close overhead and improve latency/stability. |
| Async speech queue | Serialize utterances without blocking ROS callbacks. |
| Fixed-phrase cache | Repeated phrases such as “Yes?” or “Stopping.” skip re-synthesis after first use. |
| RMS-based mouth levels | Provides simple normalized lip-sync animation. |
| Bluetooth keepalive | Very quiet output can keep some speakers awake. |
| TTS status | STT knows when to mute/drain mic input. |
| CPU Piper | Reserves GPU capacity for high-value perception while TTS remains practical on CPU. |

### Current TTS defaults

| Parameter | Value |
| --- | --- |
| piper_voice | en_US-hfc_female-medium |
| piper_data_dir | /workspace/models/piper |
| audio_samplerate | 44100 |
| chunk_ms | 50 |
| bt_latency_s | 0.1 |
| post_playback_delay_s | 0.4 |
| post_playback_delay_cached_s | 0.1 |
| startup_speech_delay_s | 5.0 |
| cache_fixed_phrases | True |

CHAPTER 7

# Control and safety package

Behavior logic, independent safety and ROS-to-embedded actuation.

## 7.1 decision_node - behavior state machine

The decision node is the high-level behavior coordinator. It fuses tracked detections, obstacle state, front distance, freer direction, normalized voice commands, audio mode and emergency-stop state. It is not a path planner and does not maintain a global map.

| State / flag | Meaning |
| --- | --- |
| voice_mode=AUTO | Autonomous callback behavior can roam or follow. |
| voice_mode=STOP | Publish/maintain zero motion. |
| voice_mode=MANEUVER | Run a timed direct command such as forward, backward, left or right. |
| follow_enabled=False | AUTO behaves as roaming/exploration. |
| follow_enabled=True | AUTO searches for and follows a person. |
| estop_active=True | Watchdog overrides all normal states immediately. |

### Wake/command behavior

On WAKE_WORD_DETECTED the node stores the prior mode/follow flag and stops. STOP remains stopped. FOLLOW_PERSON/SEARCH enables following. Direct LEFT/RIGHT/FORWARD/BACKWARD starts a timed maneuver and then returns to the saved mode. DESCRIBE_SCENE/BATTERY_STATUS and normal conversation restore the pre-wake behavior because those commands do not require a motion-mode change.

### Timed direct maneuvers

| Maneuver | Current logic |
| --- | --- |
| LEFT / RIGHT | turn_speed=0.6 rad/s; duration = 90° / measured_turn_rate(1.2 rad/s). |
| FORWARD / BACKWARD | linear_speed=0.25 m/s; distance=1.0 m; duration=1.0/0.25=4.0 s. |

Limitation Without wheel encoders, “1 meter” and “90 degrees” are open-loop time estimates. Floor friction, battery voltage, load and motor mismatch change the real distance/angle. In an interview, say this directly and propose encoder odometry/closed-loop control for V2.

### Obstacle priority inside decision

If obstacle_detected is true, obstacle avoidance takes priority over following/roaming. The current behavior initially stops, then rotates toward the /free_direction. The obstacle node’s direction lock prevents the selected side from flipping every scan. When the obstacle clears, normal behavior resumes.

### Person-following algorithm

The decision node chooses a tracked person. It tries to keep the same track ID; if that ID is unavailable it can select the largest person box as the next target. Horizontal image error is center_x(target) - 320. Angular velocity is proportional to the negative error. Box width acts as a cheap distance proxy: a wide box implies the same person is nearer.

| Condition | Command |
| --- | --- |
| width > 450 px | Stop: target is considered too close. |
| abs(error) > 60 px | Slow forward (0.08) with angular gain ~0.002. |
| otherwise | Faster forward (0.5) with angular gain ~0.003, then deadband angular to zero when centered. |
| temporary target loss < 2 s | Hold zero while waiting for reacquisition. |
| no target after timeout | Roam: linear ~0.4 and angular ~0.15. |

## 7.2 watchdog_node - independent safety supervisor

The watchdog is deliberately separate from decision logic. Safety should not disappear if the decision node has a bug. It monitors data liveness, raw LiDAR proximity and battery state. A set of active fault reasons determines whether emergency stop is asserted.

| Monitor | Timeout / threshold | Action |
| --- | --- | --- |
| /scan | 1.5 s stale | Add stale_scan fault reason. |
| /obstacle_detected | 1.5 s stale | Add stale_obstacle fault reason. |
| /tracked_detections | 2.5 s stale | Add stale tracking fault. |
| /camera/heartbeat | 2.0 s stale | Add stale camera fault without receiving images. |
| /cmd_vel | 2.0 s stale while robot was moving | Detect possible decision/control failure. |
| raw front LiDAR | <0.28 m while moving forward | Immediate critical_distance reason. |
| critical clear | >0.35 m or no longer moving forward | Clear proximity reason with hysteresis. |
| battery | ≤20% warn; ≤10% critical; >15 s stale | Speak warnings; battery reporting health tracked. |
| startup | 10 s grace | Avoid false liveness faults while stack is still launching. |

### Why it also publishes zero /cmd_vel

The watchdog publishes /emergency_stop for the decision node, but it also directly publishes a zero Twist as a failsafe. This is defense in depth: even if normal decision cooperation is delayed or broken, a safety process can still send a stop command down the actuation path.

### Reason-set model

Multiple faults can exist at once. The code stores them in active_reasons and only releases emergency stop when every active condition is cleared. This avoids a bug where one recovered sensor could clear an e-stop while another fault is still active. Spoken warnings are tracked separately so the robot does not repeat the same warning every health-check cycle.

## 7.3 motor_control_node - Twist to PWM + battery bridge

This node is the boundary between ROS motion semantics and the microcontroller’s simple signed PWM protocol. It subscribes to /cmd_vel and writes “left,right\n” at 115200 baud on /dev/ttyACM0. A background thread reads BATT:<raw> messages and publishes /battery_status.

| Parameter | Value | Purpose |
| --- | --- | --- |
| stop thresholds | 0.02 linear / 0.02 angular | Treat tiny commands as stop. |
| search_turn_multiplier | 250 | Scale in-place rotation. |
| base_speed_multiplier | 250 | Scale forward/backward linear.x. |
| follow_turn_multiplier | 150 | Scale angular steering contribution. |
| right_motor_correction | 0.90 | Open-loop compensation for mechanical motor mismatch. |
| max_pwm | 255 | Protocol/output clamp. |
| motor_deadzone | 15 | Drop ineffective tiny PWM values. |
| serial | /dev/ttyACM0 @ 115200 | Jetson-to-ESP32 link. |

### Stop / rotate / drive branches

If |linear| and |angular| are both tiny -> left=right=0.

If linear is tiny but angular is significant -> left=-turn, right=+turn for in-place rotation.

Otherwise -> differential mixing combines base and turn, then applies right-side correction.

Clamp to ±255 and zero values below the deadzone before serial transmission.

### Serial reader and shutdown safety

The serial reader runs in a daemon thread so waiting for battery data does not block velocity callbacks. If the port is unavailable it retries. During normal node shutdown the code sends 0,0, flushes and closes the port. One future improvement is an ESP32-side command timeout so a sudden USB/Jetson failure cannot leave the last PWM value active.

CHAPTER 8

# ESP32-C3 firmware and motor electronics

The low-level path from a serial line to two BTS7960 H-bridges.

## 8.1 Recovered ESP32 project pin map

| Signal | GPIO | Function |
| --- | --- | --- |
| BATTERY_PIN | 1 | ADC input from 100 kΩ / 33 kΩ divider. |
| L_RPWM | 2 | Left BTS7960 forward-direction PWM input. |
| L_LPWM | 3 | Left BTS7960 reverse-direction PWM input. |
| R_RPWM | 4 | Right BTS7960 forward-direction PWM input. |
| R_LPWM | 5 | Right BTS7960 reverse-direction PWM input. |
| L_REN | 6 | Left driver right-side enable. |
| L_LEN | 7 | Left driver left-side enable. |
| R_REN | 8 | Right driver right-side enable. |
| R_LEN | 9 | Right driver left-side enable. |

## 8.2 Serial protocol

| Direction | Example | Meaning |
| --- | --- | --- |
| Jetson -> ESP32 | 120,108\n | Signed PWM command: left group +120, right group +108. |
| Jetson -> ESP32 | -120,120\n | Opposite signs: rotate in place. |
| Jetson -> ESP32 | 0,0\n | Stop both motor groups. |
| ESP32 -> Jetson | BATT:2875\n | Raw battery-divider ADC sample; Jetson converts to voltage/percent. |

## 8.3 BTS7960 direction principle

Each side has two PWM inputs. A positive signed command drives RPWM and sets LPWM to zero; a negative command sets RPWM to zero and drives LPWM with the absolute magnitude. Zero clears both. The enable pins are held HIGH so the H-bridge accepts PWM commands.

## 8.4 Compatible reconstructed firmware

Source-status note The final .ino file was not present in the recovered file library. The motor GPIO mapping, enable GPIO mapping, 115200 baud, newline-delimited left/right parser, analogWrite direction behavior, battery ADC GPIO1 and BATT:<raw> protocol were recovered from the project history. The code below reconstructs those behaviors; the 1-second battery reporting interval and explicit analogReadResolution(12) line are compatibility details, not claimed as byte-for-byte original source.

ESP32-C3 reference .ino matching the current Jetson-side contract

// Vector AMR - ESP32-C3 motor controller reference firmware // Reconstructed to match the recovered project pin map and Jetson serial protocol. // Recovered project details: motor pins, enable pins, 115200 baud, signed left/right // commands, analogWrite direction control, battery ADC on GPIO1, and BATT:<raw> output. // The periodic battery-report interval and explicit 12-bit ADC line are documented // here as a compatible reconstruction because the final .ino file was not present. const int L_RPWM = 2; const int L_LPWM = 3; const int R_RPWM = 4; const int R_LPWM = 5; const int L_REN = 6; const int L_LEN = 7; const int R_REN = 8; const int R_LEN = 9; const int BATTERY_PIN = 1; // GPIO1 / ADC1 const unsigned long BATTERY_REPORT_INTERVAL_MS = 1000; unsigned long lastBatteryReportMs = 0; void setMotor(int rpwmPin, int lpwmPin, int speedValue) { speedValue = constrain(speedValue, -255, 255); if (speedValue > 0) { analogWrite(rpwmPin, speedValue); analogWrite(lpwmPin, 0); } else if (speedValue < 0) { analogWrite(rpwmPin, 0); analogWrite(lpwmPin, abs(speedValue)); } else { analogWrite(rpwmPin, 0); analogWrite(lpwmPin, 0); } } void stopMotors() { setMotor(L_RPWM, L_LPWM, 0); setMotor(R_RPWM, R_LPWM, 0); }

void handleMotorCommand(const String &line) { int commaIndex = line.indexOf(','); if (commaIndex < 0) { return; } int leftSpeed = line.substring(0, commaIndex).toInt(); int rightSpeed = line.substring(commaIndex + 1).toInt(); leftSpeed = constrain(leftSpeed, -255, 255); rightSpeed = constrain(rightSpeed, -255, 255); setMotor(L_RPWM, L_LPWM, leftSpeed); setMotor(R_RPWM, R_LPWM, rightSpeed); } void reportBatteryRaw() { int raw = analogRead(BATTERY_PIN); Serial.print("BATT:"); Serial.println(raw); } void setup() { Serial.begin(115200); pinMode(L_RPWM, OUTPUT); pinMode(L_LPWM, OUTPUT); pinMode(R_RPWM, OUTPUT); pinMode(R_LPWM, OUTPUT); pinMode(L_REN, OUTPUT); pinMode(L_LEN, OUTPUT); pinMode(R_REN, OUTPUT); pinMode(R_LEN, OUTPUT); digitalWrite(L_REN, HIGH); digitalWrite(L_LEN, HIGH); digitalWrite(R_REN, HIGH); digitalWrite(R_LEN, HIGH); analogReadResolution(12); stopMotors();

} void loop() { if (Serial.available()) { String line = Serial.readStringUntil('\n'); line.trim(); if (line.length() > 0) { handleMotorCommand(line); } } unsigned long now = millis(); if (now - lastBatteryReportMs >= BATTERY_REPORT_INTERVAL_MS) { lastBatteryReportMs = now; reportBatteryRaw(); } }

## 8.5 Firmware walkthrough

Pin constants map one BTS7960 to the left motor pair and one to the right motor pair; GPIO1 is reserved for battery ADC.

setMotor() converts a signed -255…+255 command into mutually exclusive forward/reverse PWM pins.

handleMotorCommand() splits the newline-delimited CSV line at the comma, converts both fields to integers and clamps them.

setup() starts USB serial at 115200, configures pins, enables both H-bridges, establishes 12-bit ADC semantics and commands stop.

loop() handles incoming motor commands and periodically reports the raw ADC value as BATT:<raw>.

## 8.6 Why battery conversion stays on Jetson

Keeping the ESP32 response as a raw ADC sample makes the firmware simple. Divider values, full/empty thresholds and percentage mapping can be changed as ROS parameters without reflashing the microcontroller. The trade-off is that the Jetson must know the ADC reference and divider calibration.

## 8.7 Embedded safety improvement to mention in interviews

V2 improvement Add an MCU-side communication watchdog: record the time of the last valid motor command and force both PWM outputs to zero if no fresh command arrives within a short timeout. That covers USB disconnect, Jetson crash or ROS process failure at a layer below the Linux watchdog.

CHAPTER 9

# Web UI and external ROS components

What you reused, what each component contributes, and how the browser connects to ROS.

## 9.1 rosbridge_websocket

rosbridge exposes ROS data/services through a JSON/WebSocket interface that browser JavaScript can use. Your web UI subscribes to robot state such as UI mode, emotion, mouth level, tracked detections and audio state without implementing DDS in the browser.

## 9.2 web_video_server

web_video_server serves ROS image topics over HTTP using browser-friendly streaming. It solves a different problem from rosbridge: rosbridge carries ROS-like messages/control data over WebSocket; web_video_server efficiently exposes camera imagery as a video/MJPEG-style stream.

## 9.3 SLAMTEC RPLIDAR ROS driver

The SLAMTEC driver handles serial/hardware protocol details and publishes standard sensor_msgs/LaserScan on /scan. Your obstacle logic starts from that standard ROS message instead of reimplementing the device protocol. In interviews, clearly separate integration of an upstream driver from the custom obstacle algorithm you wrote on top of it.

## 9.4 Browser UI state

The custom web layer can switch between a robot face and vision feed, display emotion/mouth animation and expose interaction controls. The LLM node publishes /robot_ui_mode for voice-driven “show your face / show camera” behavior. TTS publishes emotion and mouth-level state. Audio enabled state is shared across gesture/web/STT/TTS/decision logic.

## 9.5 What I built vs what I reused

| Built/integrated by project | Reused upstream component |
| --- | --- |
| Camera GStreamer ROS node and heartbeat | OpenCV, GStreamer, NVIDIA Argus stack |
| Pose inference pipeline and gesture logic | Ultralytics YOLO / TensorRT engine runtime |
| SORT-style tracker integration/parameters | Kalman/Hungarian algorithms + Python libraries |
| LiDAR sector/hysteresis logic | SLAMTEC ROS LiDAR driver |
| Voice state machine and integration | faster-whisper/CTranslate2 |
| Local routing and NIM request logic | NVIDIA NIM endpoint/models |
| Speech queue/playback/lip-sync integration | Piper and sounddevice |
| Robot decision logic and watchdog | ROS 2 messaging and standard messages |
| Motor bridge and ESP32 protocol | BTS7960 hardware |
| Custom browser interface | rosbridge_suite + web_video_server |

CHAPTER 10

# Docker, launch and bringup

How the complete robot starts, how to verify it, and what runs where.

## 10.1 Runtime environment

ROS 2 Humble and the custom packages run inside the Jetson Docker environment. The workspace is sourced before launching robot_bringup/params_my_robot.launch.py. The NIM API key is read from NVIDIA_API_KEY in the environment rather than being stored in the source code.

## 10.2 Typical launch pattern

Integrated launch shell

export OMP_NUM_THREADS=1 export OPENBLAS_NUM_THREADS=1 export MKL_NUM_THREADS=1 export NUMEXPR_NUM_THREADS=1 source /opt/ros/humble/setup.bash source /workspace/install/setup.bash ros2 launch robot_bringup params_my_robot.launch.py

## 10.3 Principal runtime processes

A full run has the 10 custom runtime nodes plus rplidar_node, rosbridge_websocket and web_video_server. An optional display node may be launched for local visualization. This is a useful interview point: the system is distributed into cooperating ROS processes rather than one large Python process.

## 10.4 Verification commands you should know

Useful runtime checks

ros2 node list ros2 topic list -t ros2 topic hz /camera/image_raw ros2 topic hz /detections ros2 topic hz /tracked_detections ros2 topic echo /obstacle_detected ros2 topic echo /front_distance ros2 topic echo /free_direction ros2 topic echo /battery_status ros2 topic echo /emergency_stop tegra_stats_or_command="tegrastats"

## 10.5 How to debug a ROS graph systematically

Confirm the producer node is alive.

Confirm topic name and exact message type.

Inspect publisher/subscriber counts with ros2 topic info -v.

Check QoS compatibility when counts exist but messages do not arrive.

Echo the topic before blaming downstream logic.

Measure rate and timestamp/freshness, not only “does a message exist?”.

Walk one edge at a time from sensor -> perception -> decision -> actuation.

For hardware, verify the serial bytes independently of ROS, then verify PWM independently of high-level behavior.

CHAPTER 11

# Performance optimization and debugging lessons

The strongest evidence that this is a real deployed system, not only a demo script.

## 11.1 Measured optimization results

| Metric | Before | After targeted optimization |
| --- | --- | --- |
| Full-stack RAM | ~4.7-4.8 GB | ~4.08 GB in optimized run |
| RAM reduction | - | ~650-670 MB |
| Detector FPS | often ~15-23 | generally ~26-29 |
| Effective /detections rate | ~24.5 Hz | ~27.2 Hz |
| Watchdog CPU | ~20-26% | ~11-17% |
| Obstacle log rate | ~20 lines/s | ~1 line/s |
| Temperature | ~59-62 °C | ~58-60 °C |
| Swap growth during run | no meaningful growth | no meaningful growth |

## 11.2 Thread oversubscription

Several Python/ML libraries can each create OpenMP/BLAS worker pools. In a multi-process ROS stack on a six-core Jetson, every process creating many compute threads caused contention, context switching, extra memory and unpredictable callback scheduling. Setting OMP_NUM_THREADS, OPENBLAS_NUM_THREADS, MKL_NUM_THREADS and NUMEXPR_NUM_THREADS to 1 at stack launch produced a large stability improvement.

Interview formulation “The bottleneck was not one slow algorithm; it was system-level oversubscription across multiple processes. I profiled per-process CPU/RSS and constrained hidden numerical-library thread pools so ROS callbacks and inference had predictable CPU access.”

## 11.3 Camera heartbeat instead of image subscription

The watchdog used to receive full camera frames solely for liveness. Moving to /camera/heartbeat cut watchdog CPU materially and removed redundant deserialization/copying. This is an example of matching the message payload to the information a consumer actually needs.

## 11.4 Log throttling

High-frequency LiDAR/decision logs can become their own load and make useful transitions hard to see. The obstacle and decision paths were changed to log summaries periodically and state transitions when they matter. “Observability” is not the same as printing every callback.

## 11.5 LiDAR front/back inversion

A critical project-specific issue was that objects physically in front could be interpreted as rear-sector data because the physical LiDAR orientation did not match the assumed zero-angle direction. The current code defines front_center_angle_deg=180 and normalizes each relative angle with atan2(sin, cos). The lesson is to validate coordinate frames with a known physical target before tuning thresholds.

## 11.6 Obstacle left-right oscillation

When left and right free-space estimates changed slightly from scan to scan, the robot could alternate turn direction instead of committing to an escape. The fix is visible in the current obstacle node: a direction deadband, obstacle enter/clear hysteresis, and a latched avoidance_direction while the obstacle remains active.

## 11.7 Watchdog proximity behavior

The raw-LiDAR watchdog condition is gated by moving_forward. This prevents a nearby object from repeatedly triggering the same “critical front” condition while the robot is already stopped or turning away. A separate clear distance (0.35 m versus 0.28 m trigger) prevents chattering around a single threshold.

## 11.8 Person-follow collision lesson

Pure image-space person following is not sufficient collision avoidance. The person target can encourage forward motion while a table or other non-person object lies in the path. The system therefore gives LiDAR obstacle/safety logic higher priority than target following. A stronger V2 would fuse navigation costmaps/depth and wheel odometry rather than relying on a single follow heuristic.

## 11.9 Measurement caveat

Tools such as ros2 topic hz are themselves subscribers and consume CPU/DDS resources. Performance numbers should therefore be interpreted with the instrumentation load in mind. For interviews, emphasize that you measured rate, latency, RAM, CPU, GPU/tegrastats and thermal behavior rather than optimizing from intuition alone.

CHAPTER 12

# Design choices, limitations and V2 roadmap

Be able to defend the current design without pretending it is a full Nav2 AMR.

## 12.1 Important design choices

| Choice | Why it made sense | Trade-off |
| --- | --- | --- |
| ROS 2 modular nodes | Isolation, observability, replaceability and standard messages. | DDS/process overhead and more integration complexity. |
| TensorRT pose engine | Fast edge inference and pose keypoints for gesture control. | Engine is target/runtime-specific and needs deployment care. |
| 2D LiDAR reactive obstacle logic | Simple metric safety signal without map requirement. | No global planning/localization. |
| Bounding-box width for follow distance | Very cheap monocular heuristic. | Not true depth; person size/pose changes estimate. |
| Open-loop timed motion | Works without encoders. | Distance/angle varies with battery, friction and load. |
| Local intent routing before LLM | Deterministic low-latency robot commands. | Requires explicit intent rules. |
| Piper local TTS | Offline, predictable voice output. | CPU work and voice/model constraints. |
| Independent watchdog | Defense in depth. | Needs careful ownership of /cmd_vel and fault-clearing logic. |
| ESP32 low-level layer | Simple, robust hardware PWM/ADC interface. | Serial protocol is custom and currently open-loop. |

## 12.2 What this robot is not

It is not currently a full SLAM/localization/Nav2 stack.

It does not have wheel-encoder odometry in the described current chassis.

Its 1 m / 90° commands are timed estimates, not closed-loop metric motion.

The bounding-box follow distance is not stereo/depth sensing.

The obstacle node is reactive sector logic, not a learned path planner.

The cloud LLM/VLM is not in the safety-critical motion loop.

## 12.3 Natural V2 architecture

A strong next version keeps most of the current perception/interaction/web stack but adds encoder motors, wheel odometry, base_link/odom/map TF frames, IMU fusion where available, SLAM Toolbox, localization and Navigation2. The decision layer can then translate higher-level intent into Nav2 goals rather than directly driving /cmd_vel for most autonomous behavior.

| V2 addition | Benefit |
| --- | --- |
| Encoder motors + PID speed loops | Closed-loop wheel speed and repeatable motion. |
| Odometry + TF | Robot pose over time and standard navigation frame tree. |
| IMU + EKF | Better heading/pose estimate during wheel slip. |
| SLAM Toolbox | 2D mapping and localization. |
| Nav2 costmaps/planners/controllers | Global/local planning and obstacle-aware navigation. |
| MCU command timeout | Stop even if Jetson/USB communication dies. |
| Depth/stereo or LiDAR-camera fusion | Better near-object geometry and follow distance. |
| rosbag test datasets | Repeatable regression tests for perception/control. |
| Lifecycle nodes/diagnostics | More production-like startup, health and recovery. |

CHAPTER 13

# Theory you must be able to explain

Concise technical foundations behind the implementation.

## 13.1 ROS 2 publisher/subscriber model

A publisher writes typed messages to a named topic. A subscriber receives compatible messages. DDS handles discovery and transport. The nodes do not call each other directly for streaming data, so producers and consumers can evolve independently.

## 13.2 Twist message

geometry_msgs/Twist contains linear and angular velocity vectors. For a planar mobile robot, this project primarily uses linear.x for forward/backward intent and angular.z for yaw/turn intent.

## 13.3 Sensor QoS

BEST_EFFORT can drop samples instead of retrying them. For a high-rate camera, a newer frame can be more useful than retransmitting an old frame. Reliable delivery is more useful for low-rate control/state where missing a transition matters.

## 13.4 TensorRT

TensorRT optimizes a trained network for inference on NVIDIA GPUs and serializes the optimized execution plan as an engine. It is a deployment/inference technology, not a training framework.

## 13.5 IoU

Intersection over Union = area(intersection) / area(union). It ranges from 0 to 1 and measures how much two bounding boxes overlap. The tracker uses it as an association similarity.

## 13.6 Hungarian algorithm

Given a cost matrix between predicted tracks and detections, the Hungarian algorithm computes a globally optimal one-to-one assignment for the matrix objective rather than greedily matching each object independently.

## 13.7 Kalman filter

A recursive estimator alternates predict and update. Prediction uses a motion model; update uses the new measurement weighted according to uncertainty. It provides both an estimated state and covariance.

## 13.8 Hysteresis

Use different thresholds for entering and leaving a state. With obstacle trigger at 0.50 m and clear at 0.60 m, noise around 0.50 no longer causes rapid on/off switching.

## 13.9 Proportional steering

angular.z = -K × horizontal_error. The farther the target is from image center, the stronger the turn. A deadband near center prevents small noise from generating continuous corrections.

## 13.10 Open-loop vs closed-loop control

Open-loop timed PWM assumes the hardware response. Closed-loop control measures output (e.g., encoder speed/distance), calculates error against a target and adjusts commands. The current direct distance/turn maneuvers are open-loop.

## 13.11 Watchdog

A watchdog independently observes whether critical subsystems are alive and whether safety constraints hold. It should fail safe: if necessary information disappears, motion is inhibited rather than assumed safe.

## 13.12 State machine

Behavior is represented by explicit states/flags and defined transitions. This is easier to reason about than many independent callbacks writing motion commands with no priority model.

CHAPTER 14

# Interview question bank with model answers

Practice these until you can answer naturally without memorizing word-for-word.

Q1. Give me the architecture of your robot.

Answer: I split it into sensing, perception, interaction, decision/safety, actuation and web layers. The main motion path is camera -> TensorRT pose detection -> tracking -> decision -> /cmd_vel -> motor bridge -> ESP32 -> BTS7960 -> motors, while LiDAR and an independent watchdog can override motion.

Q2. Why ROS 2?

Answer: I needed a modular distributed architecture where camera, perception, voice, safety and hardware could be developed and debugged independently. ROS 2 also gives standard message types, DDS discovery/QoS, launch/configuration and a large robotics ecosystem.

Q3. Why not put everything in one Python program?

Answer: It would reduce some transport overhead, but it would couple failure modes and make development/debugging harder. Separate nodes let me isolate bottlenecks, restart components, inspect topics and keep the safety watchdog independent.

Q4. Why use TensorRT?

Answer: The robot runs inference on an NVIDIA Jetson. TensorRT turns the exported network into an optimized GPU engine, which gives lower latency and higher/stabler throughput than running the training model path directly.

Q5. Is YOLO running on every frame?

Answer: The detector always keeps only the newest image available to its worker. If inference is busy, the incoming slot is replaced. This intentionally drops stale frames instead of creating latency.

Q6. Why do you need a tracker after YOLO?

Answer: YOLO detections are frame-local. The tracker adds temporal identity, smooth prediction through brief misses and lets the decision node maintain the same person rather than selecting an unrelated detection every frame.

Q7. Explain your tracker.

Answer: Each track has a Kalman state. I predict tracks, compute IoU against new detections, use the Hungarian assignment to find one-to-one matches, update matched tracks, create tracks for unmatched detections and age/remove unmatched tracks.

Q8. What is IoU?

Answer: Intersection area divided by union area for two boxes. I use it as the similarity for associating detections with predicted tracks.

Q9. What does the Kalman filter do here?

Answer: It predicts box state between measurements and corrects the estimate when a new detection arrives. The covariance represents uncertainty, so prediction and measurement are combined systematically.

Q10. What is the Hungarian algorithm doing?

Answer: It solves the assignment problem over the track-detection cost matrix, giving a global one-to-one matching instead of independent greedy matches.

Q11. How do you detect obstacles?

Answer: I convert each LaserScan index to an angle, rotate into the robot’s physical front coordinate, collect front/left/right sectors, use the minimum front distance for obstacle state, and compare side averages to choose the freer direction.

Q12. Why is front_center_angle 180 degrees?

Answer: The physical LiDAR mounting/message convention meant its zero direction did not correspond to the chassis front. I verified that experimentally and explicitly rotate the scan interpretation by 180 degrees.

Q13. How did you stop left-right oscillation?

Answer: I added a direction deadband, separate detect/clear thresholds and latched the chosen avoidance direction while an obstacle event remains active. That prevents noisy scans from changing the turn side every callback.

Q14. How does person following work?

Answer: I use the tracked person box center relative to the 640-pixel image center for steering, and box width as a rough distance proxy. If the box is too large I stop; otherwise I move and steer proportionally.

Q15. Why is box width not real distance?

Answer: It depends on person size, pose and camera geometry. It is a cheap monocular heuristic, not metric depth. For V2 I would prefer depth or navigation/odometry-based distance.

Q16. What happens if the target disappears?

Answer: There is a roughly 2-second reacquisition window. The robot stops during temporary loss. If the target remains absent, autonomous behavior returns to roaming/search.

Q17. How does a voice command reach the motors?

Answer: STT publishes a normalized token on /voice_command. The decision node changes state or starts a timed maneuver, publishes Twist on /cmd_vel, the motor node converts that to signed left/right PWM, and the ESP32 drives the H-bridges.

Q18. Why stop at the wake word?

Answer: The robot should not keep roaming while the user is trying to command it. I save the previous mode, publish zero velocity, process the command, and then either change behavior or restore the previous mode.

Q19. How accurate is your “move one meter” command?

Answer: It is an open-loop timed estimate: 0.25 m/s for 4 seconds. Without wheel encoders it is not guaranteed to be one meter. Encoder odometry and a closed-loop controller are the correct V2 solution.

Q20. What role does the LLM play in motion control?

Answer: It does not directly control the motors. Known robot intents are routed locally and the decision node owns motion. The LLM is for conversational responses and on-demand vision-language questions.

Q21. Why route commands locally before the LLM?

Answer: STOP, follow, UI and battery actions should be deterministic and low latency. Local routing also avoids unnecessary network requests, cost and variability.

Q22. Which models are local and which are remote?

Answer: YOLO pose, Whisper/faster-whisper and Piper are executed on the Jetson stack. The text and vision LLM requests use NVIDIA NIM remotely. The tracker/LiDAR/decision/watchdog are classical algorithms rather than learned models.

Q23. How does scene description work?

Answer: The LLM node caches the latest camera image. On a vision command it converts it to OpenCV, downsizes the longest side to 640 if needed, JPEG/base64 encodes it and sends one multimodal request to the NIM vision model.

Q24. How do you prevent the robot hearing itself?

Answer: TTS publishes SPEAKING/IDLE. While it is speaking, STT drains/ignores microphone audio. After IDLE it waits for a short echo-decay period and flushes buffered audio before listening again.

Q25. Why Piper on CPU?

Answer: TTS is intermittent and practical on CPU, while the GPU is more valuable for continuous perception. It also keeps speech local/offline.

Q26. Why use a persistent audio stream?

Answer: Repeatedly opening/closing the audio device adds latency and can be unreliable, especially with Bluetooth. A persistent output stream lets synthesis only update the playback buffer.

Q27. Why is your watchdog separate?

Answer: A safety monitor inside decision_node would share the same failure domain. A separate node observes raw/derived signals and can publish zero velocity independently if the normal behavior node fails.

Q28. Why monitor raw /scan instead of only /front_distance?

Answer: Raw /scan gives the watchdog an independent path. If obstacle_detection_node itself fails or publishes bad/stale derived data, the watchdog can still inspect the physical LiDAR signal.

Q29. Why did you replace camera images with a heartbeat in the watchdog?

Answer: The watchdog only needs liveness. Receiving 30 FPS full images wasted DDS/Python copying and CPU. A 1 Hz heartbeat tied to successful frames preserves the safety information at a fraction of the cost.

Q30. What happens if multiple safety faults exist?

Answer: The watchdog keeps a set of active reasons. Emergency stop remains asserted until all reasons clear, so recovery of one sensor cannot accidentally release a stop caused by another fault.

Q31. How do you avoid critical-distance chatter?

Answer: Trigger at 0.28 m and clear at 0.35 m. I also require forward motion before triggering the raw-LiDAR proximity reason.

Q32. Explain /cmd_vel to motor PWM.

Answer: For normal driving, base=int(linear.x*250) and turn=int(angular.z*150). Left=base-turn, right=(base+turn)*0.9. I clamp to ±255, apply a deadzone of 15 and send the two integers to the ESP32.

Q33. Why the 0.90 right motor correction?

Answer: The four open-loop DC motors did not have perfectly matched speed. It is a simple feed-forward compensation. Encoders with closed-loop wheel-speed control would be more robust.

Q34. Why an ESP32 at all?

Answer: The Jetson should handle perception and high-level logic, while the MCU provides simple deterministic GPIO/PWM/ADC hardware control and isolates Linux/ROS from driver pin timing.

Q35. What does the ESP32 receive?

Answer: A newline-terminated left,right pair of signed PWM values at 115200 baud. Positive/negative sign chooses the H-bridge direction and magnitude sets PWM duty.

Q36. How is battery status sent back?

Answer: The ESP32 reports a raw ADC sample as BATT:<raw>. The Jetson reconstructs voltage using 4095, 3.3 V and the 100k/33k divider, maps 9.0-12.6 V to a percentage and publishes /battery_status.

Q37. What is the biggest embedded safety gap?

Answer: The firmware should independently stop motors if valid serial commands stop arriving. The ROS watchdog covers the Linux stack, but an MCU communication timeout would cover Jetson/USB failure below ROS.

Q38. What was your biggest performance fix?

Answer: Limiting hidden OpenMP/BLAS threads across the multi-process stack. Oversubscription caused contention and unstable scheduling; setting those libraries to one thread per process improved RAM and detector stability.

Q39. How did you profile the robot?

Answer: I measured per-process CPU/RSS, topic rates, detector latency/FPS, tegrastats GPU/CPU/temperature/power and observed behavior under the full stack. I changed one bottleneck at a time and re-measured.

Q40. What is rosbridge versus web_video_server?

Answer: rosbridge exposes ROS messages/control to browser JavaScript over WebSocket/JSON. web_video_server is specialized for serving ROS image topics as browser-friendly video streams.

Q41. Is this a real AMR if it has no SLAM?

Answer: It is an autonomous mobile robot in the sense that it senses, makes autonomous reactive decisions and actuates itself. It is not yet a map-based Nav2 AMR. I explicitly distinguish reactive autonomy from localization/SLAM/navigation.

Q42. What would you add first for a production-like V2?

Answer: Encoder motors and odometry, MCU command timeout, standard TF frames, IMU fusion, SLAM/localization and Nav2 costmaps/planners. Those changes would replace timed motion and reactive roaming with closed-loop navigation.

Q43. What are the failure modes of cloud LLM dependence?

Answer: Network/API latency or outage affects conversation and scene description, but not core obstacle safety or known motion commands because those are local. That separation is deliberate.

Q44. What would you test automatically?

Answer: Unit tests for parsing/mixing/threshold state transitions, recorded rosbag regression for LiDAR and detections, integration tests for topic liveness and e-stop, and hardware-in-loop serial timeout/stop tests.

Q45. How would you make obstacle avoidance better?

Answer: Use odometry and Nav2 local costmaps/controllers, potentially fuse depth/camera semantics, and turn the current sector logic into a safety layer rather than the main navigation strategy.

Q46. How would you improve following?

Answer: Use metric target range, target re-identification if needed, a smoother controller, velocity/acceleration limits and navigation-aware collision checking rather than only image error and box width.

Q47. What did you learn from the project?

Answer: The main lesson is systems engineering: model speed alone is not enough. Coordinate frames, QoS, thread pools, stale data, feedback loops, motor mismatch, safety ownership and instrumentation all determine whether an AI robotics system works on real hardware.

CHAPTER 15

# Troubleshooting interview scenarios

Use a sensor-to-actuator method instead of guessing.

| Scenario | Debug path / expected reasoning |
| --- | --- |
| Camera topic exists but detector rate is low | Check camera hz and timestamps, detector inference latency, GPU load, worker/latest-frame behavior, thread oversubscription, image size and whether web/instrumentation subscribers increase load. |
| Detector publishes but robot does not follow | Echo /tracked_detections, verify class/person IDs and box geometry, inspect follow_enabled/voice_mode, check obstacle/e-stop state, then verify /cmd_vel. |
| Robot turns away from open space | Validate LiDAR physical front using a known target; inspect front_center_angle_deg and left/right sector signs before tuning thresholds. |
| Robot flips left/right rapidly | Check free-direction noise; use deadband, enter/clear hysteresis and latch direction during an obstacle event. |
| Watchdog says obstacle while robot is stopped | Check whether proximity trigger is gated by last_cmd_linear_x and whether self-published zero commands update the moving state correctly. |
| cmd_vel is nonzero but wheels do not move | Echo /cmd_vel, inspect serial bytes, confirm /dev/ttyACM0, baud, ESP32 parser, driver enables, PWM/deadzone, common ground/power and motor supply. |
| Robot drifts on straight command | Open-loop motor mismatch; check right_motor_correction and battery/load, but explain encoder feedback is the real long-term solution. |
| Battery always reports 0% | Read raw BATT value, measure divider voltage with a meter, verify R1/R2 and ADC pin, check ADC reference/attenuation/calibration and host formula. |
| STT hears TTS | Verify /tts_status transitions, queue draining/flush, echo-decay delay and speaker/microphone placement. |
| Web page lowers perception FPS | web_video_server/rosbridge/browser add subscribers and CPU/network copying; profile with and without UI, lower video stream demand or use compressed/efficient transport. |
| One ROS subscriber receives nothing | Check exact topic/type first, then publisher/subscriber counts and QoS compatibility. |
| Robot keeps moving after a process crash | Safety architecture should assert zero from watchdog; for failures below ROS add an MCU-side command timeout that zeros PWM. |

CHAPTER 16

# Quick-reference cheat sheets

Numbers and phrases worth knowing before an interview.

## 16.1 Core stack

| Item | Remember |
| --- | --- |
| Compute | Jetson Orin Nano 8 GB |
| ROS | ROS 2 Humble in Docker |
| Camera | IMX219-120 CSI; current 1640×1232@30 capture -> 640×480 |
| Detector | YOLOv8n-pose TensorRT engine; conf 0.6; imgsz 640 |
| Tracker | 8-state/4-measurement Kalman + IoU + Hungarian |
| LiDAR | RPLIDAR C1M1; physical front centered at 180° |
| STT | faster-whisper tiny.en; CUDA; int8_float16 |
| Chat LLM | meta/llama-3.1-8b-instruct via NVIDIA NIM |
| Vision LLM | meta/llama-3.2-11b-vision-instruct via NVIDIA NIM |
| TTS | Piper en_US-hfc_female-medium; CPU |
| MCU | ESP32-C3 Super Mini |
| Drivers | 2 × BTS7960 |
| Motors | 4 × 12 V 100 RPM Johnson geared |
| Battery | 11.1 V 6000 mAh 3S2P |
| Main launch | robot_bringup / params_my_robot.launch.py |

## 16.2 Critical thresholds

| Subsystem | Value |
| --- | --- |
| Obstacle trigger / clear | 0.50 m / 0.60 m |
| Obstacle front sector | ±20° around 180° physical front |
| Watchdog critical / clear | 0.28 m / 0.35 m |
| Watchdog forward threshold | linear.x > 0.05 |
| Watchdog rate / startup grace | 5 Hz / 10 s |
| Person target timeout | 2.0 s |
| Frame center / turn threshold | 320 px / 60 px |
| Person stop width | 450 px |
| Direct linear command | 0.25 m/s for 1.0 m => 4 s |
| Turn command | 0.6 rad/s; timing calibrated with measured 1.2 rad/s |
| Serial | /dev/ttyACM0, 115200 |
| PWM clamp / deadzone | ±255 / 15 |
| Battery divider | 100 kΩ + 33 kΩ |
| Battery mapping | 9.0 V empty -> 12.6 V full |

## 16.3 Equations

Equations to remember

angle_i = angle_min + i × angle_increment IoU = intersection_area / union_area error_x = bbox_center_x - image_center_x angular_z = -K × error_x base = linear_x × base_multiplier turn = angular_z × turn_multiplier left = base - turn right = (base + turn) × correction Vadc = raw/4095 × 3.3 Vbattery = Vadc / (33k/(100k+33k))

## 16.4 Three things to never overclaim

Do not call the current reactive roaming “SLAM” or “Nav2”.

Do not call bounding-box width “depth” or timed motion “odometry”.

Do not say the LLM controls safety-critical motion; known motion is deterministic and the watchdog is independent.

## 16.5 Resume/project bullet

Concise project bullet Built and optimized a ROS 2 autonomous mobile robot on NVIDIA Jetson Orin Nano with TensorRT YOLO pose perception, SORT-style tracking, RPLIDAR obstacle avoidance, Whisper/NIM/Piper voice interaction, independent watchdog safety, browser visualization and ESP32/BTS7960 motor control.

CHAPTER 17

# Official references for deeper study

Use these to strengthen theory beyond the project-specific code.

ROS 2 Humble QoS: https://docs.ros.org/en/humble/Concepts/Intermediate/About-Quality-of-Service-Settings.html

NVIDIA TensorRT documentation: https://docs.nvidia.com/deeplearning/tensorrt/latest/index.html

Ultralytics pose task documentation: https://docs.ultralytics.com/tasks/pose/

faster-whisper: https://github.com/SYSTRAN/faster-whisper

SLAMTEC ROS 2 LiDAR driver: https://github.com/Slamtec/sllidar_ros2

rosbridge_suite: https://github.com/RobotWebTools/rosbridge_suite

web_video_server: https://github.com/RobotWebTools/web_video_server

Version note The explanations in Chapters 1-16 prioritize the recovered August 2026 project source snapshots. External documentation may evolve; study the concepts, then verify version-specific APIs when rebuilding the stack.

SOURCE APPENDICES

The following appendices contain the recovered current/recent source snapshots used to prepare this handbook. They are included so you can trace each callback and compare the prose explanation directly with implementation. Third-party driver/server source is not reproduced; only custom project code and the compatible ESP32 reconstruction are included.

# Appendix A - camera_node.py

Perception. Current camera snapshot (1640x1232 capture -> 640x480 working frame).

import threading import time import cv2 import rclpy from cv_bridge import CvBridge from rclpy.node import Node from rclpy.qos import DurabilityPolicy, HistoryPolicy, QoSProfile, ReliabilityPolicy from sensor_msgs.msg import CameraInfo, CompressedImage, Image from std_msgs.msg import Empty def gstreamer_pipeline( camera_type="csi", sensor_id=0, device_id=0, capture_width=1640, capture_height=1232, display_width=640, display_height=480, framerate=30, flip_method=0, ): """Build the GStreamer pipeline for a CSI or USB camera.""" if camera_type == "csi": return ( f"nvarguscamerasrc sensor-id={sensor_id} ! " f"video/x-raw(memory:NVMM), width={capture_width}, " f"height={capture_height}, format=NV12, framerate={framerate}/1 ! " f"nvvidconv flip-method={flip_method} ! " f"video/x-raw, width={display_width}, height={display_height}, " "format=BGRx ! videoconvert ! video/x-raw, format=BGR ! " "appsink drop=true sync=false" ) if camera_type == "usb": return ( f"v4l2src device=/dev/video{device_id} ! " f"video/x-raw, width={capture_width}, height={capture_height}, " f"framerate={framerate}/1 ! " "videoconvert ! video/x-raw, format=BGR ! " "appsink drop=true sync=false" ) raise ValueError(f"Unsupported camera_type: {camera_type}") class CameraNode(Node): def __init__(self): super().__init__("camera_node") self.bridge = CvBridge()

# Camera source and image settings self.declare_parameter("camera_type", "csi") self.declare_parameter("sensor_id", 0) self.declare_parameter("device_id", 0) self.declare_parameter("flip_method", 0) self.declare_parameter("capture_width", 1640) self.declare_parameter("capture_height", 1232) self.declare_parameter("image_width", 640) self.declare_parameter("image_height", 480) self.declare_parameter("camera_fps", 30.0) self.declare_parameter("publish_rate", 30.0) self.declare_parameter("frame_id", "camera_link") self.declare_parameter("jpeg_quality", 80) self.declare_parameter("publish_compressed", False) self.declare_parameter("publish_camera_info", True) # ROS topic and heartbeat settings self.declare_parameter("image_raw_topic", "/camera/image_raw") self.declare_parameter( "image_compressed_topic", "/camera/image_compressed" ) self.declare_parameter("camera_info_topic", "/camera/camera_info") self.declare_parameter("camera_heartbeat_topic", "/camera/heartbeat") self.declare_parameter("camera_heartbeat_rate", 1.0) # Recovery, shutdown, and diagnostic settings self.declare_parameter("enable_reconnect", True) self.declare_parameter("reconnect_after_failures", 30) self.declare_parameter("camera_open_delay_sec", 2.0) self.declare_parameter("shutdown_release_timeout_sec", 2.0) self.declare_parameter("stats_log_interval_sec", 5.0) # Simplified intrinsic camera values self.declare_parameter("camera_fx", 600.0) self.declare_parameter("camera_fy", 600.0) self.camera_type = str(self.get_parameter("camera_type").value) self.sensor_id = int(self.get_parameter("sensor_id").value) self.device_id = int(self.get_parameter("device_id").value) self.flip_method = int(self.get_parameter("flip_method").value) self.capture_width = int(self.get_parameter("capture_width").value) self.capture_height = int(self.get_parameter("capture_height").value) self.image_width = int(self.get_parameter("image_width").value) self.image_height = int(self.get_parameter("image_height").value) self.camera_fps = float(self.get_parameter("camera_fps").value) self.publish_rate = float(self.get_parameter("publish_rate").value) self.frame_id = str(self.get_parameter("frame_id").value) self.jpeg_quality = int(self.get_parameter("jpeg_quality").value) self.publish_compressed = bool( self.get_parameter("publish_compressed").value )

self.publish_camera_info = bool( self.get_parameter("publish_camera_info").value ) self.image_raw_topic = str( self.get_parameter("image_raw_topic").value ) self.image_compressed_topic = str( self.get_parameter("image_compressed_topic").value ) self.camera_info_topic = str( self.get_parameter("camera_info_topic").value ) self.camera_heartbeat_topic = str( self.get_parameter("camera_heartbeat_topic").value ) self.camera_heartbeat_rate = float( self.get_parameter("camera_heartbeat_rate").value ) self.enable_reconnect = bool( self.get_parameter("enable_reconnect").value ) self.reconnect_after_failures = int( self.get_parameter("reconnect_after_failures").value ) self.camera_open_delay_sec = float( self.get_parameter("camera_open_delay_sec").value ) self.shutdown_release_timeout_sec = float( self.get_parameter("shutdown_release_timeout_sec").value ) self.stats_log_interval_sec = float( self.get_parameter("stats_log_interval_sec").value ) self.camera_fx = float(self.get_parameter("camera_fx").value) self.camera_fy = float(self.get_parameter("camera_fy").value) self.failed_reads = 0 self.total_failed_reads = 0 self.total_frames_published = 0 self.last_successful_frame_time = None # Sensor streams use low-latency QoS so old frames are discarded. image_qos = QoSProfile( history=HistoryPolicy.KEEP_LAST, depth=1, reliability=ReliabilityPolicy.BEST_EFFORT, durability=DurabilityPolicy.VOLATILE, ) self.image_pub = self.create_publisher( Image, self.image_raw_topic, image_qos )

self.compressed_pub = self.create_publisher( CompressedImage, self.image_compressed_topic, image_qos ) self.camera_info_pub = self.create_publisher( CameraInfo, self.camera_info_topic, image_qos ) self.camera_heartbeat_pub = self.create_publisher( Empty, self.camera_heartbeat_topic, 1 ) self.cap = None self.open_camera() self.timer = self.create_timer( 1.0 / self.publish_rate, self.timer_callback ) self.camera_heartbeat_timer = self.create_timer( 1.0 / self.camera_heartbeat_rate, self.publish_camera_heartbeat, ) self.get_logger().info("✅ Camera node started successfully") def open_camera(self): """Open the configured camera and use OpenCV capture as a fallback.""" if self.cap is not None: self.cap.release() pipeline = gstreamer_pipeline( camera_type=self.camera_type, sensor_id=self.sensor_id, device_id=self.device_id, capture_width=self.capture_width, capture_height=self.capture_height, display_width=self.image_width, display_height=self.image_height, framerate=int(self.camera_fps), flip_method=self.flip_method, ) self.get_logger().info(f"🎥 Pipeline: {pipeline}") self.cap = cv2.VideoCapture(pipeline, cv2.CAP_GSTREAMER) time.sleep(self.camera_open_delay_sec) if not self.cap.isOpened(): self.get_logger().warn( "⚠️ GStreamer failed. Falling back to OpenCV..." ) self.cap = cv2.VideoCapture(self.device_id) if not self.cap.isOpened(): raise RuntimeError("❌ Camera could not be opened")

self.get_logger().info("✅ Camera opened") def create_camera_info_msg(self, stamp): """Create the basic CameraInfo message associated with a frame.""" msg = CameraInfo() msg.header.stamp = stamp msg.header.frame_id = self.frame_id msg.width = self.image_width msg.height = self.image_height msg.k = [ self.camera_fx, 0.0, self.image_width / 2, 0.0, self.camera_fy, self.image_height / 2, 0.0, 0.0, 1.0, ] return msg def publish_camera_heartbeat(self): """Publish a heartbeat only while frames are being received.""" if self.last_successful_frame_time is None: return frame_age = time.monotonic() - self.last_successful_frame_time if frame_age <= 2.0: self.camera_heartbeat_pub.publish(Empty()) def timer_callback(self): """Read and publish one frame, reconnecting after repeated failures.""" ret, frame = self.cap.read() if not ret: self.failed_reads += 1 self.total_failed_reads += 1 if ( self.enable_reconnect and self.failed_reads >= self.reconnect_after_failures ): self.get_logger().warn("🔁 Reconnecting camera...") self.open_camera() self.failed_reads = 0 return self.failed_reads = 0 self.last_successful_frame_time = time.monotonic() stamp = self.get_clock().now().to_msg()

img_msg = self.bridge.cv2_to_imgmsg(frame, "bgr8") img_msg.header.stamp = stamp img_msg.header.frame_id = self.frame_id self.image_pub.publish(img_msg) if self.publish_compressed: success, encoded = cv2.imencode( ".jpg", frame, [int(cv2.IMWRITE_JPEG_QUALITY), self.jpeg_quality], ) if success: comp_msg = CompressedImage() comp_msg.header.stamp = stamp comp_msg.header.frame_id = self.frame_id comp_msg.format = "jpeg" comp_msg.data = encoded.tobytes() self.compressed_pub.publish(comp_msg) if self.publish_camera_info: self.camera_info_pub.publish( self.create_camera_info_msg(stamp) ) self.total_frames_published += 1 def destroy_node(self): """Release the camera without allowing shutdown to block indefinitely.""" if self.cap: cap = self.cap self.cap = None def _release(): try: cap.release() except Exception as error: self.get_logger().warn( f"Camera release error: {error}" ) release_thread = threading.Thread(target=_release, daemon=True) release_thread.start() release_thread.join(timeout=self.shutdown_release_timeout_sec) super().destroy_node() def main(args=None): rclpy.init(args=args) node = CameraNode() try:

rclpy.spin(node) except KeyboardInterrupt: if rclpy.ok(): node.get_logger().info("Camera node stopping...") finally: node.destroy_node() if rclpy.ok(): rclpy.shutdown() if __name__ == "__main__": main()

# Appendix B - detector_node.py

Perception. YOLOv8n-pose TensorRT detector with hand-gesture audio control.

import threading import time import cv2 import numpy as np import rclpy from cv_bridge import CvBridge from rclpy.callback_groups import ReentrantCallbackGroup from rclpy.executors import MultiThreadedExecutor from rclpy.node import Node from rclpy.qos import ( DurabilityPolicy, HistoryPolicy, QoSProfile, ReliabilityPolicy, ) from sensor_msgs.msg import Image from std_msgs.msg import Bool from ultralytics import YOLO from vision_msgs.msg import Detection2D, Detection2DArray, ObjectHypothesisWithPose # Standard YOLOv8-pose keypoint indices used for raised-hand detection. KP_LEFT_SHOULDER = 5 KP_RIGHT_SHOULDER = 6 KP_LEFT_WRIST = 9 KP_RIGHT_WRIST = 10 KP_CONF_MIN = 0.4 class DetectorNode(Node): def __init__(self): super().__init__("detector_node") # Model and detection parameters self.declare_parameter( "model_path", "/workspace/models/vision/yolov8n-pose.engine" ) self.declare_parameter("conf_threshold", 0.6) self.declare_parameter("imgsz", 640) self.declare_parameter("persistence_time", 0.3) self.declare_parameter("gesture_hold_sec", 1.0) self.declare_parameter("gesture_cooldown_sec", 5.0) self.declare_parameter("image_topic", "/camera/image_raw") self.declare_parameter("detections_topic", "/detections") self.declare_parameter("audio_enabled_topic", "/audio_enabled") self.declare_parameter("warmup", True) self.model_path = str(self.get_parameter("model_path").value) self.conf_threshold = float(self.get_parameter("conf_threshold").value) self.imgsz = int(self.get_parameter("imgsz").value) self.persistence_time = float(self.get_parameter("persistence_time").value) self.gesture_hold_sec = float(self.get_parameter("gesture_hold_sec").value)

self.gesture_cooldown_sec = float( self.get_parameter("gesture_cooldown_sec").value ) self.image_topic = str(self.get_parameter("image_topic").value) self.detections_topic = str(self.get_parameter("detections_topic").value) self.audio_enabled_topic = str( self.get_parameter("audio_enabled_topic").value ) self.warmup = bool(self.get_parameter("warmup").value) self.bridge = CvBridge() self.model = YOLO(self.model_path, task="pose") if self.warmup: dummy = np.zeros((self.imgsz, self.imgsz, 3), dtype=np.uint8) self.model(dummy, imgsz=self.imgsz, verbose=False) self.get_logger().info(f"🚀 Pose model loaded: {self.model_path}") # The ROS callback stores only the newest image; inference runs separately. self.callback_group = ReentrantCallbackGroup() self.latest_msg = None self.frame_ready_event = threading.Event() self.last_detections = None self.last_detection_time = 0.0 # Left hand disables audio; right hand enables it after a sustained gesture. self.audio_enabled = True self.left_hand_raised_since = None self.right_hand_raised_since = None self.last_gesture_trigger = 0.0 image_qos = QoSProfile( history=HistoryPolicy.KEEP_LAST, depth=1, reliability=ReliabilityPolicy.BEST_EFFORT, durability=DurabilityPolicy.VOLATILE, ) detection_qos = QoSProfile( history=HistoryPolicy.KEEP_LAST, depth=10, reliability=ReliabilityPolicy.RELIABLE, durability=DurabilityPolicy.VOLATILE, ) audio_qos = QoSProfile( history=HistoryPolicy.KEEP_LAST, depth=1, reliability=ReliabilityPolicy.RELIABLE, durability=DurabilityPolicy.TRANSIENT_LOCAL, )

self.image_sub = self.create_subscription( Image, self.image_topic, self.image_callback, image_qos, callback_group=self.callback_group, ) self.audio_state_sub = self.create_subscription( Bool, self.audio_enabled_topic, self._audio_state_callback, audio_qos, ) self.detections_pub = self.create_publisher( Detection2DArray, self.detections_topic, detection_qos, ) self.audio_pub = self.create_publisher( Bool, self.audio_enabled_topic, audio_qos, ) self.running = True self.worker_thread = threading.Thread( target=self.worker_loop, daemon=True, ) self.worker_thread.start() self.frame_count = 0 self.last_time = time.time() self.get_logger().info("✅ YOLO Pose Detector Node Started") def _audio_state_callback(self, msg: Bool): self.audio_enabled = msg.data def image_callback(self, msg: Image): if not self.running: return self.latest_msg = msg self.frame_ready_event.set() def worker_loop(self): while rclpy.utilities.ok() and self.running: if not self.frame_ready_event.wait(timeout=0.1): continue self.frame_ready_event.clear()

if not self.running or not rclpy.utilities.ok(): break if self.latest_msg is None: continue msg = self.latest_msg self.latest_msg = None self.process_frame(msg) def _check_hand_raised( self, keypoints_xy, keypoints_conf, wrist_idx, shoulder_idx, ): if ( keypoints_conf[wrist_idx] < KP_CONF_MIN or keypoints_conf[shoulder_idx] < KP_CONF_MIN ): return False return keypoints_xy[wrist_idx][1] < keypoints_xy[shoulder_idx][1] def _handle_gesture(self, left_raised, right_raised): now = time.time() self.left_hand_raised_since = ( now if left_raised and self.left_hand_raised_since is None else self.left_hand_raised_since if left_raised else None ) self.right_hand_raised_since = ( now if right_raised and self.right_hand_raised_since is None else self.right_hand_raised_since if right_raised else None ) if now - self.last_gesture_trigger < self.gesture_cooldown_sec: return if ( self.left_hand_raised_since is not None and now - self.left_hand_raised_since >= self.gesture_hold_sec and self.audio_enabled ): self.audio_enabled = False self.last_gesture_trigger = now self.left_hand_raised_since = None self.right_hand_raised_since = None

msg = Bool() msg.data = False self.audio_pub.publish(msg) self.get_logger().info("🖐️ Left-hand gesture detected — audio disabled") return if ( self.right_hand_raised_since is not None and now - self.right_hand_raised_since >= self.gesture_hold_sec and not self.audio_enabled ): self.audio_enabled = True self.last_gesture_trigger = now self.left_hand_raised_since = None self.right_hand_raised_since = None msg = Bool() msg.data = True self.audio_pub.publish(msg) self.get_logger().info("🖐️ Right-hand gesture detected — audio re-enabled") def process_frame(self, msg: Image): start_time = time.time() try: frame = self.bridge.imgmsg_to_cv2(msg, desired_encoding="bgr8") results = self.model(frame, imgsz=self.imgsz, verbose=False) detection_array = Detection2DArray() detection_array.header = msg.header detection_found = False any_left_raised = False any_right_raised = False for result in results: if result.boxes is None: continue # Pose keypoint arrays align index-for-index with detected boxes. kp_xy_all = ( result.keypoints.xy.cpu().numpy() if result.keypoints is not None else None ) kp_conf_all = ( result.keypoints.conf.cpu().numpy() if result.keypoints is not None and result.keypoints.conf is not None else None )

for index, box in enumerate(result.boxes): confidence = float(box.conf[0].item()) if confidence < self.conf_threshold: continue class_index = int(box.cls[0].item()) class_name = self.model.names[class_index] cx, cy, box_width, box_height = box.xywh[0].cpu().numpy() detection = Detection2D() detection.header = msg.header detection.bbox.center.position.x = float(cx) detection.bbox.center.position.y = float(cy) detection.bbox.size_x = float(box_width) detection.bbox.size_y = float(box_height) hypothesis = ObjectHypothesisWithPose() hypothesis.hypothesis.class_id = class_name hypothesis.hypothesis.score = confidence detection.results.append(hypothesis) detection_array.detections.append(detection) detection_found = True if ( kp_xy_all is not None and kp_conf_all is not None and index < len(kp_xy_all) ): if self._check_hand_raised( kp_xy_all[index], kp_conf_all[index], KP_LEFT_WRIST, KP_LEFT_SHOULDER, ): any_left_raised = True if self._check_hand_raised( kp_xy_all[index], kp_conf_all[index], KP_RIGHT_WRIST, KP_RIGHT_SHOULDER, ): any_right_raised = True self._handle_gesture(any_left_raised, any_right_raised) # Reuse recent detections briefly to reduce visible detection flicker. current_time = time.time() if detection_found: self.last_detections = detection_array self.last_detection_time = current_time

elif current_time - self.last_detection_time < self.persistence_time: detection_array = self.last_detections if detection_array is not None and rclpy.utilities.ok() and self.running: self.detections_pub.publish(detection_array) self.frame_count += 1 if self.frame_count % 30 == 0: now = time.time() fps = 30 / (now - self.last_time) self.last_time = now latency = (time.time() - start_time) * 1000 if rclpy.utilities.ok() and self.running: self.get_logger().info( f"⚡ FPS={fps:.2f} ¦ Latency={latency:.1f} ms ¦ " f"Detections={len(detection_array.detections)}" ) except Exception as error: if rclpy.utilities.ok() and self.running: self.get_logger().error(f"❌ Detection error: {error}") def destroy_node(self): self.running = False self.frame_ready_event.set() if hasattr(self, "worker_thread") and self.worker_thread.is_alive(): self.worker_thread.join(timeout=0.5) super().destroy_node() def main(args=None): rclpy.init(args=args) node = DetectorNode() executor = MultiThreadedExecutor(num_threads=4) executor.add_node(node) try: executor.spin() except KeyboardInterrupt: if rclpy.ok(): node.get_logger().info("Detector node stopping...") finally: node.destroy_node() if rclpy.ok(): rclpy.shutdown() if __name__ == "__main__":

main()

# Appendix C - tracking_node.py

Perception. SORT-style tracking with Kalman prediction, IoU and Hungarian assignment.

import numpy as np import rclpy from rclpy.node import Node from scipy.optimize import linear_sum_assignment from filterpy.kalman import KalmanFilter from vision_msgs.msg import ( Detection2DArray, Detection2D, ObjectHypothesisWithPose, ) from rclpy.qos import ( QoSProfile, ReliabilityPolicy, HistoryPolicy, DurabilityPolicy, ) class Track: def __init__( self, bbox, class_name, score, track_id, covariance_scale=10.0, measurement_noise=1.0, process_noise=0.01, ): self.id = track_id self.class_name = class_name self.score = score self.hits = 1 self.age = 0 self.covariance_scale = covariance_scale self.measurement_noise = measurement_noise self.process_noise = process_noise self.kf = self.create_kf(bbox) def create_kf(self, bbox): x1, y1, x2, y2 = bbox cx = (x1 + x2) / 2 cy = (y1 + y2) / 2 w = x2 - x1 h = y2 - y1 kf = KalmanFilter(dim_x=8, dim_z=4) kf.F = np.array([ [1, 0, 0, 0, 1, 0, 0, 0],

[0, 1, 0, 0, 0, 1, 0, 0], [0, 0, 1, 0, 0, 0, 1, 0], [0, 0, 0, 1, 0, 0, 0, 1], [0, 0, 0, 0, 1, 0, 0, 0], [0, 0, 0, 0, 0, 1, 0, 0], [0, 0, 0, 0, 0, 0, 1, 0], [0, 0, 0, 0, 0, 0, 0, 1], ]) kf.H = np.array([ [1, 0, 0, 0, 0, 0, 0, 0], [0, 1, 0, 0, 0, 0, 0, 0], [0, 0, 1, 0, 0, 0, 0, 0], [0, 0, 0, 1, 0, 0, 0, 0], ]) kf.P *= self.covariance_scale kf.R *= self.measurement_noise kf.Q *= self.process_noise kf.x = np.array([ [cx], [cy], [w], [h], [0], [0], [0], [0], ]) return kf def predict(self): self.kf.predict() self.age += 1 def update(self, bbox, score): x1, y1, x2, y2 = bbox cx = (x1 + x2) / 2 cy = (y1 + y2) / 2 w = x2 - x1 h = y2 - y1 z = np.array([ [cx], [cy], [w], [h], ]) self.kf.update(z)

self.score = score self.age = 0 self.hits += 1 def bbox(self): cx = self.kf.x[0][0] cy = self.kf.x[1][0] w = self.kf.x[2][0] h = self.kf.x[3][0] return [ cx - w / 2, cy - h / 2, cx + w / 2, cy + h / 2, ] class TrackerNode(Node): def __init__(self): super().__init__('tracker_node') # ----------------------------- # Parameters # ----------------------------- self.declare_parameter('detections_topic', '/detections') self.declare_parameter( 'tracked_detections_topic', '/tracked_detections', ) self.declare_parameter('iou_threshold', 0.3) self.declare_parameter('max_age', 30) self.declare_parameter('min_hits', 4) self.declare_parameter('max_publish_age', 2) self.declare_parameter('kalman_covariance_scale', 10.0) self.declare_parameter('kalman_measurement_noise', 1.0) self.declare_parameter('kalman_process_noise', 0.01) # ----------------------------- # Get parameters # ----------------------------- self.detections_topic = str( self.get_parameter('detections_topic').value ) self.tracked_detections_topic = str( self.get_parameter('tracked_detections_topic').value ) self.iou_threshold = float( self.get_parameter('iou_threshold').value

) self.max_age = int( self.get_parameter('max_age').value ) self.min_hits = int( self.get_parameter('min_hits').value ) self.max_publish_age = int( self.get_parameter('max_publish_age').value ) self.kalman_covariance_scale = float( self.get_parameter('kalman_covariance_scale').value ) self.kalman_measurement_noise = float( self.get_parameter('kalman_measurement_noise').value ) self.kalman_process_noise = float( self.get_parameter('kalman_process_noise').value ) self.tracks = [] self.next_track_id = 1 qos = QoSProfile( history=HistoryPolicy.KEEP_LAST, depth=10, reliability=ReliabilityPolicy.RELIABLE, durability=DurabilityPolicy.VOLATILE, ) self.sub = self.create_subscription( Detection2DArray, self.detections_topic, self.detection_callback, qos, ) self.pub = self.create_publisher( Detection2DArray, self.tracked_detections_topic, qos, ) self.get_logger().info('Real SORT tracker started') def det_to_xyxy(self, det): cx = det.bbox.center.position.x cy = det.bbox.center.position.y w = det.bbox.size_x h = det.bbox.size_y

return [ cx - w / 2, cy - h / 2, cx + w / 2, cy + h / 2, ] def iou(self, a, b): x_a = max(a[0], b[0]) y_a = max(a[1], b[1]) x_b = min(a[2], b[2]) y_b = min(a[3], b[3]) inter = max(0, x_b - x_a) * max(0, y_b - y_a) if inter == 0: return 0.0 area_a = (a[2] - a[0]) * (a[3] - a[1]) area_b = (b[2] - b[0]) * (b[3] - b[1]) return inter / (area_a + area_b - inter) def assign(self, detections): if len(self.tracks) == 0: return [], [], list(range(len(detections))) cost = np.ones((len(self.tracks), len(detections))) for i, track in enumerate(self.tracks): for j, detection in enumerate(detections): if track.class_name != detection['class_name']: cost[i, j] = 1.0 else: cost[i, j] = 1 - self.iou( track.bbox(), detection['bbox'], ) rows, cols = linear_sum_assignment(cost) matches = [] unmatched_tracks = list(range(len(self.tracks))) unmatched_dets = list(range(len(detections))) for row, col in zip(rows, cols): if 1 - cost[row, col] < self.iou_threshold: continue matches.append((row, col)) if row in unmatched_tracks:

unmatched_tracks.remove(row) if col in unmatched_dets: unmatched_dets.remove(col) return matches, unmatched_tracks, unmatched_dets def detection_callback(self, msg): detections = [] for det in msg.detections: if len(det.results) == 0: continue detections.append({ 'bbox': self.det_to_xyxy(det), 'class_name': det.results[0].hypothesis.class_id, 'score': det.results[0].hypothesis.score, }) for track in self.tracks: track.predict() matches, unmatched_tracks, unmatched_dets = self.assign( detections ) for track_index, detection_index in matches: detection = detections[detection_index] self.tracks[track_index].update( detection['bbox'], detection['score'], ) for detection_index in unmatched_dets: detection = detections[detection_index] self.tracks.append( Track( detection['bbox'], detection['class_name'], detection['score'], self.next_track_id, covariance_scale=self.kalman_covariance_scale, measurement_noise=self.kalman_measurement_noise, process_noise=self.kalman_process_noise, ) ) self.next_track_id += 1 self.tracks = [

track for track in self.tracks if track.age <= self.max_age ] out = Detection2DArray() out.header = msg.header for track in self.tracks: if track.hits < self.min_hits: continue if track.age > self.max_publish_age: continue x1, y1, x2, y2 = track.bbox() det = Detection2D() det.header = msg.header det.id = str(track.id) det.bbox.center.position.x = (x1 + x2) / 2 det.bbox.center.position.y = (y1 + y2) / 2 det.bbox.size_x = x2 - x1 det.bbox.size_y = y2 - y1 hypothesis = ObjectHypothesisWithPose() hypothesis.hypothesis.class_id = track.class_name hypothesis.hypothesis.score = track.score det.results.append(hypothesis) out.detections.append(det) self.pub.publish(out) def main(args=None): rclpy.init(args=args) node = TrackerNode() try: rclpy.spin(node) except KeyboardInterrupt: if rclpy.ok(): node.get_logger().info('Tracker node stopping...') finally: node.destroy_node() if rclpy.ok(): rclpy.shutdown()

if __name__ == '__main__': main()

# Appendix D - obstacle_detection_node.py

Perception. LiDAR sector processing with detection/clear hysteresis and direction lock.

import math import time import rclpy from rclpy.node import Node from sensor_msgs.msg import LaserScan from std_msgs.msg import Bool, Float32, String class ObstacleDetectionNode(Node): """Detect front obstacles and identify the side with more free space.""" def __init__(self): super().__init__('obstacle_detection_node') # ROS topics self.declare_parameter('scan_topic', '/scan') self.declare_parameter('obstacle_topic', '/obstacle_detected') self.declare_parameter('front_distance_topic', '/front_distance') self.declare_parameter('free_direction_topic', '/free_direction') # Detection and direction-selection settings self.declare_parameter('obstacle_distance_threshold', 0.50) self.declare_parameter('obstacle_clear_threshold', 0.60) self.declare_parameter('front_angle_range', 20) self.declare_parameter('front_center_angle_deg', 180.0) self.declare_parameter('left_sector_start', 30) self.declare_parameter('left_sector_end', 90) self.declare_parameter('right_sector_start', 90) self.declare_parameter('right_sector_end', 30) self.declare_parameter('direction_deadband', 0.30) self.declare_parameter('default_direction', 'LEFT') self.declare_parameter('stats_log_interval_sec', 1.0) self.scan_topic = str(self.get_parameter('scan_topic').value) self.obstacle_topic = str(self.get_parameter('obstacle_topic').value) self.front_distance_topic = str( self.get_parameter('front_distance_topic').value ) self.free_direction_topic = str( self.get_parameter('free_direction_topic').value ) self.obstacle_distance_threshold = float( self.get_parameter('obstacle_distance_threshold').value ) self.obstacle_clear_threshold = float( self.get_parameter('obstacle_clear_threshold').value ) self.front_angle_range = float( self.get_parameter('front_angle_range').value ) self.front_center_angle_deg = float(

self.get_parameter('front_center_angle_deg').value ) self.left_sector_start = float( self.get_parameter('left_sector_start').value ) self.left_sector_end = float( self.get_parameter('left_sector_end').value ) self.right_sector_start = float( self.get_parameter('right_sector_start').value ) self.right_sector_end = float( self.get_parameter('right_sector_end').value ) self.direction_deadband = float( self.get_parameter('direction_deadband').value ) self.default_direction = str( self.get_parameter('default_direction').value ).upper() self.stats_log_interval_sec = float( self.get_parameter('stats_log_interval_sec').value ) self.last_stats_log_time = 0.0 self.previous_obstacle_state = False self.obstacle_active = False self.avoidance_direction = self.default_direction self.scan_subscriber = self.create_subscription( LaserScan, self.scan_topic, self.scan_callback, 10, ) self.obstacle_publisher = self.create_publisher( Bool, self.obstacle_topic, 10, ) self.distance_publisher = self.create_publisher( Float32, self.front_distance_topic, 10, ) self.free_direction_publisher = self.create_publisher( String, self.free_direction_topic, 10, ) self.get_logger().info('Obstacle Detection Node Started')

def scan_callback(self, msg: LaserScan): """Process one LiDAR scan and publish obstacle information.""" front_ranges = [] left_ranges = [] right_ranges = [] front_angle = math.radians(self.front_angle_range) front_center = math.radians(self.front_center_angle_deg) left_start = math.radians(self.left_sector_start) left_end = math.radians(self.left_sector_end) right_start = -math.radians(self.right_sector_start) right_end = -math.radians(self.right_sector_end) for index, distance in enumerate(msg.ranges): if distance <= msg.range_min or distance > msg.range_max: continue if math.isinf(distance) or math.isnan(distance): continue angle = msg.angle_min + index * msg.angle_increment relative_angle = angle - front_center relative_angle = math.atan2( math.sin(relative_angle), math.cos(relative_angle), ) if abs(relative_angle) <= front_angle: front_ranges.append(distance) if left_start <= relative_angle <= left_end: left_ranges.append(distance) if right_start <= relative_angle <= right_end: right_ranges.append(distance) if not front_ranges: self.get_logger().warn('No valid front LiDAR data') self.publish_obstacle_status(True, 0.0) return min_front_distance = min(front_ranges) left_avg = ( sum(left_ranges) / len(left_ranges) if left_ranges else 0.0 )

right_avg = ( sum(right_ranges) / len(right_ranges) if right_ranges else 0.0 ) difference = abs(left_avg - right_avg) if difference < self.direction_deadband: candidate_direction = self.default_direction elif left_avg > right_avg: candidate_direction = 'LEFT' else: candidate_direction = 'RIGHT' if self.obstacle_active: obstacle_detected = ( min_front_distance < self.obstacle_clear_threshold ) else: obstacle_detected = ( min_front_distance < self.obstacle_distance_threshold ) if obstacle_detected and not self.obstacle_active: self.obstacle_active = True self.avoidance_direction = candidate_direction self.get_logger().warn( f'Obstacle Detected at {min_front_distance:.2f} m ¦ ' f'Avoiding {self.avoidance_direction}' ) elif not obstacle_detected and self.obstacle_active: self.obstacle_active = False self.get_logger().info('Front path clear') direction_msg = String() if self.obstacle_active: direction_msg.data = self.avoidance_direction else: direction_msg.data = candidate_direction current_time = time.monotonic() if ( current_time - self.last_stats_log_time >= self.stats_log_interval_sec ): self.get_logger().info( f'Left:{left_avg:.2f}m ¦ ' f'Right:{right_avg:.2f}m ¦ '

f'Front:{min_front_distance:.2f}m ¦ ' f'Direction:{direction_msg.data}' ) self.last_stats_log_time = current_time self.previous_obstacle_state = self.obstacle_active self.free_direction_publisher.publish(direction_msg) self.publish_obstacle_status( self.obstacle_active, min_front_distance, ) def publish_obstacle_status(self, detected: bool, distance: float): """Publish the obstacle flag and minimum front distance.""" obstacle_msg = Bool() obstacle_msg.data = detected distance_msg = Float32() distance_msg.data = float(distance) self.obstacle_publisher.publish(obstacle_msg) self.distance_publisher.publish(distance_msg) def main(args=None): rclpy.init(args=args) node = ObstacleDetectionNode() try: rclpy.spin(node) except KeyboardInterrupt: if rclpy.ok(): node.get_logger().info('Obstacle detection node stopping...') finally: node.destroy_node() if rclpy.ok(): rclpy.shutdown() if __name__ == '__main__': main()

# Appendix E - stt_node.py

Interaction. Whisper tiny.en wake-word + command speech recognition.

import time import rclpy from rclpy.node import Node from std_msgs.msg import String, Bool from faster_whisper import WhisperModel import sounddevice as sd import numpy as np import threading import queue from scipy.signal import resample_poly class STTNode(Node): def __init__(self): super().__init__('stt_node') self._stop_event = threading.Event() self.declare_parameter('voice_command_topic', '/voice_command') self.declare_parameter('tts_status_topic', '/tts_status') self.declare_parameter('audio_enabled_topic', '/audio_enabled') self.declare_parameter('model_name', 'tiny.en') self.declare_parameter('device', 'cuda') self.declare_parameter('compute_type', 'int8_float16') self.declare_parameter('sample_rate', 44100) self.declare_parameter('chunk_size', 4410) self.declare_parameter('whisper_rate', 16000) self.declare_parameter('channels', 1) self.declare_parameter('audio_dtype', 'float32') self.declare_parameter('input_device', '') self.declare_parameter('rms_wake', 600.0) self.declare_parameter('rms_command', 150.0) self.declare_parameter('silence_sec', 1.5) self.declare_parameter('command_max_sec', 6.0) self.declare_parameter('tts_echo_decay_sec', 0.6) self.declare_parameter('ack_start_timeout', 0.8) self.declare_parameter('ack_total_timeout', 8.0) self.voice_command_topic = self.get_parameter('voice_command_topic').value self.tts_status_topic = self.get_parameter('tts_status_topic').value self.audio_enabled_topic = self.get_parameter('audio_enabled_topic').value self.model_name = self.get_parameter('model_name').value self.device = self.get_parameter('device').value self.compute_type = self.get_parameter('compute_type').value self.sample_rate = self.get_parameter('sample_rate').value self.chunk_size = self.get_parameter('chunk_size').value self.whisper_rate = self.get_parameter('whisper_rate').value self.channels = self.get_parameter('channels').value

self.audio_dtype = self.get_parameter('audio_dtype').value self.input_device = self.get_parameter('input_device').value self.rms_wake = self.get_parameter('rms_wake').value self.rms_command = self.get_parameter('rms_command').value self.silence_sec = self.get_parameter('silence_sec').value self.command_max_sec = self.get_parameter('command_max_sec').value self.tts_echo_decay_sec = self.get_parameter('tts_echo_decay_sec').value self.ack_start_timeout = self.get_parameter('ack_start_timeout').value self.ack_total_timeout = self.get_parameter('ack_total_timeout').value self.pub = self.create_publisher(String, self.voice_command_topic, 10) self.create_subscription( String, self.tts_status_topic, self._tts_status_callback, 10 ) self.is_tts_speaking = False # Audio ON/OFF toggle (webpage button or raised-hand gesture). self.audio_enabled = True self.create_subscription( Bool, self.audio_enabled_topic, self._audio_enabled_callback, 10 ) self.get_logger().info( f'Loading Whisper ({self.model_name}, {self.device}, {self.compute_type})...' ) # int8_float16: int8 weights, float16 compute — good speed/accuracy # balance on Jetson GPUs. Try "int8" (pure int8) if you want to # push speed further, or "float16" if accuracy matters more. self.model = WhisperModel( self.model_name, device=self.device, compute_type=self.compute_type ) self.audio_q = queue.Queue() stream_kwargs = { 'samplerate': self.sample_rate, 'channels': self.channels, 'dtype': self.audio_dtype, 'blocksize': self.chunk_size, 'callback': lambda i, f, t, s: self.audio_q.put(i.copy().flatten()), }

if self.input_device: stream_kwargs['device'] = self.input_device self.stream = sd.InputStream(**stream_kwargs) self.stream.start() self._stop_event = threading.Event() self._listen_thread = threading.Thread(target=self._listen_loop, daemon=True) self._listen_thread.start() self.get_logger().info('★ Ready. Say "Vector".') # ─── TTS mute / unmute ────────────────────────────────────── def _tts_status_callback(self, msg: String): status = msg.data.upper() if status == 'SPEAKING': self.is_tts_speaking = True self.get_logger().info('🔇 TTS Active: muting STT.') elif status == 'IDLE': time.sleep(self.tts_echo_decay_sec) # let room echo decay self._flush_queue() self.is_tts_speaking = False self.get_logger().info('🔊 TTS Idle: STT resumed.') def _audio_enabled_callback(self, msg: Bool): was_enabled = self.audio_enabled self.audio_enabled = msg.data if was_enabled and not self.audio_enabled: self.get_logger().info('🔇 Audio mode OFF — ignoring wake word until re-enabled.') self._flush_queue() elif not was_enabled and self.audio_enabled: self.get_logger().info('🔊 Audio mode ON — listening for "Vector" again.') self._flush_queue() def _flush_queue(self): while not self.audio_q.empty(): try: self.audio_q.get_nowait() except queue.Empty: break def _wait_for_ack_tts(self, start_timeout=None, total_timeout=None): """ After publishing WAKE_WORD_DETECTED, tts_node speaks a short "Yes?" acknowledgment. Wait for that to actually start (SPEAKING) and then finish (IDLE) before opening the real command-listening window — otherwise _collect_until_silence races the ack and gets killed by the is_tts_speaking mute check within milliseconds. """ if start_timeout is None: start_timeout = self.ack_start_timeout if total_timeout is None: total_timeout = self.ack_total_timeout

start = time.time() while not self.is_tts_speaking and (time.time() - start) < start_timeout: time.sleep(0.02) while self.is_tts_speaking and (time.time() - start) < total_timeout: time.sleep(0.02) # ─── Resample + transcribe ────────────────────────────────── def _to_whisper(self, audio: np.ndarray) -> np.ndarray: """Resample from 44100 → 16000 for Whisper.""" return resample_poly(audio, self.whisper_rate, self.sample_rate).astype(np.float32) def _transcribe(self, audio: np.ndarray, prompt: str) -> str: audio_16k = self._to_whisper(audio) # faster-whisper accepts a numpy float32 array directly — no need # to round-trip through a temp WAV file like openai-whisper required. segments, info = self.model.transcribe( audio_16k, language='en', initial_prompt=prompt, beam_size=1, best_of=1, temperature=0.0, no_speech_threshold=0.6, ) text_parts = [] for seg in segments: # Hallucination guard — compression ratio (same check as before) if getattr(seg, 'compression_ratio', 0) > 2.4: self.get_logger().warn('[STT] Hallucination detected — ignored') return '' text_parts.append(seg.text) text = ''.join(text_parts).lower().strip() # Hallucination guard — word repetition loop words = text.split() if len(words) > 5: top = max(set(words), key=words.count) if words.count(top) / len(words) > 0.5: self.get_logger().warn(f'[STT] Repetition loop on "{top}" — ignored') return '' return text # ─── Collect audio until silence ──────────────────────────── def _collect_until_silence(self, max_sec=6.0, prefill=None) -> np.ndarray: """ Collect audio chunks until silence or timeout. prefill: list of chunks already captured (e.g. tail of wake word audio)

""" silence_chunks = int(self.silence_sec * self.sample_rate / self.chunk_size) max_chunks = int(max_sec * self.sample_rate / self.chunk_size) # Seed buffer with any audio already captured buf = list(prefill) if prefill else [] silent = 0 started = False # Check if prefill already contains voice for chunk in buf: rms = np.sqrt(np.mean(chunk ** 2)) * 32767 if rms > self.rms_command: started = True break self.get_logger().info( f'[CMD] collecting — prefill:{len(buf)} chunks, ' f'voice_detected:{started}, threshold:{self.rms_command}' ) remaining = max_chunks - len(buf) for _ in range(remaining): if self.is_tts_speaking or not self.audio_enabled: return np.array([], dtype=np.float32) try: chunk = self.audio_q.get(timeout=0.3) except queue.Empty: # If we already heard voice, silence counts as end of speech if started: silent += 1 if silent >= silence_chunks: break continue buf.append(chunk) rms = np.sqrt(np.mean(chunk ** 2)) * 32767 if rms > self.rms_command: started = True silent = 0 elif started: silent += 1 if silent >= silence_chunks: break self.get_logger().info(f'[CMD] done — total:{len(buf)} chunks, voice:{started}') return np.concatenate(buf) if buf else np.array([], dtype=np.float32) # ─── Main listen loop ─────────────────────────────────────── def _listen_loop(self):

WAKE_WORDS = [ 'vector', 'vektor', 'hector', 'victor', 'vectra', 'specter', 'vactor', 'wector', 'picture', 'with that', ] while not self._stop_event.is_set() and rclpy.ok(): # Audio mode OFF (webpage toggle / gesture) — drain mic input # and skip wake-word detection entirely until re-enabled. if not self.audio_enabled: try: self.audio_q.get(timeout=0.1) except queue.Empty: pass continue # Drain queue while TTS is speaking if self.is_tts_speaking: try: self.audio_q.get(timeout=0.1) except queue.Empty: pass continue # ── Step 1: wait for any RMS spike ────────────────── try: chunk = self.audio_q.get(timeout=0.1) except queue.Empty: continue if self.is_tts_speaking: continue rms = np.sqrt(np.mean(chunk ** 2)) * 32767 if rms < self.rms_wake: continue # ── Step 2: collect ~1.5s around the spike ────────── pre = [chunk] for _ in range(14): if self.is_tts_speaking: break try: pre.append(self.audio_q.get(timeout=0.15)) except queue.Empty: break if self.is_tts_speaking: continue # ── Step 3: check for wake word ────────────────────── audio = np.concatenate(pre)

text = self._transcribe(audio, prompt='Vector') self.get_logger().info(f'[Wake] heard: "{text}"') if not text.strip(): continue words = text.split() if words and words.count(max(set(words), key=words.count)) > 4: continue if not any(w in text for w in WAKE_WORDS): continue # ── Step 4: wake confirmed — collect command ───────── self.get_logger().info('✅ Vector! Listening for command...') self._publish('WAKE_WORD_DETECTED') # Let the "Yes?" acknowledgment fully play out first — starting # command collection immediately used to race the TTS mute # check and get killed within milliseconds of "Yes?" starting. self._wait_for_ack_tts() # Buffer is already flushed by _tts_status_callback's IDLE # handler, so prefill is stale/irrelevant here — start fresh. cmd_audio = self._collect_until_silence(max_sec=self.command_max_sec) if self.is_tts_speaking: continue if cmd_audio.size == 0: self.get_logger().info('[CMD] No audio captured — resuming') self._publish('RESUME') continue cmd_rms = np.sqrt(np.mean(cmd_audio ** 2)) * 32767 if cmd_rms < self.rms_command: self.get_logger().info(f'[CMD] Too quiet (rms={cmd_rms:.0f}) — resuming') self._publish('RESUME') continue # ── Step 5: transcribe command ─────────────────────── cmd_text = self._transcribe( cmd_audio, prompt='follow me, stop, forward, backward, left, right, search, describe the scene' ) if not cmd_text.strip(): self.get_logger().info('[CMD] Empty transcription — resuming') self._publish('RESUME') continue command = self._parse(cmd_text) self.get_logger().info(f'[CMD] "{cmd_text}" → {command}')

if command == 'NONE': self.get_logger().info(f'[CMD] Sending to LLM: "{cmd_text}"') self._publish(cmd_text) else: self._publish(command) # ── Step 6: clean up before next wake cycle ────────── self._flush_queue() time.sleep(0.5) self._flush_queue() self.get_logger().info('🔄 Listening for "Vector"...') # ─── Helpers ──────────────────────────────────────────────── def _publish(self, data: str): msg = String() msg.data = data self.pub.publish(msg) def _parse(self, text: str) -> str: t = text.lower() if any(x in t for x in ['follow me', 'follow person', 'come with me', 'follow']): return 'FOLLOW_PERSON' elif any(x in t for x in ['stop', 'halt', 'freeze']): return 'STOP' elif any(x in t for x in ['search', 'find person', 'look for']): return 'SEARCH' elif any(x in t for x in ['turn left', 'left']): return 'LEFT' elif any(x in t for x in ['turn right', 'right']): return 'RIGHT' elif any(x in t for x in ['move forward', 'go ahead', 'forward']): return 'FORWARD' elif any(x in t for x in ['move backward', 'go back', 'backward']): return 'BACKWARD' elif any(x in t for x in [ 'describe the scene', 'describe scene', 'describe the seen', 'describe seen', 'what do you see', 'what can you see', 'whats around', "what's around", 'look around', 'describe what you see', ]) or 'describe' in t: return 'DESCRIBE_SCENE' return 'NONE' def main(args=None): rclpy.init(args=args) node = STTNode() try: rclpy.spin(node) except KeyboardInterrupt: if rclpy.ok(): node.get_logger().info('STT node stopping...')

finally: node._stop_event.set() node._listen_thread.join(timeout=3.0) node.stream.stop() node.stream.close() node.destroy_node() if rclpy.ok(): rclpy.shutdown() if __name__ == '__main__': main()

# Appendix F - llm_node.py

Interaction. Local intent router plus NVIDIA NIM text/vision calls.

import rclpy from rclpy.node import Node from std_msgs.msg import String, Float32 from sensor_msgs.msg import Image from rclpy.qos import qos_profile_sensor_data from cv_bridge import CvBridge import cv2 import base64 import os import threading import time from dotenv import load_dotenv from openai import OpenAI load_dotenv(dotenv_path="/workspace/.env", override=True) class LLMNode(Node): def __init__(self): super().__init__('llm_node') self._req_count = 0 self.last_request = 0 self.declare_parameter('min_interval', 4.0) self.min_interval = self.get_parameter('min_interval').value self.bridge = CvBridge() self.latest_frame = None self.frame_lock = threading.Lock() self.last_vision_time = 0 self.declare_parameter('vision_cooldown', 3.0) self.vision_cooldown = self.get_parameter('vision_cooldown').value self.latest_battery_pct = None api_key = os.environ.get('NVIDIA_API_KEY', '').strip() if not api_key: self.get_logger().error('❌ NVIDIA_API_KEY not found in /workspace/.env!') if not api_key.startswith('nvapi-'): self.get_logger().error(f'❌ Key looks wrong: "{api_key[:10]}..."') self.client = OpenAI( base_url="https://integrate.api.nvidia.com/v1", api_key=api_key, timeout=10.0 ) self.declare_parameter('chat_model', 'meta/llama-3.1-8b-instruct') self.chat_model = self.get_parameter('chat_model').value self.declare_parameter('vision_model', 'meta/llama-3.2-11b-vision-instruct') self.vision_model = self.get_parameter('vision_model').value self.declare_parameter('vision_max_dimension', 640)

self.vision_max_dimension = self.get_parameter('vision_max_dimension').value self.declare_parameter('vision_jpeg_quality', 80) self.vision_jpeg_quality = self.get_parameter('vision_jpeg_quality').value self.declare_parameter('llm_response_topic', '/llm_response') self.declare_parameter('robot_ui_mode_topic', '/robot_ui_mode') self.declare_parameter('voice_command_topic', '/voice_command') self.declare_parameter('camera_topic', '/camera/image_raw') self.declare_parameter('battery_topic', '/battery_status') self.llm_response_topic = self.get_parameter('llm_response_topic').value self.robot_ui_mode_topic = self.get_parameter('robot_ui_mode_topic').value self.voice_command_topic = self.get_parameter('voice_command_topic').value self.camera_topic = self.get_parameter('camera_topic').value self.battery_topic = self.get_parameter('battery_topic').value self.pub = self.create_publisher(String, self.llm_response_topic, 10) self.ui_pub = self.create_publisher(String, self.robot_ui_mode_topic, 10) self._last_cmd = None self._last_cmd_time = 0.0 self.declare_parameter('dedupe_window', 3.0) self._dedupe_window = self.get_parameter('dedupe_window').value self.create_subscription(String, self.voice_command_topic, self.command_callback, 10) self.create_subscription( Image, self.camera_topic, self.camera_callback, qos_profile_sensor_data ) self.create_subscription(Float32, self.battery_topic, self._battery_callback, 10) self.get_logger().info(f'★ LLM ready — chat: {self.chat_model}') def command_callback(self, msg: String): t_received = time.time() cmd = msg.data.strip() cmd_lower = cmd.lower() cmd_upper = cmd.upper() now = time.time() if cmd_upper == self._last_cmd and (now - self._last_cmd_time) < self._dedupe_window: self.get_logger().info(f'[LLM] Duplicate command ignored: "{cmd}"') return self._last_cmd = cmd_upper self._last_cmd_time = now if 'face' in cmd_lower: self._publish_ui('face') self._publish_response("Okay, showing my face.") return if 'vision' in cmd_lower or 'feed' in cmd_lower or 'camera' in cmd_lower: self._publish_ui('vision') self._publish_response("Okay, showing the camera feed.")

return if 'battery' in cmd_lower: self._report_battery() return if cmd_upper == 'DESCRIBE_SCENE' or 'describe' in cmd_lower: self._trigger_vision(t_received) return if 'follow' in cmd_lower: self._publish_response("Looking for you now. I will follow you.") return if ('explore' in cmd_lower or 'roam' in cmd_lower) and 'stop' not in cmd_lower: self._publish_response("Back to roaming and exploring on my own.") return control_cmds = { 'WAKE_WORD_DETECTED', 'RESUME', 'STOP', 'FORWARD', 'BACKWARD', 'LEFT', 'RIGHT', 'NONE', 'FOLLOW_PERSON', 'SEARCH', 'DESCRIBE_SCENE' } if cmd_upper in control_cmds: return now = time.time() if now - self.last_request < self.min_interval: self.get_logger().warn('[LLM] Too soon — request dropped.') return self.last_request = now threading.Thread(target=self._query_chat, args=(cmd, t_received), daemon=True).start() def camera_callback(self, msg: Image): with self.frame_lock: first_frame = self.latest_frame is None self.latest_frame = msg if first_frame: self.get_logger().info('[LLM] ✅ First camera frame received.') def _trigger_vision(self, t_received=None): now = time.time() if now - self.last_vision_time < self.vision_cooldown: self.get_logger().warn('[LLM] Vision request too soon — dropped.') return with self.frame_lock: frame = self.latest_frame if frame is None: self._publish_response("I don't have a camera frame yet, try again in a moment.") return

self.last_vision_time = now threading.Thread(target=self._query_vision, args=(frame, t_received), daemon=True).start() def _battery_callback(self, msg: Float32): self.latest_battery_pct = msg.data def _report_battery(self): if self.latest_battery_pct is None: self._publish_response("I don't have a battery reading yet.") return pct = self.latest_battery_pct self._publish_response(f"My battery is at {pct:.0f} percent.") def _publish_ui(self, mode: str): out = String() out.data = mode self.ui_pub.publish(out) self.get_logger().info(f'[UI] /robot_ui_mode → {mode}') def _query_chat(self, cmd: str, t_received=None): self._req_count += 1 t0 = time.time() self.get_logger().info(f'[LLM] Query #{self._req_count}: "{cmd}"') try: response = self.client.chat.completions.create( model=self.chat_model, messages=[ { "role": "system", "content": ( "You are Vector, a helpful assistant robot. " "Give a friendly, clear, plain text answer in 10 to 15 words. " "Do not use actions, emojis, asterisks, or formatting." ) }, {"role": "user", "content": cmd} ], max_tokens=60, temperature=0.7 ) t1 = time.time() self.get_logger().info(f'[TIMING][chat] API call: {t1 - t0:.2f}s') text = response.choices[0].message.content if not text: self.get_logger().warn(f'[LLM] Empty response — finish_reason: {response.choices[0].finish_reason}') self._publish_response("Sorry, I did not get a response.") return text = text.strip().replace('*', '').replace('_', '').replace('#', '') self._publish_response(text)

t2 = time.time() self.get_logger().info(f'[TIMING][chat] TOTAL query_start → published: {t2 - t0:.2f}s') if t_received is not None: self.get_logger().info( f'[TIMING][chat] TOTAL command_received → published: {t2 - t_received:.2f}s' ) except Exception as e: self.get_logger().error(f'[LLM] Chat failed: {type(e).__name__}: {e}') def _query_vision(self, img_msg: Image, t_received=None): try: t0 = time.time() cv_frame = self.bridge.imgmsg_to_cv2(img_msg, desired_encoding="bgr8") t1 = time.time() h, w = cv_frame.shape[:2] longer_side = max(h, w) if longer_side > self.vision_max_dimension: scale = self.vision_max_dimension / longer_side cv_frame = cv2.resize( cv_frame, (int(w * scale), int(h * scale)), interpolation=cv2.INTER_AREA ) t2 = time.time() _, buffer = cv2.imencode( ".jpg", cv_frame, [int(cv2.IMWRITE_JPEG_QUALITY), self.vision_jpeg_quality] ) base64_image = base64.b64encode(buffer).decode("utf-8") t3 = time.time() self.get_logger().info( f'[TIMING][vision] cv_convert={t1-t0:.2f}s resize={t2-t1:.2f}s ' f'encode={t3-t2:.2f}s frame={cv_frame.shape[1]}x{cv_frame.shape[0]} ' f'payload_kb={len(base64_image)/1024:.0f}' ) response = self.client.chat.completions.create( model=self.vision_model, messages=[{ "role": "user", "content": [ { "type": "text", "text": ( "You are Vector, an autonomous mobile robot. " "In one or two sentences under 25 words, describe what you see. " "Note any people, paths, or obstacles. Plain text only." ) }, {

"type": "image_url", "image_url": {"url": f"data:image/jpeg;base64,{base64_image}"} } ] }], max_tokens=50, temperature=0.4 ) t4 = time.time() self.get_logger().info(f'[TIMING][vision] API call: {t4-t3:.2f}s ¦ TOTAL: {t4-t0:.2f}s') text = response.choices[0].message.content if not text: self._publish_response("I couldn't make out a clear description.") return text = text.strip().replace('*', '').replace('_', '').replace('#', '') self._publish_response(text) if t_received is not None: self.get_logger().info( f'[TIMING][vision] TOTAL command_received → published: {time.time() - t_received:.2f}s' ) except Exception as e: self.get_logger().error(f'[LLM] Vision failed: {type(e).__name__}: {e}') self._publish_response("Sorry, my vision check failed.") def _publish_response(self, text: str): self.get_logger().info(f'[LLM] → "{text}"') out = String() out.data = text.strip() self.pub.publish(out) def main(args=None): rclpy.init(args=args) node = LLMNode() try: rclpy.spin(node) except KeyboardInterrupt: if rclpy.ok(): node.get_logger().info('LLM node stopping...') finally: node.destroy_node() if rclpy.ok(): rclpy.shutdown() if __name__ == '__main__': main()

# Appendix G - tts_node.py

Interaction. Offline Piper TTS, persistent audio playback, cache and lip-sync.

import os os.environ.setdefault("XDG_RUNTIME_DIR", "/run/user/1000") import asyncio import threading import time from pathlib import Path import numpy as np import rclpy import sounddevice as sd from piper import PiperVoice, SynthesisConfig from rclpy.node import Node from std_msgs.msg import Bool, Float32, String class TTSNode(Node): """Provide offline Piper TTS, persistent playback, and mouth lip-sync.""" SUPPORTED_VOICES = { "en_US-hfc_male-medium", "en_US-hfc_female-medium", } VOICE_PROFILES = { "en_US-hfc_male-medium": { "length_scale": 0.94, "noise_scale": 0.55, "noise_w_scale": 0.70, }, "en_US-hfc_female-medium": { "length_scale": 1.05, "noise_scale": 0.62, "noise_w_scale": 0.75, }, } COMMAND_CONFIRMATIONS = { "FOLLOW_PERSON": "Following you.", "STOP": "Stopping.", "SEARCH": "Searching.", "FORWARD": "Moving forward.", "BACKWARD": "Moving backward.", "LEFT": "Turning left.", "RIGHT": "Turning right.", "RESUME": "Resuming.", "WAKE_WORD_DETECTED": "Yes?", } FIXED_PHRASES = { "WAKE_WORD_DETECTED": "Yes?",

"AUDIO_ON": "Voice commands on.", "AUDIO_OFF": "Voice commands off.", "FOLLOW_PERSON": "Following you.", "STOP": "Stopping.", "SEARCH": "Searching.", "FORWARD": "Moving forward.", "BACKWARD": "Moving backward.", "LEFT": "Turning left.", "RIGHT": "Turning right.", "RESUME": "Resuming.", "READY": "Vector is ready.", } def __init__(self): super().__init__("tts_node") # Piper voice and synthesis self.declare_parameter( "piper_voice", "en_US-hfc_female-medium", ) self.declare_parameter( "piper_data_dir", "/workspace/models/piper", ) self.declare_parameter("use_voice_profile", True) self.declare_parameter("length_scale", 0.94) self.declare_parameter("noise_scale", 0.55) self.declare_parameter("noise_w_scale", 0.70) self.declare_parameter("volume", 1.0) self.declare_parameter("normalize_audio", True) # ROS topics self.declare_parameter("tts_status_topic", "/tts_status") self.declare_parameter( "robot_emotion_topic", "/robot_emotion", ) self.declare_parameter("mouth_level_topic", "/mouth_level") self.declare_parameter( "llm_response_topic", "/llm_response", ) self.declare_parameter( "voice_command_topic", "/voice_command", ) self.declare_parameter( "audio_enabled_topic", "/audio_enabled", )

# Playback, caching, and timing self.declare_parameter("chunk_ms", 50) self.declare_parameter("bt_latency_s", 0.1) self.declare_parameter("post_playback_delay_s", 0.4) self.declare_parameter( "post_playback_delay_cached_s", 0.1, ) self.declare_parameter("voice_command_dedupe_s", 2.0) self.declare_parameter("startup_speech_delay_s", 5.0) self.declare_parameter( "startup_announcement_enabled", True, ) self.declare_parameter("cache_fixed_phrases", True) # Audio output self.declare_parameter("audio_samplerate", 44100) self.declare_parameter("bt_keepalive_enabled", True) self.declare_parameter("bt_keepalive_volume", 0.003) self.piper_voice_name = str( self.get_parameter("piper_voice").value ) self.piper_data_dir = Path( self.get_parameter("piper_data_dir").value ) self.use_voice_profile = bool( self.get_parameter("use_voice_profile").value ) self.length_scale = float( self.get_parameter("length_scale").value ) self.noise_scale = float( self.get_parameter("noise_scale").value ) self.noise_w_scale = float( self.get_parameter("noise_w_scale").value ) self.volume = float(self.get_parameter("volume").value) self.normalize_audio = bool( self.get_parameter("normalize_audio").value ) self._validate_voice_selection() if self.use_voice_profile: profile = self.VOICE_PROFILES[self.piper_voice_name] self.length_scale = profile["length_scale"] self.noise_scale = profile["noise_scale"] self.noise_w_scale = profile["noise_w_scale"]

self.tts_status_topic = str( self.get_parameter("tts_status_topic").value ) self.robot_emotion_topic = str( self.get_parameter("robot_emotion_topic").value ) self.mouth_level_topic = str( self.get_parameter("mouth_level_topic").value ) self.llm_response_topic = str( self.get_parameter("llm_response_topic").value ) self.voice_command_topic = str( self.get_parameter("voice_command_topic").value ) self.audio_enabled_topic = str( self.get_parameter("audio_enabled_topic").value ) self.chunk_ms = int(self.get_parameter("chunk_ms").value) self.bt_latency_s = float( self.get_parameter("bt_latency_s").value ) self.post_playback_delay_s = float( self.get_parameter("post_playback_delay_s").value ) self.post_playback_delay_cached_s = float( self.get_parameter( "post_playback_delay_cached_s" ).value ) self.voice_command_dedupe_s = float( self.get_parameter("voice_command_dedupe_s").value ) self.startup_speech_delay_s = float( self.get_parameter("startup_speech_delay_s").value ) self.startup_announcement_enabled = bool( self.get_parameter( "startup_announcement_enabled" ).value ) self.cache_fixed_phrases = bool( self.get_parameter("cache_fixed_phrases").value ) configured_sample_rate = int( self.get_parameter("audio_samplerate").value ) self.bt_keepalive_enabled = bool( self.get_parameter("bt_keepalive_enabled").value )

self.bt_keepalive_volume = float( self.get_parameter("bt_keepalive_volume").value ) self.piper_voice = self._load_piper_voice() self.piper_sample_rate = int( self.piper_voice.config.sample_rate ) self.audio_samplerate = ( configured_sample_rate or self.piper_sample_rate ) self.synthesis_config = SynthesisConfig( length_scale=self.length_scale, noise_scale=self.noise_scale, noise_w_scale=self.noise_w_scale, volume=self.volume, normalize_audio=self.normalize_audio, ) self.status_pub = self.create_publisher( String, self.tts_status_topic, 10, ) self.emotion_pub = self.create_publisher( String, self.robot_emotion_topic, 10, ) self.mouth_pub = self.create_publisher( Float32, self.mouth_level_topic, 10, ) self.loop = asyncio.new_event_loop() self.speech_queue = None self.cacheable_texts = set(self.FIXED_PHRASES.values()) self.cache_decoded = {} self.last_voice_command = None self.last_voice_command_time = 0.0 self.last_audio_enabled = None self.audio_lock = threading.Lock() self.synthesis_lock = threading.Lock() self.playback_buffer = None self.playback_position = 0 self.tone_phase = 0 self.audio_stream = None

self.audio_thread = threading.Thread( target=self._start_audio_engine, daemon=True, ) self.audio_thread.start() self.async_thread = threading.Thread( target=self._start_async_loop, daemon=True, ) self.async_thread.start() self.response_sub = self.create_subscription( String, self.llm_response_topic, self.response_callback, 10, ) self.command_sub = self.create_subscription( String, self.voice_command_topic, self.command_callback, 10, ) self.audio_enabled_sub = self.create_subscription( Bool, self.audio_enabled_topic, self._audio_enabled_callback, 10, ) self._publish_status("IDLE") self._publish_emotion("neutral") self._publish_mouth(0.0) self.get_logger().info( f"Piper TTS ready: voice={self.piper_voice_name}, " f"model_rate={self.piper_sample_rate} Hz, " f"output_rate={self.audio_samplerate} Hz, " f"lazy_cache={self.cache_fixed_phrases}" ) def _validate_voice_selection(self): """Verify that the configured voice is supported.""" if self.piper_voice_name in self.SUPPORTED_VOICES: return supported = ", ".join(sorted(self.SUPPORTED_VOICES)) raise ValueError( f'Unsupported piper_voice "{self.piper_voice_name}". ' f"Supported voices: {supported}"

) def _load_piper_voice(self): """Load the selected Piper ONNX model from disk.""" model_path = ( self.piper_data_dir / f"{self.piper_voice_name}.onnx" ) config_path = Path(f"{model_path}.json") if not model_path.is_file(): raise FileNotFoundError( f"Piper model not found: {model_path}. " "Download it with: python3 -m " "piper.download_voices --data-dir " f"{self.piper_data_dir} {self.piper_voice_name}" ) if not config_path.is_file(): raise FileNotFoundError( f"Piper config not found: {config_path}" ) self.get_logger().info( f"Loading Piper model: {model_path}" ) return PiperVoice.load(str(model_path), use_cuda=False) def _keepalive_chunk(self, frames: int) -> np.ndarray: """Generate a quiet tone to keep Bluetooth audio awake.""" volume = ( self.bt_keepalive_volume if self.bt_keepalive_enabled else 0.0 ) frequency = 40.0 sample_positions = ( self.tone_phase + np.arange(frames) ) / self.audio_samplerate self.tone_phase += frames return ( volume * np.sin(2 * np.pi * frequency * sample_positions) ).astype(np.float32) def _audio_callback( self, output_data, frames, _time_info, _status, ):

"""Feed speech or keep-alive samples to the output stream.""" with self.audio_lock: buffer = self.playback_buffer position = self.playback_position if buffer is not None and position < len(buffer): end = min(position + frames, len(buffer)) sample_count = end - position output_data[:sample_count, 0] = buffer[position:end] if sample_count < frames: output_data[sample_count:, 0] = ( self._keepalive_chunk(frames - sample_count) ) with self.audio_lock: self.playback_position = end else: output_data[:, 0] = self._keepalive_chunk(frames) def _start_audio_engine(self): """Open and start the persistent sounddevice output stream.""" try: self.audio_stream = sd.OutputStream( samplerate=self.audio_samplerate, channels=1, dtype="float32", callback=self._audio_callback, ) self.audio_stream.start() self.get_logger().info( "[TTS] Persistent audio engine started " f"(keepalive={self.bt_keepalive_enabled}, " f"volume={self.bt_keepalive_volume})" ) except Exception as error: self.get_logger().error( f"[TTS] Failed to start audio engine: {error}" ) def _stop_playback(self): """Clear any speech currently assigned to the audio callback.""" with self.audio_lock: self.playback_buffer = None self.playback_position = 0 def _synthesize_text(self, text): """Generate mono float32 samples locally with Piper.""" chunks = [] source_sample_rate = None # Piper inference is blocking and must not be used concurrently by

# live speech and lazy-cache generation. with self.synthesis_lock: for chunk in self.piper_voice.synthesize( text, syn_config=self.synthesis_config, ): source_sample_rate = int(chunk.sample_rate) pcm = np.frombuffer( chunk.audio_int16_bytes, dtype=np.int16, ) channels = int(chunk.sample_channels) if channels > 1: pcm = pcm.reshape(-1, channels).mean(axis=1) chunks.append( pcm.astype(np.float32) / 32768.0 ) if not chunks or source_sample_rate is None: raise RuntimeError( "Piper returned no audio samples" ) samples = np.concatenate(chunks) if source_sample_rate != self.audio_samplerate: samples = self._resample_audio( samples, source_sample_rate, self.audio_samplerate, ) levels = self._calculate_lipsync_levels(samples) return samples, levels @staticmethod def _resample_audio( samples, source_rate, target_rate, ): """Linearly resample mono audio without another dependency.""" if source_rate == target_rate or len(samples) < 2: return samples.astype(np.float32, copy=False) target_length = max( 1, int( round( len(samples)

* target_rate / source_rate ) ), ) source_positions = np.arange( len(samples), dtype=np.float64, ) target_positions = np.linspace( 0, len(samples) - 1, target_length, dtype=np.float64, ) return np.interp( target_positions, source_positions, samples, ).astype(np.float32) def _calculate_lipsync_levels(self, samples): """Calculate normalized RMS levels for mouth animation.""" chunk_samples = max( 1, int( self.audio_samplerate * self.chunk_ms / 1000 ), ) raw_levels = [] for index in range(0, len(samples), chunk_samples): window = samples[index:index + chunk_samples] rms = ( float(np.sqrt(np.mean(window * window))) if len(window) else 0.0 ) raw_levels.append(rms) max_rms = max(raw_levels) if raw_levels else 1.0 if max_rms <= 0.0: max_rms = 1.0 return [ min(1.0, rms / max_rms) for rms in raw_levels ]

def response_callback(self, msg: String): """Queue a non-empty LLM response for speech.""" text = msg.data.strip() if text and self.speech_queue: self.loop.call_soon_threadsafe( self.speech_queue.put_nowait, text, ) def command_callback(self, msg: String): """Queue a fixed confirmation for recognized control commands.""" command = msg.data.strip().upper() now = time.time() if ( command == self.last_voice_command and now - self.last_voice_command_time < self.voice_command_dedupe_s ): self.get_logger().info( f'[TTS] Duplicate command ignored: "{command}"' ) return self.last_voice_command = command self.last_voice_command_time = now text = self.COMMAND_CONFIRMATIONS.get(command) if text and self.speech_queue: self.loop.call_soon_threadsafe( self.speech_queue.put_nowait, text, ) def _audio_enabled_callback(self, msg: Bool): """Announce audio enable-state changes once.""" enabled = msg.data if enabled == self.last_audio_enabled: return self.last_audio_enabled = enabled text = ( "Voice commands on." if enabled else "Voice commands off." ) if self.speech_queue: self.loop.call_soon_threadsafe( self.speech_queue.put_nowait,

text, ) def _publish_status(self, text): """Publish SPEAKING or IDLE.""" msg = String() msg.data = text self.status_pub.publish(msg) def _publish_emotion(self, text): """Publish the current robot emotion.""" msg = String() msg.data = text self.emotion_pub.publish(msg) def _publish_mouth(self, level): """Publish a clamped mouth-animation level.""" msg = Float32() msg.data = max(0.0, min(1.0, float(level))) self.mouth_pub.publish(msg) def _start_async_loop(self): """Create the speech queue and run the async speech worker.""" asyncio.set_event_loop(self.loop) self.speech_queue = asyncio.Queue() self.loop.run_until_complete(self._speech_loop()) async def _lipsync_stream( self, levels, start_delay=0.0, ): """Publish mouth levels in time with audio playback.""" try: if start_delay > 0: await asyncio.sleep(start_delay) for level in levels: self._publish_mouth(level) await asyncio.sleep(self.chunk_ms / 1000.0) except asyncio.CancelledError: pass finally: self._publish_mouth(0.0) async def _queue_startup_message(self): """Queue the optional startup announcement after a delay.""" await asyncio.sleep(self.startup_speech_delay_s) if rclpy.ok() and self.speech_queue is not None: await self.speech_queue.put( "Vector is ready."

) async def _speech_loop(self): """Synthesize and play queued speech sequentially.""" if self.startup_announcement_enabled: asyncio.create_task( self._queue_startup_message() ) while rclpy.ok(): text = await self.speech_queue.get() if not text: self.speech_queue.task_done() continue lipsync_task = None is_cached = text in self.cache_decoded try: start_time = time.time() self._publish_status("SPEAKING") self.get_logger().info(f"TTS: {text}") if is_cached: samples, levels = self.cache_decoded[text] else: running_loop = asyncio.get_running_loop() samples, levels = ( await running_loop.run_in_executor( None, self._synthesize_text, text, ) ) if ( self.cache_fixed_phrases and text in self.cacheable_texts ): self.cache_decoded[text] = ( samples, levels, ) self.get_logger().info( "[TTS] Cached after first use: " f'"{text}"' ) synthesis_done = time.time() self.get_logger().info( "[TIMING] Piper synthesis "

f"(cached={is_cached}): " f"{synthesis_done - start_time:.2f}s" ) self._publish_emotion("talking") with self.audio_lock: self.playback_buffer = samples self.playback_position = 0 if levels: lipsync_task = asyncio.create_task( self._lipsync_stream( levels, start_delay=self.bt_latency_s, ) ) duration = len(samples) / self.audio_samplerate await asyncio.sleep(duration) playback_done = time.time() self.get_logger().info( "[TIMING] Playback: " f"{playback_done - synthesis_done:.2f}s" ) self._stop_playback() delay = ( self.post_playback_delay_cached_s if is_cached else self.post_playback_delay_s ) await asyncio.sleep(delay) self.get_logger().info( "[TIMING] Total SPEAKING to IDLE: " f"{time.time() - start_time:.2f}s" ) except Exception as error: self.get_logger().error( f"[TTS] {error}" ) finally: if lipsync_task: lipsync_task.cancel() self._publish_mouth(0.0) self._stop_playback() self._publish_status("IDLE") self._publish_emotion("neutral") self.speech_queue.task_done()

async def stop_speaking(self): """Stop the current playback buffer.""" self._stop_playback() def destroy_node(self): """Stop playback, close audio, and stop the async loop.""" self._stop_playback() if self.audio_stream is not None: try: self.audio_stream.stop() self.audio_stream.close() except Exception as error: self.get_logger().warning( "[TTS] Audio stream shutdown warning: " f"{error}" ) if self.loop.is_running(): self.loop.call_soon_threadsafe(self.loop.stop) elif not self.loop.is_closed(): self.loop.close() return super().destroy_node() def main(args=None): rclpy.init(args=args) node = None try: node = TTSNode() rclpy.spin(node) except KeyboardInterrupt: if node is not None and rclpy.ok(): node.get_logger().info( "TTS node stopping..." ) except Exception as error: if node is not None: node.get_logger().fatal( f"TTS node failed: {error}" ) else: print(f"TTS node failed: {error}") raise finally: if node is not None: node.destroy_node() if rclpy.ok():

rclpy.shutdown() if __name__ == "__main__": main()

# Appendix H - decision_node.py

Control. Latest supplied decision snapshot with 1 m timed linear maneuver.

import math import time import rclpy from geometry_msgs.msg import Twist from rclpy.node import Node from rclpy.qos import DurabilityPolicy, HistoryPolicy, QoSProfile, ReliabilityPolicy from std_msgs.msg import Bool, Float32, String from vision_msgs.msg import Detection2DArray class DecisionNode(Node): """Coordinate autonomous roaming, person following, voice control, and safety stops.""" def __init__(self): super().__init__('decision_node') self.last_target_time = time.time() self.target_timeout = 2.0 self.obstacle_start_time = None self.frame_width = 640 self.center_x = self.frame_width // 2 self.turn_threshold = 60 self.stop_width = 450 self.target_id = None self.obstacle_detected = False self.front_distance = 999.0 self.free_direction = 'LEFT' self.obstacle_log_active = False self.last_motion_log_time = 0.0 self.motion_log_interval = 1.0 # AUTO roams or follows, STOP blocks motion, and MANEUVER runs a timed command. self.voice_mode = 'AUTO' self.follow_enabled = False self.pre_wake_voice_mode = self.voice_mode self.pre_wake_follow_enabled = self.follow_enabled self.maneuver_twist = (0.0, 0.0) self.maneuver_duration = 0.0 self.maneuver_start_time = 0.0 self.maneuver_return_mode = 'AUTO' # The measured turn rate compensates for real-world motor response. self.turn_speed_rad_s = 0.6 self.measured_turn_rate_rad_s = 1.2 self.turn_angle_rad = math.radians(90.0) self.linear_speed_m_s = 0.25 self.linear_distance_m = 1.0 qos = QoSProfile( history=HistoryPolicy.KEEP_LAST,

depth=10, reliability=ReliabilityPolicy.RELIABLE, durability=DurabilityPolicy.VOLATILE, ) self.detection_sub = self.create_subscription( Detection2DArray, '/tracked_detections', self.detection_callback, qos, ) self.obstacle_sub = self.create_subscription( Bool, '/obstacle_detected', self.obstacle_callback, qos, ) self.distance_sub = self.create_subscription( Float32, '/front_distance', self.distance_callback, qos, ) self.direction_sub = self.create_subscription( String, '/free_direction', self.direction_callback, qos, ) self.voice_sub = self.create_subscription( String, '/voice_command', self.voice_callback, qos, ) self.audio_sub = self.create_subscription( Bool, '/audio_enabled', self.audio_enabled_callback, qos, ) # The watchdog emergency stop overrides every motion mode. self.estop_active = False self.estop_sub = self.create_subscription( Bool, '/emergency_stop', self.emergency_stop_callback, qos, ) self.cmd_vel_pub = self.create_publisher(Twist, '/cmd_vel', qos)

# STOP and MANEUVER remain active independently of camera callback frequency. self.control_timer = self.create_timer(0.05, self._control_loop) self.get_logger().info('Decision node started — default mode: AUTO/ROAM') def obstacle_callback(self, msg: Bool): self.obstacle_detected = msg.data def distance_callback(self, msg: Float32): self.front_distance = msg.data def direction_callback(self, msg: String): self.free_direction = msg.data def emergency_stop_callback(self, msg: Bool): self.estop_active = msg.data if self.estop_active: self._stop() self.get_logger().error( '🛑 [WATCHDOG] Emergency stop ACTIVE — overriding all motion' ) else: self.get_logger().info( '✅ [WATCHDOG] Emergency stop CLEARED — resuming normal operation' ) def audio_enabled_callback(self, msg: Bool): """Switch to following while audio is muted and restore the prior mode later.""" if not msg.data: self.pre_audio_off_follow_enabled = self.follow_enabled self.pre_audio_off_voice_mode = ( self.voice_mode if self.voice_mode != 'MANEUVER' else 'AUTO' ) self.voice_mode = 'AUTO' self.follow_enabled = True self.target_id = None self.last_target_time = time.time() self.get_logger().info('🔇 Audio OFF — auto-switching to FOLLOW mode') else: self.voice_mode = getattr(self, 'pre_audio_off_voice_mode', 'AUTO') self.follow_enabled = getattr( self, 'pre_audio_off_follow_enabled', False, ) if self.follow_enabled: self.target_id = None self.last_target_time = time.time()

self.get_logger().info( '🔊 Audio ON — voice control restored, resuming previous mode ' f'(follow_enabled={self.follow_enabled})' ) def voice_callback(self, msg: String): """Map normalized STT command tokens to motion modes and actions.""" command = msg.data.strip().upper() if command == 'WAKE_WORD_DETECTED': self.pre_wake_voice_mode = ( self.voice_mode if self.voice_mode != 'MANEUVER' else self.maneuver_return_mode ) self.pre_wake_follow_enabled = self.follow_enabled self.voice_mode = 'STOP' self._stop() self.get_logger().info( '🎤 [VOICE] Wake word — motors stopped, awaiting command' ) return if command == 'RESUME': self._restore_pre_wake_mode() self.get_logger().info( f'⏱ [VOICE] No command — resuming {self.voice_mode}' ) return if command == 'STOP': self.voice_mode = 'STOP' self.follow_enabled = False self._stop() self.get_logger().info('🛑 [VOICE] STOP — halting indefinitely') return if command in ('FOLLOW_PERSON', 'SEARCH'): self.voice_mode = 'AUTO' self.follow_enabled = True self.target_id = None self.last_target_time = time.time() self.get_logger().info( '🤖 [VOICE] Follow mode ENABLED — searching for a person' ) return if command in ('LEFT', 'RIGHT', 'FORWARD', 'BACKWARD'): self._start_maneuver(command) return if command in ('DESCRIBE_SCENE', 'BATTERY_STATUS'): self._restore_pre_wake_mode()

self.get_logger().info( f'[VOICE] {command} — resuming {self.voice_mode}' ) return self._restore_pre_wake_mode() self.get_logger().info( f'[VOICE] Conversation — resuming {self.voice_mode}' ) def _restore_pre_wake_mode(self): self.voice_mode = self.pre_wake_voice_mode self.follow_enabled = self.pre_wake_follow_enabled def _start_maneuver(self, command: str): """Start a timed turn or linear movement and remember the return mode.""" self.maneuver_return_mode = ( self.voice_mode if self.voice_mode != 'MANEUVER' else 'AUTO' ) if command == 'LEFT': self.maneuver_twist = (0.0, self.turn_speed_rad_s) self.maneuver_duration = ( self.turn_angle_rad / self.measured_turn_rate_rad_s ) elif command == 'RIGHT': self.maneuver_twist = (0.0, -self.turn_speed_rad_s) self.maneuver_duration = ( self.turn_angle_rad / self.measured_turn_rate_rad_s ) elif command == 'FORWARD': self.maneuver_twist = (self.linear_speed_m_s, 0.0) self.maneuver_duration = ( self.linear_distance_m / self.linear_speed_m_s ) elif command == 'BACKWARD': self.maneuver_twist = (-self.linear_speed_m_s, 0.0) self.maneuver_duration = ( self.linear_distance_m / self.linear_speed_m_s ) self.maneuver_start_time = time.time() self.voice_mode = 'MANEUVER' self.get_logger().info( f'🎯 [VOICE] Maneuver {command} — {self.maneuver_duration:.2f}s, ' f'will return to {self.maneuver_return_mode}' ) def _control_loop(self): """Maintain emergency-stop, timed-maneuver, and stopped states at 20 Hz.""" if self.estop_active: self._stop()

return if self.voice_mode == 'MANEUVER': elapsed = time.time() - self.maneuver_start_time if elapsed >= self.maneuver_duration: self._stop() self.voice_mode = self.maneuver_return_mode self.get_logger().info( f'[VOICE] Maneuver complete → {self.voice_mode}' ) else: twist = Twist() twist.linear.x, twist.angular.z = self.maneuver_twist self.cmd_vel_pub.publish(twist) elif self.voice_mode == 'STOP': self._stop() def _stop(self): self.cmd_vel_pub.publish(Twist()) def _log_motion(self, text): now = time.monotonic() if now - self.last_motion_log_time >= self.motion_log_interval: self.get_logger().info(text) self.last_motion_log_time = now def detection_callback(self, msg: Detection2DArray): """Generate autonomous obstacle, roaming, and person-following commands.""" if self.estop_active: self._stop() return # STOP and MANEUVER are controlled by the independent 20 Hz timer. if self.voice_mode != 'AUTO': return twist = Twist() if self.obstacle_detected: if self.obstacle_start_time is None: self.obstacle_start_time = time.time() elapsed = time.time() - self.obstacle_start_time if elapsed < 1.0: twist.linear.x = 0.0 twist.angular.z = 0.0 else: twist.linear.x = 0.0 twist.angular.z = 0.8 if self.free_direction == 'LEFT' else -0.8

self.cmd_vel_pub.publish(twist) if not self.obstacle_log_active: self.get_logger().warn( f'Obstacle detected ¦ Distance: {self.front_distance:.2f} m' ) self.obstacle_log_active = True return self.obstacle_start_time = None if self.obstacle_log_active: self.get_logger().info('Obstacle cleared') self.obstacle_log_active = False person_detections = [] if self.follow_enabled: for detection in msg.detections: if not detection.results: continue if detection.results[0].hypothesis.class_id == 'person': person_detections.append(detection) if not person_detections: if self.follow_enabled: elapsed = time.time() - self.last_target_time if elapsed < self.target_timeout: self.cmd_vel_pub.publish(twist) self._log_motion('Temporary target loss') return twist.linear.x = 0.4 twist.angular.z = 0.15 self.cmd_vel_pub.publish(twist) self._log_motion('Roaming freely') return target = None if self.target_id is not None: for detection in person_detections: if detection.id == self.target_id: target = detection break if target is None: target = max( person_detections, key=lambda detection: ( detection.bbox.size_x * detection.bbox.size_y ),

) self.target_id = target.id self.last_target_time = time.time() center_x = target.bbox.center.position.x width = target.bbox.size_x error = center_x - self.center_x if width > self.stop_width: twist.linear.x = 0.0 twist.angular.z = 0.0 self.get_logger().info( f'Target close stop ¦ ID:{self.target_id}' ) elif width > 250: twist.linear.x = 0.08 twist.angular.z = -error * 0.002 else: twist.linear.x = 0.5 twist.angular.z = -error * 0.003 if abs(error) < 25: twist.angular.z = 0.0 self.get_logger().info( f'Following target ¦ Error:{error:.1f} ¦ ' f'Distance:{self.front_distance:.2f} m' ) self.cmd_vel_pub.publish(twist) def main(args=None): rclpy.init(args=args) node = DecisionNode() try: rclpy.spin(node) except KeyboardInterrupt: if rclpy.ok(): node.get_logger().info('Decision node stopping...') finally: node.destroy_node() if rclpy.ok(): rclpy.shutdown() if __name__ == '__main__': main()

# Appendix I - watchdog_node.py

Control. Independent liveness/proximity/battery safety watchdog.

import math import time import rclpy from geometry_msgs.msg import Twist from rclpy.node import Node from rclpy.qos import ( DurabilityPolicy, HistoryPolicy, QoSProfile, ReliabilityPolicy, ) from sensor_msgs.msg import LaserScan from std_msgs.msg import Bool, Empty, Float32, String from vision_msgs.msg import Detection2DArray class WatchdogNode(Node): """Monitor critical robot data and enforce an emergency stop when unsafe.""" def __init__(self): super().__init__("watchdog_node") # ROS topics self.declare_parameter("scan_topic", "/scan") self.declare_parameter("obstacle_topic", "/obstacle_detected") self.declare_parameter( "tracked_detections_topic", "/tracked_detections", ) self.declare_parameter("camera_topic", "/camera/heartbeat") self.declare_parameter("cmd_vel_topic", "/cmd_vel") self.declare_parameter("battery_topic", "/battery_status") self.declare_parameter( "emergency_stop_topic", "/emergency_stop", ) self.declare_parameter("speech_topic", "/llm_response") self.declare_parameter("emotion_topic", "/robot_emotion") # Topic-liveness thresholds self.declare_parameter("scan_timeout", 1.5) self.declare_parameter("obstacle_timeout", 1.5) self.declare_parameter("tracked_detection_timeout", 2.5) self.declare_parameter("camera_timeout", 2.0) self.declare_parameter("cmd_vel_timeout", 2.0) # Battery-warning thresholds self.declare_parameter("battery_warn_pct", 20.0) self.declare_parameter("battery_critical_pct", 10.0) self.declare_parameter("battery_stale_sec", 15.0)

# Immediate LiDAR safety thresholds self.declare_parameter("critical_distance_m", 0.28) self.declare_parameter("critical_clear_distance_m", 0.35) self.declare_parameter("forward_cone_deg", 25.0) self.declare_parameter("min_forward_linear_x", 0.05) self.declare_parameter('front_center_angle_deg', 180.0) self.declare_parameter("check_rate_hz", 5.0) self.declare_parameter("startup_grace_sec", 10.0) self.scan_topic = str(self.get_parameter("scan_topic").value) self.obstacle_topic = str( self.get_parameter("obstacle_topic").value ) self.tracked_detections_topic = str( self.get_parameter("tracked_detections_topic").value ) self.camera_topic = str( self.get_parameter("camera_topic").value ) self.cmd_vel_topic = str( self.get_parameter("cmd_vel_topic").value ) self.battery_topic = str( self.get_parameter("battery_topic").value ) self.emergency_stop_topic = str( self.get_parameter("emergency_stop_topic").value ) self.speech_topic = str( self.get_parameter("speech_topic").value ) self.emotion_topic = str( self.get_parameter("emotion_topic").value ) self.timeouts = { "scan": float(self.get_parameter("scan_timeout").value), "obstacle_detected": float( self.get_parameter("obstacle_timeout").value ), "tracked_detections": float( self.get_parameter("tracked_detection_timeout").value ), "camera": float( self.get_parameter("camera_timeout").value ), "cmd_vel": float( self.get_parameter("cmd_vel_timeout").value ), }

self.battery_warn_pct = float( self.get_parameter("battery_warn_pct").value ) self.battery_critical_pct = float( self.get_parameter("battery_critical_pct").value ) self.battery_stale_sec = float( self.get_parameter("battery_stale_sec").value ) self.critical_distance_m = float( self.get_parameter("critical_distance_m").value ) self.critical_clear_distance_m = float( self.get_parameter("critical_clear_distance_m").value ) self.forward_cone_deg = float( self.get_parameter("forward_cone_deg").value ) self.min_forward_linear_x = float( self.get_parameter("min_forward_linear_x").value ) self.front_center_angle_deg = float( self.get_parameter('front_center_angle_deg').value ) self.check_rate_hz = float( self.get_parameter("check_rate_hz").value ) self.startup_grace_sec = float( self.get_parameter("startup_grace_sec").value ) reliable_qos = QoSProfile( history=HistoryPolicy.KEEP_LAST, depth=10, reliability=ReliabilityPolicy.RELIABLE, durability=DurabilityPolicy.VOLATILE, ) scan_qos = QoSProfile( history=HistoryPolicy.KEEP_LAST, depth=5, reliability=ReliabilityPolicy.BEST_EFFORT, durability=DurabilityPolicy.VOLATILE, ) now = time.time() self.last_seen = { "scan": now, "obstacle_detected": now, "tracked_detections": now,

"camera": now, "cmd_vel": now, } self.battery_pct = None self.battery_last_seen = now self.last_cmd_linear_x = 0.0 self.last_cmd_angular_z = 0.0 # Multiple faults may exist at once. The e-stop clears only when all # active reasons have been removed. self.active_reasons = set() self.emergency_active = False self.spoken_warnings = set() self.scan_sub = self.create_subscription( LaserScan, self.scan_topic, self._scan_callback, scan_qos, ) self.obstacle_sub = self.create_subscription( Bool, self.obstacle_topic, self._make_liveness_callback("obstacle_detected"), reliable_qos, ) self.detections_sub = self.create_subscription( Detection2DArray, self.tracked_detections_topic, self._make_liveness_callback("tracked_detections"), reliable_qos, ) self.camera_sub = self.create_subscription( Empty, self.camera_topic, self._make_liveness_callback("camera"), 1, ) self.cmd_vel_sub = self.create_subscription( Twist, self.cmd_vel_topic, self._cmd_vel_callback, reliable_qos, ) self.battery_sub = self.create_subscription( Float32, self.battery_topic, self._battery_callback, reliable_qos, ) self.estop_pub = self.create_publisher(

Bool, self.emergency_stop_topic, reliable_qos, ) # Directly publishing zero velocity remains effective even when the # decision node is the component that stopped responding. self.cmd_vel_pub = self.create_publisher( Twist, self.cmd_vel_topic, reliable_qos, ) self.speech_pub = self.create_publisher( String, self.speech_topic, reliable_qos, ) self.emotion_pub = self.create_publisher( String, self.emotion_topic, reliable_qos, ) self.startup_time = time.time() self.check_timer = self.create_timer( 1.0 / self.check_rate_hz, self._check_loop, ) self.get_logger().info( "Watchdog started - monitoring critical topics and battery" ) def _make_liveness_callback(self, key): """Create a callback that records when a monitored topic is received.""" def callback(_msg): self.last_seen[key] = time.time() return callback def _cmd_vel_callback(self, msg: Twist): """Track command liveness and whether the robot is moving forward.""" if self.emergency_active: return self.last_seen["cmd_vel"] = time.time() self.last_cmd_linear_x = msg.linear.x self.last_cmd_angular_z = msg.angular.z def _scan_callback(self, msg: LaserScan): """Check scan liveness and detect an immediate forward collision risk."""

self.last_seen["scan"] = time.time() cone = math.radians(self.forward_cone_deg) min_range = None for index, distance in enumerate(msg.ranges): if distance <= msg.range_min or distance > msg.range_max: continue angle = msg.angle_min + index * msg.angle_increment front_center = math.radians( self.front_center_angle_deg ) relative_angle = angle - front_center relative_angle = math.atan2( math.sin(relative_angle), math.cos(relative_angle), ) if abs(relative_angle) <= cone: if min_range is None or distance < min_range: min_range = distance moving_forward = ( self.last_cmd_linear_x > self.min_forward_linear_x ) critical_active = "critical_distance" in self.active_reasons if critical_active: front_clear = ( min_range is None or min_range > self.critical_clear_distance_m ) if front_clear: self.active_reasons.discard("critical_distance") self._clear_spoken("critical_distance") self._update_estop_state() return critical_distance = ( min_range is not None and min_range < self.critical_distance_m and moving_forward ) if critical_distance: self.active_reasons.add("critical_distance")

self._speak_once( "critical_distance", "Obstacle very close, stopping.", ) self.get_logger().error( "[WATCHDOG] Critical distance " f"{min_range:.2f} m while moving forward" ) self._update_estop_state() def _battery_callback(self, msg: Float32): """Store the latest battery percentage and reporting time.""" if msg.data <= 0.0 or msg.data > 100.0: return self.battery_pct = msg.data self.battery_last_seen = time.time() def _speak_once(self, key, text): """Publish a warning once until the associated condition clears.""" if key in self.spoken_warnings: return self.spoken_warnings.add(key) msg = String() msg.data = text self.speech_pub.publish(msg) self.get_logger().warn(f"[WATCHDOG] {text}") def _clear_spoken(self, key): """Allow a resolved warning to be announced again if it returns.""" self.spoken_warnings.discard(key) def _set_emotion(self, emotion): """Publish the emotion associated with the watchdog state.""" msg = String() msg.data = emotion self.emotion_pub.publish(msg) def _check_loop(self): """Check topic liveness, maintain the e-stop, and warn on battery.""" now = time.time() startup_deadline = ( self.startup_time + self.startup_grace_sec ) if now < startup_deadline: return for key, timeout in self.timeouts.items(): effective_last_seen = max( self.last_seen[key],

startup_deadline, ) age = now - effective_last_seen reason = f"stale_{key}" if key == "cmd_vel" and self.emergency_active: self.last_seen["cmd_vel"] = now if reason in self.active_reasons: self.active_reasons.discard(reason) self._clear_spoken(reason) continue if key == "cmd_vel": robot_was_moving = ( abs(self.last_cmd_linear_x) > 0.01 or abs(self.last_cmd_angular_z) > 0.01 ) if not robot_was_moving: if reason in self.active_reasons: self.active_reasons.discard(reason) self._clear_spoken(reason) continue if age > timeout: if reason not in self.active_reasons: self.active_reasons.add(reason) self._speak_once( reason, f'Warning, lost {key.replace("_", " ")} data.', ) elif reason in self.active_reasons: self.active_reasons.discard(reason) self._clear_spoken(reason) self._update_estop_state() if self.emergency_active: self._publish_estop(True) self._publish_failsafe_stop() # Startup time is also excluded from the battery-staleness age. effective_battery_last_seen = max( self.battery_last_seen, startup_deadline, ) battery_age = now - effective_battery_last_seen if battery_age > self.battery_stale_sec: self._speak_once( "battery_stale", "Warning, battery status is not reporting.",

) return self._clear_spoken("battery_stale") if self.battery_pct is None: return if self.battery_pct <= self.battery_critical_pct: self._speak_once( "battery_critical", "Battery critically low, Please charge me soon.", ) elif self.battery_pct <= self.battery_warn_pct: self._clear_spoken("battery_critical") self._speak_once( "battery_warn", f"Battery low, {self.battery_pct:.0f} percent.", ) else: self._clear_spoken("battery_critical") self._clear_spoken("battery_warn") def _update_estop_state(self): """Publish emergency-stop transitions when fault state changes.""" should_estop = bool(self.active_reasons) if should_estop and not self.emergency_active: self.emergency_active = True self._publish_estop(True) self._publish_failsafe_stop() self._set_emotion("sad") self.get_logger().error( "[WATCHDOG] EMERGENCY STOP - reasons: " f"{self.active_reasons}" ) elif not should_estop and self.emergency_active: self.emergency_active = False self._publish_estop(False) self._set_emotion("neutral") self.get_logger().info( "[WATCHDOG] All clear - emergency stop released" ) def _publish_estop(self, active: bool): """Publish the current emergency-stop state.""" msg = Bool() msg.data = active self.estop_pub.publish(msg) def _publish_failsafe_stop(self): """Publish a zero-velocity command immediately."""

self.last_cmd_linear_x = 0.0 self.last_cmd_angular_z = 0.0 self.cmd_vel_pub.publish(Twist()) def main(args=None): rclpy.init(args=args) node = WatchdogNode() try: rclpy.spin(node) except KeyboardInterrupt: if rclpy.ok(): node.get_logger().info("Watchdog node stopping...") finally: node.destroy_node() if rclpy.ok(): rclpy.shutdown() if __name__ == "__main__": main()

# Appendix J - motor_control_node.py

Control. Twist-to-PWM serial bridge and battery conversion.

import threading import time import rclpy import serial from geometry_msgs.msg import Twist from rclpy.node import Node from std_msgs.msg import Float32 class MotorControlNode(Node): """Convert velocity commands to motor PWM and publish battery percentage.""" def __init__(self): super().__init__("motor_control_node") # ROS interfaces and serial communication. self.declare_parameter("cmd_vel_topic", "/cmd_vel") self.declare_parameter("battery_status_topic", "/battery_status") self.declare_parameter("serial_port", "/dev/ttyACM0") self.declare_parameter("baudrate", 115200) self.declare_parameter("serial_timeout", 1.0) # Velocity-to-PWM conversion. self.declare_parameter("stop_linear_threshold", 0.02) self.declare_parameter("stop_angular_threshold", 0.02) self.declare_parameter("search_turn_multiplier", 250.0) self.declare_parameter("base_speed_multiplier", 250.0) self.declare_parameter("follow_turn_multiplier", 150.0) self.declare_parameter("right_motor_correction", 0.90) self.declare_parameter("max_pwm", 255) self.declare_parameter("motor_deadzone", 15) # Serial recovery timing. self.declare_parameter("reconnect_delay", 2.0) self.declare_parameter("serial_error_delay", 1.0) # Battery ADC and voltage-divider calibration. self.declare_parameter("adc_max_raw", 4095) self.declare_parameter("adc_vref", 3.3) self.declare_parameter("r1_ohms", 100000.0) self.declare_parameter("r2_ohms", 33000.0) self.declare_parameter("battery_full_voltage", 12.6) self.declare_parameter("battery_empty_voltage", 9.0) self.cmd_vel_topic = str(self.get_parameter("cmd_vel_topic").value) self.battery_status_topic = str( self.get_parameter("battery_status_topic").value ) self.serial_port = str(self.get_parameter("serial_port").value) self.baudrate = int(self.get_parameter("baudrate").value) self.serial_timeout = float(self.get_parameter("serial_timeout").value)

self.stop_linear_threshold = float( self.get_parameter("stop_linear_threshold").value ) self.stop_angular_threshold = float( self.get_parameter("stop_angular_threshold").value ) self.search_turn_multiplier = float( self.get_parameter("search_turn_multiplier").value ) self.base_speed_multiplier = float( self.get_parameter("base_speed_multiplier").value ) self.follow_turn_multiplier = float( self.get_parameter("follow_turn_multiplier").value ) self.right_motor_correction = float( self.get_parameter("right_motor_correction").value ) self.max_pwm = int(self.get_parameter("max_pwm").value) self.motor_deadzone = int(self.get_parameter("motor_deadzone").value) self.reconnect_delay = float( self.get_parameter("reconnect_delay").value ) self.serial_error_delay = float( self.get_parameter("serial_error_delay").value ) self.adc_max_raw = int(self.get_parameter("adc_max_raw").value) self.adc_vref = float(self.get_parameter("adc_vref").value) self.r1_ohms = float(self.get_parameter("r1_ohms").value) self.r2_ohms = float(self.get_parameter("r2_ohms").value) self.divider_ratio = self.r2_ohms / (self.r1_ohms + self.r2_ohms) self.battery_full_voltage = float( self.get_parameter("battery_full_voltage").value ) self.battery_empty_voltage = float( self.get_parameter("battery_empty_voltage").value ) self.cmd_vel_sub = self.create_subscription( Twist, self.cmd_vel_topic, self.cmd_callback, 10, ) self.battery_pub = self.create_publisher( Float32, self.battery_status_topic, 10, )

self.ser = None try: self.ser = serial.Serial( self.serial_port, self.baudrate, timeout=self.serial_timeout, ) self.get_logger().info( f"ESP32 serial connected on {self.serial_port}" ) except Exception as error: self.get_logger().error(f"Serial connection failed: {error}") # Read battery messages without blocking motor-command writes. self.stop_flag = False self.reader_thread = threading.Thread( target=self._serial_read_loop, daemon=True, ) self.reader_thread.start() self.get_logger().info("Motor control node started") def cmd_callback(self, msg: Twist): """Convert a Twist command into left and right motor PWM values.""" linear = msg.linear.x angular = msg.angular.z if ( abs(linear) < self.stop_linear_threshold and abs(angular) < self.stop_angular_threshold ): left_speed = 0 right_speed = 0 elif ( abs(linear) < self.stop_linear_threshold and abs(angular) >= self.stop_angular_threshold ): turn_speed = int(angular * self.search_turn_multiplier) left_speed = -turn_speed right_speed = turn_speed else: base_speed = int(linear * self.base_speed_multiplier) turn_speed = int(angular * self.follow_turn_multiplier) left_speed = base_speed - turn_speed right_speed = base_speed + turn_speed right_speed = int(right_speed * self.right_motor_correction) left_speed = max(min(left_speed, self.max_pwm), -self.max_pwm) right_speed = max(min(right_speed, self.max_pwm), -self.max_pwm)

if abs(left_speed) < self.motor_deadzone: left_speed = 0 if abs(right_speed) < self.motor_deadzone: right_speed = 0 command = f"{left_speed},{right_speed}\n" try: if self.ser: self.ser.write(command.encode()) except Exception as error: self.get_logger().error(f"Serial write failed: {error}") def _serial_read_loop(self): """Read battery ADC messages from the ESP32 in a background thread.""" while not self.stop_flag: if self.ser is None: time.sleep(self.reconnect_delay) self._reconnect_serial() continue try: line = ( self.ser.readline() .decode("utf-8", errors="ignore") .strip() ) if not line.startswith("BATT:"): continue raw = int(line.split(":", 1)[1]) percent = self._raw_to_percent(raw) battery_msg = Float32() battery_msg.data = percent self.battery_pub.publish(battery_msg) self.get_logger().info(f"[BATTERY] {percent:.1f}%") except Exception as error: self.get_logger().warn( f"[BATTERY] Serial read error: {error}" ) time.sleep(self.serial_error_delay) def _reconnect_serial(self): """Attempt to reopen the configured serial port.""" try: self.ser = serial.Serial( self.serial_port, self.baudrate, timeout=self.serial_timeout, ) self.get_logger().info( f"Reconnected to {self.serial_port}"

) except Exception: self.ser = None def _raw_to_percent(self, raw: int) -> float: """Convert an ADC sample into a clamped battery percentage.""" raw = max(0, min(self.adc_max_raw, raw)) adc_voltage = (raw / self.adc_max_raw) * self.adc_vref battery_voltage = adc_voltage / self.divider_ratio voltage_span = ( self.battery_full_voltage - self.battery_empty_voltage ) percent = ( (battery_voltage - self.battery_empty_voltage) / voltage_span * 100.0 ) return max(0.0, min(100.0, percent)) def main(args=None): rclpy.init(args=args) node = MotorControlNode() try: rclpy.spin(node) except KeyboardInterrupt: if rclpy.ok(): node.get_logger().info("Motor control node stopping...") finally: node.stop_flag = True # Send a final motor stop before closing the serial connection. if node.ser: try: node.ser.write(b"0,0\n") node.ser.flush() node.ser.close() node.get_logger().info("Serial port closed cleanly") except Exception as error: node.get_logger().error( "Failed to send stop command during shutdown: " f"{error}" ) node.destroy_node() if rclpy.ok(): rclpy.shutdown() if __name__ == "__main__": main()

# Appendix K - display_node.py (optional)

Debug/Visualization. Optional local OpenCV debug display.

import cv2 import rclpy from cv_bridge import CvBridge from rclpy.node import Node from rclpy.qos import ( DurabilityPolicy, HistoryPolicy, QoSProfile, ReliabilityPolicy, ) from sensor_msgs.msg import Image from vision_msgs.msg import Detection2DArray class DisplayNode(Node): """Draw tracked detections on camera frames and publish a debug image.""" def __init__(self): super().__init__("display_node") self.bridge = CvBridge() self.latest_detections = None image_qos = QoSProfile( history=HistoryPolicy.KEEP_LAST, depth=1, reliability=ReliabilityPolicy.BEST_EFFORT, durability=DurabilityPolicy.VOLATILE, ) detection_qos = QoSProfile( history=HistoryPolicy.KEEP_LAST, depth=10, reliability=ReliabilityPolicy.RELIABLE, durability=DurabilityPolicy.VOLATILE, ) self.image_sub = self.create_subscription( Image, "/camera/image_raw", self.image_callback, image_qos, ) self.detection_sub = self.create_subscription( Detection2DArray, "/tracked_detections", self.detection_callback, detection_qos, ) self.debug_image_pub = self.create_publisher( Image, "/rviz_debug_image", 10,

) self.get_logger().info("Display node started") def detection_callback(self, msg: Detection2DArray): """Store the latest tracked detections for the next image frame.""" self.latest_detections = msg def image_callback(self, msg: Image): """Draw the latest detections on an incoming camera frame.""" frame = self.bridge.imgmsg_to_cv2( msg, desired_encoding="bgr8", ) if self.latest_detections is not None: for detection in self.latest_detections.detections: cx = detection.bbox.center.position.x cy = detection.bbox.center.position.y width = detection.bbox.size_x height = detection.bbox.size_y x_min = int(cx - width / 2) y_min = int(cy - height / 2) x_max = int(cx + width / 2) y_max = int(cy + height / 2) if detection.results: hypothesis = detection.results[0].hypothesis label = ( f"{hypothesis.class_id} " f"ID:{detection.id} " f"{hypothesis.score:.2f}" ) else: label = "Object" cv2.rectangle( frame, (x_min, y_min), (x_max, y_max), (0, 255, 0), 2, ) cv2.putText( frame, label, (x_min, max(20, y_min - 10)), cv2.FONT_HERSHEY_SIMPLEX, 0.6, (0, 255, 0), 2,

) output_msg = self.bridge.cv2_to_imgmsg(frame, "bgr8") output_msg.header = msg.header self.debug_image_pub.publish(output_msg) cv2.imshow("YOLO/MediaPipe Detections", frame) cv2.waitKey(1) def main(args=None): rclpy.init(args=args) node = DisplayNode() try: rclpy.spin(node) except KeyboardInterrupt: if rclpy.ok(): node.get_logger().info("Display node stopping...") finally: node.destroy_node() if rclpy.ok(): rclpy.shutdown() if __name__ == "__main__": main()

# Appendix L - Vector_AMR_ESP32_reference.ino

Compatible reconstructed firmware. See Chapter 8 for provenance: pin map/protocol behaviors are recovered; periodic battery interval and explicit ADC resolution line are reconstruction details.

// Vector AMR - ESP32-C3 motor controller reference firmware // Reconstructed to match the recovered project pin map and Jetson serial protocol. // Recovered project details: motor pins, enable pins, 115200 baud, signed left/right // commands, analogWrite direction control, battery ADC on GPIO1, and BATT:<raw> output. // The periodic battery-report interval and explicit 12-bit ADC line are documented // here as a compatible reconstruction because the final .ino file was not present. const int L_RPWM = 2; const int L_LPWM = 3; const int R_RPWM = 4; const int R_LPWM = 5; const int L_REN = 6; const int L_LEN = 7; const int R_REN = 8; const int R_LEN = 9; const int BATTERY_PIN = 1; // GPIO1 / ADC1 const unsigned long BATTERY_REPORT_INTERVAL_MS = 1000; unsigned long lastBatteryReportMs = 0; void setMotor(int rpwmPin, int lpwmPin, int speedValue) { speedValue = constrain(speedValue, -255, 255); if (speedValue > 0) { analogWrite(rpwmPin, speedValue); analogWrite(lpwmPin, 0); } else if (speedValue < 0) { analogWrite(rpwmPin, 0); analogWrite(lpwmPin, abs(speedValue)); } else { analogWrite(rpwmPin, 0); analogWrite(lpwmPin, 0); } } void stopMotors() { setMotor(L_RPWM, L_LPWM, 0); setMotor(R_RPWM, R_LPWM, 0); } void handleMotorCommand(const String &line) { int commaIndex = line.indexOf(','); if (commaIndex < 0) { return; } int leftSpeed = line.substring(0, commaIndex).toInt(); int rightSpeed = line.substring(commaIndex + 1).toInt(); leftSpeed = constrain(leftSpeed, -255, 255);

rightSpeed = constrain(rightSpeed, -255, 255); setMotor(L_RPWM, L_LPWM, leftSpeed); setMotor(R_RPWM, R_LPWM, rightSpeed); } void reportBatteryRaw() { int raw = analogRead(BATTERY_PIN); Serial.print("BATT:"); Serial.println(raw); } void setup() { Serial.begin(115200); pinMode(L_RPWM, OUTPUT); pinMode(L_LPWM, OUTPUT); pinMode(R_RPWM, OUTPUT); pinMode(R_LPWM, OUTPUT); pinMode(L_REN, OUTPUT); pinMode(L_LEN, OUTPUT); pinMode(R_REN, OUTPUT); pinMode(R_LEN, OUTPUT); digitalWrite(L_REN, HIGH); digitalWrite(L_LEN, HIGH); digitalWrite(R_REN, HIGH); digitalWrite(R_LEN, HIGH); analogReadResolution(12); stopMotors(); } void loop() { if (Serial.available()) { String line = Serial.readStringUntil('\n'); line.trim(); if (line.length() > 0) { handleMotorCommand(line); } } unsigned long now = millis(); if (now - lastBatteryReportMs >= BATTERY_REPORT_INTERVAL_MS) { lastBatteryReportMs = now; reportBatteryRaw(); } }

# Appendix M - Source snapshot manifest and revision notes

| Handbook name | Recovered file | Area | Notes |
| --- | --- | --- | --- |
| camera_node.py | Pasted text (2)(10).txt | Perception | Current camera snapshot (1640x1232 capture -> 640x480 working frame). |
| detector_node.py | detector_node_cleaned.py | Perception | YOLOv8n-pose TensorRT detector with hand-gesture audio control. |
| tracking_node.py | tracker_node.py | Perception | SORT-style tracking with Kalman prediction, IoU and Hungarian assignment. |
| obstacle_detection_node.py | obstacle_detection_node(1).py | Perception | LiDAR sector processing with detection/clear hysteresis and direction lock. |
| stt_node.py | stt_node.py | Interaction | Whisper tiny.en wake-word + command speech recognition. |
| llm_node.py | Pasted text (2)(9).txt | Interaction | Local intent router plus NVIDIA NIM text/vision calls. |
| tts_node.py | tts_node_piper_updated_cleaned.py | Interaction | Offline Piper TTS, persistent audio playback, cache and lip-sync. |
| decision_node.py | Pasted text (2)(20260820-114602).txt | Control | Latest supplied decision snapshot with 1 m timed linear maneuver. |
| watchdog_node.py | watchdog_node(1).py | Control | Independent liveness/proximity/battery safety watchdog. |
| motor_control_node.py | motor_control_node_cleaned.py | Control | Twist-to-PWM serial bridge and battery conversion. |
| display_node.py (optional) | display_node_cleaned.py | Debug/Visualization | Optional local OpenCV debug display. |

## Historical/current differences that matter

Camera: earlier handbook/runs used 1920×1080 capture; the recovered later camera snapshot uses 1640×1232 while still publishing a 640×480 working frame.

Detector: the current recovered detector is YOLOv8n-pose.engine rather than the older plain yolov8n.engine description, because pose keypoints are now used for hand gestures.

Obstacle/Watchdog: the current code explicitly centers physical front at 180° and includes separate clear thresholds to prevent chattering.

Decision: the August 20 supplied snapshot uses 1.0 m for timed FORWARD/BACKWARD commands, matching the later movement requirement.

Watchdog output in current code is /emergency_stop. Earlier documentation sometimes called the concept /safety_stop; use /emergency_stop when describing the current implementation.