VECTOR AMR

# Complete Project Learning Handbook

ROS 2 • Computer Vision • LiDAR • Voice AI • Embedded Control • Web Interface

A technical study guide for understanding the complete autonomous mobile robot from sensor input to physical motion.

| Platform | Final stack |
| --- | --- |
| Compute | NVIDIA Jetson Orin Nano Developer Kit, 8 GB |
| OS / middleware | JetPack 6.1/ Ubuntu 22.04 on Jetson; ROS 2 Humble inside Docker |
| Robot type | 4-wheel differential-drive autonomous mobile robot |
| Primary functions | Person following, autonomous roaming/search, obstacle avoidance, voice interaction, scene description, battery reporting, browser UI |
| Primary language | Python for custom ROS 2 nodes; C/C++ in external drivers/libraries; Arduino-style C++ on ESP32 |

# How to use this handbook

This handbook is written as a learning document rather than a README. Read Chapters 1–4 first to understand the system, then study each node in Chapter 6. Chapters 8–11 explain the external repositories, runtime flow, performance work, and interview-level concepts.

| Chapter | Focus |
| --- | --- |
| 1 | Project purpose and system architecture |
| 2 | Hardware |
| 3 | Software stack and AI models |
| 4 | ROS 2 packages, node count, and topic map |
| 5 | Data flow |
| 6 | Detailed node-by-node study |
| 7 | Robot behaviours and decision logic |
| 8 | External repositories: SLAMTEC LiDAR, web_video_server, rosbridge |
| 9 | Web dashboard and interaction |
| 10 | Launch, Docker, and bringup |
| 11 | Performance and optimization |
| 12 | Important design decisions and lessons |
| 13 | Study questions / interview revision |

# 1. Project overview

Vector is a real, hardware-deployed AMR built around ROS 2 Humble. The design is modular: sensing, perception, interaction, decision-making, safety, actuation, and browser visualization communicate through ROS topics rather than being written as one monolithic program.

## 1.1 What the robot can do

Capture a live CSI-camera stream and publish it as ROS images.

Run real-time YOLOv8 object detection using a TensorRT engine on the Jetson GPU.

Track detected objects across frames and maintain track IDs.

Use a 2D RPLIDAR C1 scan to detect nearby frontal obstacles and estimate the freer side.

Follow a person, keep distance, search/roam when the person is absent, and react to obstacles.

Accept wake-word and voice commands, answer short questions through NVIDIA NIM, and describe the camera scene with a vision-language model.

Generate local speech with Piper TTS and coordinate STT muting while the robot is speaking.

Send differential left/right motor commands to an ESP32-C3 over USB serial.

Read and publish battery status.

Expose robot state and camera video to a browser dashboard through rosbridge and web_video_server.

Watch critical subsystems through a watchdog and force a safety stop when required.

## 1.2 Layered architecture

| Layer | Components | Responsibility |
| --- | --- | --- |
| Sensors | IMX219 camera, RPLIDAR C1, USB microphone, battery ADC | Observe the environment and robot state |
| Perception | camera, detector, tracking, obstacle detection | Convert raw sensing into useful structured information |
| Interaction | STT, LLM/vision, TTS | Human–robot voice and conversational interface |
| Decision / safety | decision node, watchdog | Choose motion and prevent unsafe operation |
| Actuation | motor-control node, ESP32-C3, BTS7960 | Convert ROS velocity intent into motor PWM |
| Web | rosbridge, web_video_server, HTML/CSS/JS UI | Remote visualization and dashboard control |

# 2. Hardware handbook

| Hardware | Exact / project specification | Role |
| --- | --- | --- |
| Main computer | NVIDIA Jetson Orin Nano Developer Kit, 8 GB | Runs ROS 2, TensorRT perception, STT, LLM client, TTS, control and web services |
| Camera | Waveshare IMX219-120 CSI, ~120° FOV | Primary visual sensor |
| LiDAR | SLAMTEC RPLIDAR C1M1, ToF, up to ~12 m | 2D ranging for obstacle detection |
| Microcontroller | ESP32-C3 Super Mini, USB-C | Receives motor commands and reads battery ADC |
| Motor drivers | 2 × BTS7960 43 A H-bridge modules | Drive left and right motor groups |
| Motors | 4 × Johnson 12 V, 100 RPM DC geared motors | 4WD differential-drive propulsion |
| Battery | 11.1 V 6000 mAh 3S2P Li-ion pack | Main robot power |
| Charger | 12.6 V, 2 A | 3S battery charging |
| Buck converter | XL4015 5 A step-down | Voltage regulation where required |
| Battery sensing | 100 kΩ + 33 kΩ divider to ESP32-C3 ADC GPIO1 | Scales pack voltage for ADC measurement |
| Microphone | USB PnP microphone | Speech input |
| Speaker | Bluetooth speaker | Robot speech output |
| Chassis | 4WD metal mobile-robot chassis | Mechanical platform |

## 2.1 Camera pipeline

The CSI camera uses NVIDIA's Jetson multimedia path. The working GStreamer chain is conceptually: nvarguscamerasrc → nvvidconv → videoconvert → appsink. Capture was configured at 1920×1080 @ 30 FPS and converted to a 640×480 BGR working frame. appsink uses drop=true and sync=false so old frames are discarded rather than building latency.

Camera calibration was performed with an 8×6 chessboard and 0.025 m square size. A representative intrinsic matrix recorded during the project was approximately K=[489.7,0,317.46; 0,654.72,252.84; 0,0,1].

## 2.2 Differential drive

The four motors are electrically/logically grouped into a left side and a right side. ROS publishes a Twist command; the motor-control node converts linear/angular intent into left/right PWM values in approximately −255…255 and sends a compact serial message to the ESP32. Equal positive PWM drives forward; unequal values turn; opposite signs rotate in place.

# 3. Software, frameworks, and model inventory

| Component | Model / technology | Where it runs | Purpose |
| --- | --- | --- | --- |
| Object detector | YOLOv8n → TensorRT engine (yolov8n.engine) | Jetson GPU | Person/object detection |
| Tracker | Kalman Filter (8-state/4-measurement) + Hungarian assignment | CPU | Persistent object IDs |
| STT | Whisper tiny.en / faster-whisper-style CTranslate2 pipeline | Jetson; CUDA-enabled configuration used in final interaction stack | English speech transcription |
| Text LLM | meta/llama-3.1-8b-instruct via NVIDIA NIM | NVIDIA cloud API | Short conversational replies |
| Vision LLM | meta/llama-3.2-11b-vision-instruct via NVIDIA NIM | NVIDIA cloud API | One-shot scene description from camera frame |
| TTS | Piper ONNX, en_US-hfc_female-medium final configured default | Jetson CPU | Offline speech synthesis |
| ROS | ROS 2 Humble Hawksbill | Docker on Jetson | Middleware / modular node communication |
| DL runtime | TensorRT + CUDA | Jetson GPU | Fast YOLO inference |
| Vision | OpenCV + CvBridge | Jetson | Image conversion/encoding |
| Web ROS bridge | rosbridge_suite / rosbridge_websocket | Jetson | ROS ↔ browser JSON/WebSocket bridge |
| Video | RobotWebTools web_video_server | Jetson | HTTP camera streaming |
| Model distinction: YOLO and Piper are local. The NVIDIA NIM text/vision models are remote API models. The tracker and obstacle detector are algorithms, not learned AI models. |  |  |  |

# 4. ROS 2 package and node inventory

The final full runtime can be understood as 10 custom project nodes plus 3 external runtime nodes = 13 principal ROS nodes. An optional local display node used in earlier/alternate builds makes 14 when launched.

| # | Node | Package / source | Type |
| --- | --- | --- | --- |
| 1 | camera_node | perception_pkg | Custom Python |
| 2 | detector_node | perception_pkg | Custom Python |
| 3 | tracking_node | perception_pkg | Custom Python |
| 4 | obstacle_detection_node | perception_pkg | Custom Python |
| 5 | stt_node | interaction_pkg | Custom Python |
| 6 | llm_node | interaction_pkg | Custom Python |
| 7 | tts_node | interaction_pkg | Custom Python |
| 8 | decision_node | control_pkg | Custom Python |
| 9 | watchdog_node | control_pkg | Custom Python |
| 10 | motor_control_node | control_pkg | Custom Python |
| 11 | rplidar_node | SLAMTEC LiDAR driver package | External C/C++ driver |
| 12 | rosbridge_websocket | rosbridge_suite | External ROS web bridge |
| 13 | web_video_server | RobotWebTools web_video_server | External C++ ROS node |
| Optional | display_node | display/custom package in earlier architecture | Custom Python |

## 4.1 Important topic map

| Topic | Type | Main producer | Main consumers / meaning |
| --- | --- | --- | --- |
| /camera/image_raw | sensor_msgs/Image | camera | detector, LLM vision, web video |
| /camera/image_compressed | sensor_msgs/CompressedImage | camera | web/alternate clients |
| /camera/camera_info | sensor_msgs/CameraInfo | camera | camera calibration metadata |
| /camera/heartbeat | lightweight heartbeat | camera | watchdog |
| /detections | vision_msgs/Detection2DArray | detector | tracker |
| /tracked_detections | vision_msgs/Detection2DArray | tracker | decision |
| /scan | sensor_msgs/LaserScan | RPLIDAR driver | obstacle detection, watchdog |
| /obstacle_detected | std_msgs/Bool | obstacle detection | decision, watchdog |
| /front_distance | std_msgs/Float32 | obstacle detection | decision, watchdog |
| /free_direction | std_msgs/String | obstacle detection | decision |
| /voice_command | std_msgs/String | STT | decision, LLM, TTS command confirmations |
| /llm_response | std_msgs/String | LLM | TTS / UI response path |
| /tts_status | std_msgs/String | TTS | STT coordination / UI |
| /robot_emotion | std_msgs/String | TTS | web face UI |
| /mouth_level | std_msgs/Float32 | TTS | web lip-sync |
| /audio_enabled | std_msgs/Bool | web/interaction control | STT/TTS |
| /robot_ui_mode | std_msgs/String | LLM/UI control | browser UI |
| /cmd_vel | geometry_msgs/Twist | decision | motor control, watchdog |
| /battery_status | std_msgs/Float32 | motor control | LLM, watchdog, UI |
| /safety_stop | std_msgs/Bool | watchdog | safety/control path |

# 5. End-to-end data flow

## 5.1 Person-following path

Camera → /camera/image_raw → YOLO TensorRT detector → /detections → tracker → /tracked_detections → decision node → /cmd_vel → motor-control node → USB serial → ESP32 → BTS7960 → motors.

## 5.2 Obstacle path

RPLIDAR C1 → rplidar_node → /scan → obstacle_detection_node → /obstacle_detected + /front_distance + /free_direction → decision node → safer /cmd_vel.

## 5.3 Voice conversation path

USB microphone → STT → /voice_command → LLM routing → NVIDIA NIM (when needed) → /llm_response → Piper TTS → Bluetooth speaker. TTS status is also used to prevent the microphone from transcribing the robot's own speech.

## 5.4 Vision-question path

Voice command such as “what do you see?” → STT → LLM node recognizes a scene-description intent → copies latest camera frame → ROS Image to OpenCV → resize/JPEG/base64 → NVIDIA vision model → short text → /llm_response → Piper → speaker.

# 6. Detailed node-by-node handbook

## 6.1 camera_node

Camera acquisition and ROS publishing

Owns the Jetson CSI/GStreamer camera pipeline and continuously obtains the newest frame.

Publishes raw and compressed image streams plus CameraInfo.

Uses sensor-style BEST_EFFORT QoS with shallow depth for image data so latency is preferred over guaranteed delivery.

Final optimization added a lightweight ~1 Hz /camera/heartbeat tied to successful frame reception, allowing watchdog health checking without copying full images.

Key design idea: for real-time robotics, a fresh frame is usually more valuable than an old guaranteed frame.

## 6.2 detector_node

GPU object perception

Loads yolov8n.engine, the TensorRT-converted YOLOv8 nano model.

Uses approximately imgsz=640 and confidence threshold around 0.6 in the established configuration.

Subscribes to camera images, runs inference, converts results into Detection2DArray, and publishes /detections.

The engine is pre-warmed with a dummy input to avoid a large first-inference penalty.

Typical measured performance after optimization: roughly 26–29 FPS, with project logs commonly around 27–30 FPS and ~30–45 ms class of latency.

## 6.3 tracking_node

Temporal object tracking

Consumes /detections and maintains identities across consecutive frames.

Uses a Kalman Filter with dim_x=8 and dim_z=4 to predict bounding-box motion/state.

Uses Hungarian assignment to match current detections to predicted tracks.

Publishes /tracked_detections including track ID/class/score information.

Tracking lets the decision layer reason about a continuing target rather than treating every frame as unrelated.

## 6.4 obstacle_detection_node

2D LiDAR interpretation

Consumes /scan from the RPLIDAR driver.

Examines a frontal angular sector (historically ~±20° around front) and compares ranges with an obstacle threshold around 0.50 m.

Publishes whether an obstacle exists, the nearest front distance, and a LEFT/RIGHT free-direction recommendation.

Compares left-side and right-side distance statistics to select the freer direction.

Logging was optimized from scan-rate spam to about one compact statistics line per second, with obstacle warnings only when state changes.

## 6.5 stt_node

Wake word, VAD, and speech transcription

Captures microphone audio and detects speech/silence before transcription.

Uses the lightweight English Whisper tiny.en family in a CTranslate2/faster-whisper-style pipeline; final settings included beam_size=1, best_of=1, temperature=0.0 and no_speech_threshold=0.6.

Recognizes the wake word “Vector”, gathers the following command, and publishes normalized commands on /voice_command.

Includes hallucination/repetition filtering and responsive shutdown logic.

Listens to TTS state so STT is muted while Vector is speaking, reducing feedback loops.

## 6.6 llm_node

Intent router + cloud text/vision intelligence

Receives /voice_command but does not send every command to an LLM. Local routing handles UI mode, battery, known robot controls, follow/explore acknowledgements, and vision triggers first.

Uses NVIDIA's OpenAI-compatible NIM endpoint. Text model: meta/llama-3.1-8b-instruct. Vision model: meta/llama-3.2-11b-vision-instruct.

Caches the latest camera frame. For vision questions it converts to OpenCV, downsizes when required, JPEG-encodes, base64-encodes, and submits one image with a short scene prompt.

The text system prompt identifies the robot as Vector and requests concise 10–15-word plain-text replies.

Rate limiting, duplicate suppression, vision cooldown, and detailed timing logs prevent request storms and help profile latency.

## 6.7 tts_node

Offline voice output and robot face animation

Loads a Piper ONNX voice locally on CPU (use_cuda=False). The preserved final default is en_US-hfc_female-medium; HFC male/female medium voices are supported in the cleaned node.

Subscribes to /llm_response and /voice_command, queues speech, and publishes SPEAKING/IDLE status.

Uses a persistent sounddevice output stream instead of repeatedly opening audio hardware.

Maintains a very quiet 40 Hz keep-alive option to keep Bluetooth audio awake.

Caches fixed phrases lazily after first synthesis, publishes robot emotion, and derives normalized RMS mouth levels for lip-sync.

## 6.8 decision_node

Robot behaviour and motion decisions

Fuses tracked person detections, LiDAR obstacle information, free direction, and voice commands.

Main behavioural concepts are FOLLOW, SEARCH/ROAM, PAUSED/stop, plus direct manual motion commands.

Person-following uses horizontal bounding-box center error relative to frame center (~320 px) to compute angular correction. A turn threshold around 60 px prevents unnecessary oscillation.

Bounding-box width is used as a simple visual distance proxy; a stop-width setting around 450 px prevents the robot from approaching too closely.

On temporary target loss it holds/reacquires for a short timeout (~2 s); in search mode it moves/turns to find a target; obstacle logic temporarily overrides normal following and chooses the freer LiDAR side.

## 6.9 watchdog_node

Independent safety supervision

Monitors critical liveness/state signals such as LiDAR, camera heartbeat, obstacle/front-distance, battery and cmd_vel.

Publishes /safety_stop when configured critical conditions are violated.

The watchdog is intentionally separate from decision logic: a fault in normal behaviour code should not remove the independent safety layer.

An important optimization replaced subscription to full /camera/image_raw with /camera/heartbeat, reducing DDS image-copy overhead and watchdog CPU from roughly 20–26% to about 11–17% in profiling.

## 6.10 motor_control_node

ROS-to-embedded actuation bridge

Subscribes to /cmd_vel and maps Twist linear/angular values into differential left/right motor commands.

Sends compact left,right PWM values over /dev/ttyACM0 at 115200 baud to the ESP32-C3.

Commands are constrained to the motor-control range (approximately −255…255).

Sends a stop command during shutdown so the motors are not left running when ROS exits normally.

Receives/parses battery information from the microcontroller path and publishes /battery_status.

# 7. Behaviour logic

| Situation | Decision behaviour |
| --- | --- |
| Person visible, no obstacle | Center target using angular.z; move forward if target appears far enough |
| Person too close | Set/approach zero linear velocity using bounding-box width threshold |
| Person temporarily lost | Short hold/reacquisition window before changing behaviour |
| Search/roam | Move autonomously and periodically turn while looking for a person |
| Obstacle ahead | Prioritize obstacle handling; use /free_direction to turn/veer toward clearer side |
| Wake word / stop | Pause or immediately stop as defined by command logic |
| Manual command | Forward/backward/left/right/stop overrides autonomous intent for the command path |
| Safety fault | Watchdog publishes safety stop; safety takes priority over normal behaviour |

## 7.1 Why bounding-box width works for distance keeping

No monocular depth model was required. For the same person, a larger bounding box generally means the person occupies more of the image and is therefore closer. This is not metric depth, but it is cheap and adequate for a follow/stop heuristic. A future encoder/depth/SLAM system could replace this heuristic with metric distance.

# 8. External repositories used by the project

## 8.1 SLAMTEC RPLIDAR ROS 2 driver

The project uses a SLAMTEC ROS driver to turn the RPLIDAR C1 serial data stream into standard ROS LaserScan messages. The final runtime process appears as rplidar_node and publishes /scan. The driver is external code and is predominantly C/C++, so there was no need to rewrite the hardware protocol in Python.

For ROS 2, SLAMTEC's current dedicated repository is sllidar_ros2. It includes C1 launch support (for example, view_sllidar_c1_launch.py). The older Slamtec/rplidar_ros repository is the long-standing RPLIDAR ROS package and also documents C1 support. In the handbook, treat the driver as the hardware-abstraction layer: your obstacle node should consume /scan and remain independent of the low-level serial protocol.

Repository: https://github.com/Slamtec/sllidar_ros2

Related legacy/long-standing repository: https://github.com/Slamtec/rplidar_ros

### What the LiDAR driver does for us

Opens/configures the LiDAR communication interface.

Starts scanning and decodes SLAMTEC scan packets.

Converts polar samples into sensor_msgs/LaserScan.

Publishes standardized ranges/angles so the rest of the robot is vendor-independent.

Handles device-specific details such as baud rate and scan modes.

## 8.2 RobotWebTools web_video_server

web_video_server is an external RobotWebTools ROS package used to expose ROS image topics through HTTP. Instead of implementing MJPEG/H.264 streaming ourselves, the dashboard can request the camera topic from this server. It supports ROS 2 Humble+, multiple stream formats, snapshots, QoS selection, resizing, and quality controls.

Repository: https://github.com/RobotWebTools/web_video_server

Default HTTP port is commonly 8080.

Can stream raw ROS image topics as MJPEG, VP8, VP9, H264, PNG, or ROS-compressed streams.

The project intentionally left this cloned component unchanged because it already worked correctly.

This separation is useful: the camera node only publishes ROS images; the web-video component is responsible for HTTP delivery.

## 8.3 rosbridge_suite

rosbridge_suite provides the browser-facing JSON/WebSocket bridge. The rosbridge_server package exposes ROS publishers, subscribers and related functionality through WebSockets. This is what allows JavaScript in the dashboard to exchange ROS messages without running a native ROS client in the browser.

Repository: https://github.com/RobotWebTools/rosbridge_suite

rosbridge_websocket normally uses port 9090 unless configured otherwise.

Browser JavaScript can subscribe/publish through the rosbridge protocol.

Video does not need to travel through rosbridge; web_video_server handles the image stream more efficiently as HTTP video.

# 9. Web dashboard and robot face

The browser interface combines two communication paths: rosbridge for lightweight ROS state/control messages and web_video_server for camera imagery. The dashboard includes face/vision controls, audio ON/OFF, fullscreen/close controls, and the camera feed. Voice commands can also switch face/vision UI modes through /robot_ui_mode.

## 9.1 Why two web channels?

| Channel | Best for | Reason |
| --- | --- | --- |
| rosbridge WebSocket | commands, state, emotion, mouth level, audio state | Small structured ROS messages; bidirectional |
| web_video_server HTTP | camera images/video | Designed for image streaming and browser-compatible codecs/formats |

# 10. Bringup, Docker, and repository organization

The deployed system is launched from robot_bringup, with the final process listing showing ros2 launch robot_bringup params_my_robot.launch.py. The stack runs in the Jetson ROS 2 Docker environment. A start_robot.sh wrapper was used to enter/execute the container, source the workspace, set performance-related environment variables, and launch the robot.

## 10.1 Key runtime environment

Base history includes dustynv/l4t-pytorch:r36.4.0 and ROS 2 Humble installation/integration.

Jetson software recorded CUDA 12.6 and PyTorch 2.6.0+cu126 with CUDA available.

YOLO inference uses TensorRT rather than ONNX Runtime GPU.

Thread limits added before launch: OMP_NUM_THREADS=1, OPENBLAS_NUM_THREADS=1, MKL_NUM_THREADS=1, NUMEXPR_NUM_THREADS=1.

USB devices such as ESP32 serial and microphone must be accessible to the container; camera access depends on Jetson/NVIDIA runtime integration.

## 10.2 Learning repository progression

| Phase | Main learning |
| --- | --- |
| Phase 1 | ROS 2 basics, publishers/subscribers, services, USB camera/WSL setup |
| Phase 2 | Camera, object detection, display, TF/RViz learning, YOLO ONNX experiments |
| Phase 3 | Jetson deployment, IMX219 CSI, Docker, CUDA/TensorRT, tracker |
| Phase 4 | Decision logic, motor control, ESP32 serial, differential drive |
| Phase 5 | RPLIDAR integration, obstacle avoidance, sensor/decision fusion |
| Phase 6 | STT, NVIDIA NIM LLM/vision, Piper TTS, wake word and voice interaction |
| Final my_robot | Clean integrated AMR: perception + LiDAR + control + safety + interaction + web |

# 11. Performance and optimization lessons

| Metric | Observed final / representative result |
| --- | --- |
| Full-stack RAM after July optimization | ~4.08 GB |
| Detector | Generally ~26–29 FPS; project commonly demonstrated ~27–30 FPS |
| Effective /detections rate | ~27.2 Hz |
| Detector latency | Commonly ~30–45 ms class; logs also showed high-20/low-30 ms samples |
| Watchdog CPU | Reduced from ~20–26% to ~11–17% |
| Temperature | ~58–60 °C after optimization |
| Vision + Piper user-perceived interaction | ~5 s class reported for 'what do you see?' in testing |

## 11.1 The three highest-value optimizations

Limit OpenMP/BLAS worker threads. Multiple Python/ML processes creating large thread pools caused contention, memory overhead, and unstable scheduling.

Do not send full images to nodes that only need liveness. /camera/heartbeat removed unnecessary DDS image handling from the watchdog.

Throttle high-frequency logs. LiDAR statistics at scan rate created avoidable formatting, terminal, and journaling work.

## 11.2 Profiling caution

Running ros2 topic hz on multiple high-rate topics at the same time adds its own CPU/DDS workload. Measure one important high-rate stream at a time when trying to understand the robot's real performance.

# 12. Important engineering decisions and lessons

| Decision | Why it was sensible |
| --- | --- |
| TensorRT YOLO engine | Uses Jetson GPU efficiently and met real-time FPS target |
| Python for most custom nodes | Faster iteration and sufficient performance for current control rates |
| External C/C++ LiDAR/video packages | Avoids rewriting stable low-level infrastructure |
| BEST_EFFORT shallow image QoS | Prevents stale-frame queues in a real-time camera pipeline |
| Tracker separate from detector | Modularity and easier debugging/tuning |
| Obstacle detector separate from decision | LiDAR interpretation can evolve without rewriting behaviour logic |
| Independent watchdog | Safety supervision remains logically separate from normal behaviour |
| Local Piper TTS | No cloud latency/cost for speech output |
| NIM for text/vision | Avoids hosting large LLM/VLM weights on an 8 GB Jetson |
| rosbridge + web_video_server split | Structured browser control and efficient video streaming use the right transport for each job |

## 12.1 What is not SLAM yet

The current robot is reactive/autonomous but not map-based. It does not yet maintain a global occupancy map, localize itself on that map, plan a global path, or use wheel-encoder odometry. Search/roam and obstacle avoidance are behaviour logic, not SLAM or Navigation2.

## 12.2 Natural V2 path

A future encoder-equipped chassis can retain much of this stack while adding wheel odometry, TF frames, SLAM Toolbox, localization, and Navigation2. The current camera/YOLO/interaction/web layers can remain useful above that navigation foundation.

# 13. Study and interview revision

1. Why did you choose ROS 2 topics instead of putting all functionality in one Python process?

2. Explain the exact camera → detector → tracker → decision → motor data path.

3. Why is BEST_EFFORT QoS appropriate for camera images?

4. What does TensorRT change compared with running a .pt model directly?

5. Why do you still need a tracker after YOLO detects a person every frame?

6. How does the Hungarian algorithm fit into multi-object tracking?

7. What does a Kalman Filter predict in your tracker?

8. How is /scan converted into /obstacle_detected and /free_direction?

9. Why is bounding-box width only a distance proxy rather than real depth?

10. What happens when the target disappears for two seconds?

11. What is the difference between SEARCH/ROAM and SLAM-based navigation?

12. Why does the LLM node route robot commands locally before calling NVIDIA NIM?

13. Which models are local and which are cloud-hosted?

14. How do you prevent the STT from hearing the TTS speaker and creating a loop?

15. Why is Piper running on CPU, and why is that acceptable in this architecture?

16. What is rosbridge doing that web_video_server is not doing?

17. Why did the watchdog move from /camera/image_raw to /camera/heartbeat?

18. Why did limiting OMP/BLAS threads improve the whole robot?

19. What happens from /cmd_vel until physical wheel motion?

20. What safety behaviour should occur if LiDAR or camera data stops arriving?

# 14. Quick-reference cheat sheet

| Item | Remember |
| --- | --- |
| Robot name/persona | Vector |
| Compute | Jetson Orin Nano 8 GB |
| ROS | ROS 2 Humble |
| Camera | IMX219-120 CSI |
| Detector | YOLOv8n TensorRT engine |
| Tracker | Kalman + Hungarian |
| LiDAR | RPLIDAR C1M1 |
| STT | Whisper tiny.en family |
| Chat | meta/llama-3.1-8b-instruct via NVIDIA NIM |
| Vision | meta/llama-3.2-11b-vision-instruct via NVIDIA NIM |
| TTS | Piper en_US-hfc_female-medium, CPU |
| MCU | ESP32-C3 Super Mini |
| Motor driver | 2 × BTS7960 |
| Motors | 4 × 12 V 100 RPM Johnson geared motors |
| Battery | 11.1 V 6000 mAh 3S2P Li-ion |
| Web | rosbridge_suite + web_video_server + custom HTML/CSS/JS UI |
| Custom runtime nodes | 10 |
| Principal runtime ROS nodes incl. external | 13 (14 with optional display node) |
| Main launch | robot_bringup / params_my_robot.launch.py |

# 15. Source references for continued learning

These are the upstream projects/services that are most useful to revisit while studying the system:

SLAMTEC ROS 2 LiDAR driver — https://github.com/Slamtec/sllidar_ros2

SLAMTEC long-standing RPLIDAR ROS package — https://github.com/Slamtec/rplidar_ros

RobotWebTools web_video_server — https://github.com/RobotWebTools/web_video_server

RobotWebTools rosbridge_suite — https://github.com/RobotWebTools/rosbridge_suite

Project-specific values in this handbook were consolidated from the final node/code explanations, runtime logs, performance optimization notes, and the established project architecture. Where the project had multiple historical variants, this handbook prioritizes the final parameterized runtime and explicitly labels optional/earlier components.