# Parameterized Watchdog Node Explanation

ROS 2 topic monitoring, immediate LiDAR safety, and emergency-stop logic

# 1. Purpose

This watchdog independently checks the liveness of the LiDAR, obstacle detector, tracker, camera heartbeat, and velocity-command stream. It also checks raw LiDAR distance while the robot is moving forward and monitors battery reporting. Any active safety reason causes an emergency stop and a direct zero-velocity command.

# 2. Important update in this version

Unlike the earlier fixed-constant version, topics and thresholds are ROS 2 parameters. Camera liveness is monitored through /camera/heartbeat using std_msgs/Empty. Startup-grace time is excluded from both topic and battery timeout calculations.

# 3. ROS interfaces

| Type | Default topic | Message | Purpose |
| --- | --- | --- | --- |
| Subscriber | /scan | LaserScan | LiDAR liveness and immediate distance check. |
| Subscriber | /obstacle_detected | Bool | Obstacle-node liveness. |
| Subscriber | /tracked_detections | Detection2DArray | Tracker liveness. |
| Subscriber | /camera/heartbeat | Empty | Camera liveness. |
| Subscriber | /cmd_vel | Twist | Decision-node liveness and forward motion. |
| Subscriber | /battery_status | Float32 | Battery percentage and reporting age. |
| Publisher | /emergency_stop | Bool | Emergency-stop state. |
| Publisher | /cmd_vel | Twist | Direct zero-velocity failsafe. |
| Publisher | /llm_response | String | Spoken warning text. |
| Publisher | /robot_emotion | String | Sad/neutral watchdog emotion. |

# 4. Parameters

Topic parameters: Allow every subscribed and published topic to be changed from YAML or the command line.

Timeout parameters: Define how long each critical topic may remain silent.

Battery parameters: Define warning, critical, and stale-reporting thresholds.

LiDAR safety parameters: Define the critical distance, forward cone, and minimum forward speed.

Loop parameters: Define watchdog check rate and startup grace period.

# 5. Initialization flow

Declare all topic and threshold parameters.

Read values and convert them to explicit string, float, or integer types.

Create reliable QoS for status/control topics and best-effort QoS for LaserScan.

Initialize last-seen timestamps and battery state.

Create the fault-reason set and spoken-warning suppression set.

Create all subscribers, publishers, and the periodic watchdog timer.

# 6. Liveness callbacks

_make_liveness_callback() returns a small callback that updates the last-seen timestamp for a specific monitored topic. The message itself is unused because only arrival time matters.

# 7. Velocity callback

_cmd_vel_callback() records command-stream liveness and the latest linear.x value. While an emergency is active, it ignores /cmd_vel because the watchdog itself is publishing zero Twist messages on the same topic.

# 8. LiDAR callback

Refresh the /scan liveness timestamp.

Convert the forward-cone parameter from degrees to radians.

Ignore ranges outside the LaserScan sensor limits.

Calculate and normalize each sample angle.

Find the nearest valid range inside the forward cone.

Check whether the robot is commanded to move forward.

Add or remove the critical_distance reason and update the e-stop.

# 9. Periodic check loop

Return until the startup grace period ends.

Exclude grace-period time from each topic's calculated age.

Add stale_<topic> reasons for expired inputs.

Remove stale reasons when data resumes.

Maintain the e-stop and continuously publish zero velocity while active.

Exclude grace-period time from battery reporting age.

Publish battery stale, low, or critical warnings once per occurrence.

# 10. Emergency-stop behavior

active_reasons may contain several simultaneous faults. The e-stop is activated when the set becomes non-empty and released only when it becomes empty. Activation publishes True, sends zero velocity, and sets sad emotion. Recovery publishes False and restores neutral emotion.

# 11. Cleanup performed

Removed decorative separators, emojis, repetitive comments, and excess whitespace.

Reordered imports and formatted all multiline calls consistently.

Changed uppercase instance variables to normal instance-variable names.

Renamed TIMEOUTS to timeouts and _mk_cb to _make_liveness_callback.

Stored subscriber and timer references with descriptive names.

Added explicit type conversions, type hints, and concise docstrings.

Preserved every topic, parameter name, default value, timeout, threshold, and safety transition.

# 12. Review observations

Battery conditions do not activate e-stop. They currently generate speech warnings only.

check_rate_hz must be greater than zero. A zero value would cause division by zero while creating the timer.

Wall-clock timing is used. time.time() can be affected by system-clock adjustments; behavior was preserved.

All monitored topics are treated as mandatory. Disabling one intentionally will eventually create a stale-topic e-stop reason.